3GPP TSG-RAN WG4 #106-bis-e		   R4-2305811

Electronic meeting,  April 17th – 26th, 2023

Agenda item:			5.20.2.2.4

Source:	Qualcomm Inc

Title:	FR2 UE modelling

Document for:	Approval

# Scope

In this submission we discuss some open items for UE modelling in FR2.

# RX modelling

## Sub-band selectivity performance level for legacy UE

In the last meeting companies were encouraged to describe their method of calculation. We find subband interference is computed from total input at the receiver input, on other words signal + any interference hitting the receiver antenna.

Agreement: Companies are encouraged to provide their method of calculation of sub-band/in-channel selectivity - Different methods are provided to assist companies to understand the achievable sub-band selectivity performance, while not for alignment purpose.

Proposal: FR1 and FR2 subband selectivity are computed from the total input receive power

sub-band selectivity performance:

In the last meeting RAN4 agreed on this aspect:

Agreement: For FR2-1 companies are encouraged to further discuss values in the range of [20 to 34 dB] for sub-band/in-channel selectivity with accompanying clarification as how they calculate DL subband interference based on one value from this range and what guard band is assumed.

For FR2 typical UE  selectivity is 34 dB as described our contribution last meeting, based on analysis of commercial devices.

Proposal: Typical FR2-1 UE sub-band selectivity is 34 dB.

## NF modelling for co-channel CLI

There have been discussions on how to model the NF aspects, whether power-dependent or fixed values. RAN4 has agreed on the following:

Agreement: - Use a fixed value noise figure model for the purpose of system level simulation for SBFD - FR1 noise figure value in the range [7 to 9 dB] - FR2-1 noise figure value in the range [7.5 to 10 dB]

Our submission from last meeting we analyzed a typical UE and arrived at 7.5 dB noise figure for FR2-1.

Proposal: Typical FR2-1 noise figure to be modelled as 7.5 dB.

## NF modelling for adjacent-channel CLI

In the last meeting RAN4 agreed adjacent channel and co-channel NF should be treated the same

Agreement: Same agreement as for co-channel CLI model (Issue 2-2-1)

Proposal: NF is 7.5 dB for adjacent channel, just as in co-channel.

# Conclusions

Proposal: Typical FR2-1 UE sub-band selectivity is 34 dB.

Proposal: Typical FR2-1 noise figure to be modelled as 7.5 dB.

Proposal: NF is 7.5 dB for adjacent channel, just as in co-channel.

# References

[1] - R4-2302884,WF for SBFD feasibility study and requirements impact from UE aspect