3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305845

# E-meeting, April 17th – April 26th, 2023

# Title:	Discussion on RAN LS to RAN1 for NTN above 10 GHz

# Source:	THALES

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	5.27.5

Release	Rel-18

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

At the meeting RAN4#106 (Athens, Greece), in R4-2302998 (WF for system parameters of above 10 GHz NTN band), the following agreement was made with respect to above 10 GHz NTN frequency bands and frequency range specifications:

Agreement 1:

The following bands and frequency ranges were agreed, without square bracket.  Note that the DL lower frequency range was modified from 17.7 to 17.3 to reflect regional regulations.

| NTN operating band | UL (Earth-to-Space) | DL (Space-to-Earth) |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz |

The notes were then treated individually:

- Note 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01.

- Note 2: This band is applicable in the USA subject to FCC 47 CFR part 25.

- Note 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202).

Agreement 2:

Wait for RRM conclusion on the feasibility of using sub-carrier spacing values 60 and 120 kHz for NTN above 10 GHz.

# Discussion on system parameters

RAN4 discussions are still ongoing on how to define the maximum timing errors (Te) for 60 kHz and 120 kHz sub-carrier spacing, which were not been defined for NTN FR1 in Release-17. Moreover, maximum timing errors (Te) limits defined in Release-17 may not be compatible with 60 kHz and 120 kHz sub-carrier spacing configurations due to lower CP length.

For instance, one can find the FR1 configurations with 15 kHz and 30 kHz SCS in the latest approved TS 38.133:

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Proposal 1: Send RAN4 LS to RAN1. The details are in the Annex and in the attached corresponding file.

Proposal 2: RAN4 RRM to further investigate the impact on the NTN specifications of supporting 60 kHz and 120 kHz in frequency range above 10 GHz.

Proposal 3: RAN4 RRM to further investigate maximum timing errors (Te_NTN) limits, maximum autonomous time adjustment step Tq_NTN and the aggregate adjustment rate Tp_NTN, for 60 kHz and 120 kHz sub-carrier spacing for NTN in above 10 GHz.

# Annex - Draft LS

3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-230xxxx

E-meeting, April 17th – April 26th, 2023

# Title:	Draft LS on support of FDD mode in above 10 GHz for NTN

# Response to:	-

# Release:	Release-18

Work Item:	NR_NTN_enh-Core

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:	TSG RAN WG2

Contact Person:

#### Name:	Dorin Panaitopol

#### E-mail Address:	dorin.panaitopol [at] thalesgroup [dot] com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN4 is currently investigating NTN core specifications for above 10 GHz considering FDD mode for the following frequency bands:

| NTN operating band | UL (Earth-to-Space) | DL (Space-to-Earth) |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz |
| Note 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01. Note 2: This band is applicable in the USA subject to FCC 47 CFR part 25. Note 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). |  |  |

RAN4 discussions are still ongoing on how to define the maximum timing errors (Te) for 60 kHz and 120 kHz sub-carrier spacing, which were not been defined for NTN FR1 in Release-17. Moreover, maximum timing errors (Te) limits defined in Release-17 may not be compatible with 60 kHz and 120 kHz sub-carrier spacing configurations due to lower CP length.

RAN4 has identified the following question that need inputs from RAN1:

Question 1: What would be the impact on the NTN specifications of supporting FDD mode with 60 kHz and 120 kHz in frequency range above 10 GHz?

2. Actions:

To RAN1 group:

ACTION: RAN4 respectfully asks RAN1 to provide feedback on Question 1 by taking into account FDD mode with 60 kHz and 120 kHz sub-carrier spacing for NTN related studies for above 10 GHz in Release-18.

3. Date of Next RAN4 Meetings:

TSG RAN WG4#107			22th of May – 26th of May 2023                                   Korea, Incheon

TSG RAN WG4#108			21st of August – 25th of August 2023                           France, Toulouse

-END-