3GPP TSG-RAN WG4 Meeting # 106bis-e			       			R4-2306634

Online, April 17th  – April 26th , 2023

Agenda Item:	5.31.2.1.2

Source: 	Xiaomi

Title: 	TP to TR on the SL-e TX requirement

Document for:	Approval

- Introduction

The sidelink unlicensed TX requirement has been initially discussed in the last RAN4 meeting with some general aspects. In this paper, we give further discussion on these single carrier TX requirements.

The SLU single carrier TX requirements have been discussed in the first round and the summary [1] is agreed. Based on the summary, some of the requirements are agreed and hence the TP to TR is revised to capture the agreed requirements. A table is for better checking as below:

| Issue | Status |
| --- | --- |
| Issue 3-1-2: Transmit power density | Agreed |
| Issue 3-5-1: Pcmax for SL-U single CC | Agreed |
| Issue 3-10-1: Freq Error for SL-U single CC | Agreed |
| Issue 3-11-1: EVM for SL-U single CC | Agreed |
| Issue 3-12-1: Carrier leakage for SL-U single CC | Agreed |
| Issue 3-13-1: Equalizer spectrum flatness for SL-U single CC | Agreed |
| Issue 3-14-1: In-band emission for SL-U single CC | Agreed |
| Issue 3-15-1: Occupied BW for SL-U single CC | Agreed |
| Issue 3-16-1: SEM for SL-U single CC | Agreed |
| Issue 3-17-1: ASEM for SL-U single CC | Agreed |
| Issue 3-18-1: ACLR for SL-U single CC | Agreed |
| Issue 3-19-1: General Spurious Emission for SL-U single CC | Agreed |
| Issue 3-21-1: UE co-existence for SL-U single CC | Agreed |
| Issue 3-22-1: Transmit intermodulation for SL-U single CC | Agreed |

Hence corresponding agreed requirements is captured in below TP to TR.

2 Reference

[1] R4-2306223 		Topic summary for [106bis-e][144] NR_SL_enh2_UERF_part1, Moderator (OPPO)

3 Annex TP to TR 38.786

6.1	Tx requirements for NR SL single carrier operation in unlicensed band

6.1.1	Maximum output power for NR SL-U

6.1.2	UE maximum output power reduction for NR SL-U

6.1.3	UE additional maximum output power reduction for NR SL-U

6.1.4	Configured transmitted power for NR SL-U

The configured transmitted power for NR-U is defined in clause 6.2F.4 of TS 38.101-1 and the normal NR requirement in clause 6.2.4 of TS 38.101-1 apply. For NR V2X, PCMAX is defined for PSSCH\PSCCH, S-SSB and PSFCH in clause 6.2E.4 specifically.

It has been agreed to reuse the NR V2X Configured transmitted power requirements in 38.101-1 clause 6.2E.4 for NR SL-U.

6.1.5	Minimum output power for NR SL-U

6.1.6	Transmit OFF power for NR SL-U

6.1.7	ON/OFF time mask for NR SL-U

6.1.8	Power control for NR SL-U

6.1.9	Transmit signal quality for NR SL-U

6.1.9.1	Frequency Error

The Frequency error for  NR-U is defined in clause 6.4F.1 of TS 38.101-1 and the normal NR requirement in clause 6.4.1 of TS 38.101-1 apply. For NR V2X, frequency error is defined in clause 6.4E,1 as ±0.1 PPM accuracy observed over a period of 1 ms against GNSS.”

It has been agreed to reuse the NR V2X Frequency error requirements in 38.101-1 clause 6.4E.1 for NR SL-U.

6.1.9.2	EVM

The EVM of NR-U is defined in clause 6.4F.2 of TS 38.101-1 and the normal NR requirement in clause 6.4.2.1 of TS 38.101-1 apply. For NR V2X, EVM is defined in clause 6.4E.2.2  with other than the Pi/2-BPSK modulation, which is not supported for NR V2X.

It has been agreed to reuse the NR V2X EVM requirements in 38.101-1 clause 6.4E.2.2 for NR SL-U.

- For NR V2X the EVM requirements in clause 6.4.2.1 are valid for PSCCH, PSSCH and PSBCH physical channels with other than pi/2-BPSK modulation, which is not supported for NR V2X, and excluding the guard symbol at the end of transmitted slots

6.1.9.3 	Carrier Leakage

The Carrier Leakage of NR-U is defined in clause 6.4.2.2 of TS 38.101-1 and  the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC carrier leakage requirements in 38.101-1 clause 6.4.2.2 for NR SL-U.

6.1.9.4	Equalizer spectrum flatness

The Equalizer spectrum flatness of NR-U is referred to clause 6.4.2.4 of TS 38.101-1 and the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC Equalizer spectrum flatness requirements in 38.101-1 clause 6.4.2.4 for NR SL-U.

6.1.9.5	In-band Emission

The In-band Emission of NR-U is defined in clause 6.4F.2.3 of TS 38.101-1. For NR V2X, In-band Emission is defined in clause 6.4E,2.4  where normal NR requirement of clause 6.4.2.3 apply.

It has been agreed to reuse the NR-U In-band emission requirements in 38.101-1 Table 6.4F.2.3-1 for NR SL-U. Minimum requirements for in-band emissions for NR SL-U PSCCH, PSSCH and PSBCH transmissions over the measurement interval that takes into account guard period at the end of the transmit slot.

6.1.10	Spectrum emission mask for NR SL-U

6.1.10.1	Occupied Bandwidth

The Occupied Bandwidth of NR-U is defined in clause 6.5.1 of TS 38.101-1 and the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC Occupied Bandwidth requirements in 38.101-1 clause 6.5.1 for NR SL-U.

6.1.10.2	SEM

The SEM of NR-U is defined in clause 6.5F.2.2 of TS 38.101-1. For NR V2X, In-band Emission is defined in clause 6.5E,2.2  where normal NR requirement of clause 6.5.2.2 apply.

It has been agreed to reuse the NR-U SEM requirements in 38.101-1 Table 6.4F.2.3-1 for NR SL-U.

6.1.10.3	A-SEM

Additional spectrum emission mask requirements are not defined for NR-U.

It is agreed not to define A-SEM requirements for NR SL-U when operating in bands n46, n96 and n102.

6.1.11	ACLR requirements for NR SL-U

The ACLR of NR-U is defined in clause 6.5F.2.4 of TS 38.101-1. For NR V2X, ACLR is defined in clause 6.5E.2.4  where normal NR requirement of clause 6.5.2.4 apply. The ACLR requirement was optimized for operation in un-licensed band during the development of the NR-U requirements.

It has been agreed to reuse the NR-U ACLR requirements in 38.101-1 clause 6.5F.2.4 for NR SL-U.

6.1.12	Spurious emissions for NR SL-U

The general spurious emission of NR-U is defined in clause 6.5.3.1 of TS 38.101-1 and the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC general spurious emission requirements in 38.101-1 clause 6.5.3.1 for NR SL-U.

6.1.13	Spurious emission band UE co-existence for NR SL-U

UE coexistence requirements are not defined for NR-U.

It has been agreed not to define UE coexistence spurious emission requirements for NR SL-U when operating in bands n46, n96 and n102 in case of non-concurrent operation.

6.1.14	Transmit intermodulation for NR SL-U

The Transmit intermodulation of NR-U is defined in clause 6.5.4 of TS 38.101-1 and the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC Transmit intermodulation requirements in 38.101-1 clause 6.5.4 for NR SL-U.