3GPP TSG-RAN WG4 Meeting # 106bis-e                                                       R4-2306556

Online, April 17 – April 26, 2023

Agenda item:			6.5

Source:	Inmarsat

Title:	WF on IoT NTN Extended L-band

Document for:	Approval

# System Parameters

Issue 1-1: Operating Bands and Band numbering

Candidate options:

- Option 1: The Extended L-band should be numbered as Table 2.1-1.


Table 2.1-1: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [253] | 1668 MHz | – | 1675 MHz | 1518 MHz | – | 1525 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

- Option 2: Further discuss frequency range after further checking of the ECC report.

MODERATOR NOTE: The Extended L-band frequency range for UL and DL has been agreed as part of WID approval in RAN#99

Tentative agreements:

Agree on Band 253 as the band numbering.

Recommended WF:

- Considering WID agreement in RAN#99 and views from companies, agree on the frequency range in Option 1, but encourage companies to further check ECC Report 263.

Issue 1-2: Channel numbering, Channel Raster and EARFCN

Tentative Agreement:

- Postpone discussion of Channel Numbering, Channel Raster and EARFCN after other system parameters are more stable.

Issue 1-3: Default UE TX-RX separation

Candidate options:

Proposal:

- Option 1: Agree on Default TX-RX separation of -150 MHz

- Option 2: Further discuss Default TX-RX separation

Recommended WF:

- Consider if Option 1 can be agreed as a starting point, pending further checking of the frequency range.

Issue 1-4: Compliance with ECC recommendations

Candidate options:

- Option 1: Evaluate the LTE NB1, NB2, and M1 blocking specification to determine if it complies with the ECC assumption for enhanced performance

Recommended WF:

- Companies to further check ECC Report 263 and evaluate, based on current blocking requirements for NB1, NB2 and M1, whether additional blocking requirements need to be considered.

# UE RF Requirements

Issue 2-1: UE Maximum Output Power

Candidate options:

- Option 1: Reuse existing UE Maximum Output Power requirements from 36.102 at least as a starting point for further discussion, for category M1 and NB1/NB2, 23dBm with +/-2dB tolerance (Mediatek)

- Option 2: For the Extended L-band, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Table 2.5-1 and 2.5-2, respectively (ZTE)
 
Table 2.5-1: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| [253] |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table 2.5-2: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| [253] | 23 | +/-2 | 20 | +/-2 |

MODERATOR NOTE: One company expressed skepticism on inclusion of PC5.  However other companies have expressed interest in keeping PC5, which is also currently supported by other NTN bands.

Recommended WF:

- Agree Option 2

Issue 2-2: MPR

Tentative agreement:

- Reuse existing MPR requirements from 36.102 at least as a starting point for M1 and NB1/NB2. Different MPR tables for category M1 and NB1/NB2 separately

Issue 2-5: Transmission bandwidth for eMTC

Tentative Agreement:

- For the Extended L-band, use UE channel bandwidth and transmission bandwidth configuration for eMTC NTN UL operation defined in Table 2.2-1 below.

Table 2.2-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

Issue 2-3: A-MPR

Tentative Agreement:

- Further discuss whether A-MPR requirements are needed.

Issue 2-4: Spurious emissions & additional spurious emissions

Tentative Agreement:

- Further discuss spurious emissions requirements.

Issue 2-6: Transmission bandwidth for NB-IoT

Recommended WF:

- For the Extended L-band, use UE channel bandwidth and transmission bandwidth configuration for NB-IoT NTN UL operation defined in Table 2.2-2 below as a starting point.

Table 2.2-2: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Issue 2-7: In-band blocking

Candidate options:

- Option 1: Reuse existing In-band blocking requirements from 36.102 at least as a starting point for further discussion, if there is no specific concern.  Different band group may have different In-band blocking tables for category M1 and NB1/NB2 separately. (Mediatek)

- Option 2: Further discuss in-band blocking requirements

Recommended WF:

- Consider in-band blocking requirements from 36.102 as a starting point for further discussion, pending clarifications on system parameters.

Issue 2-8: Out-of-band blocking

Candidate options:

- Option 1: Consider reusing basic Out-of-band blocking requirements for the UE as a starting point, and further discuss any additional blocking requirements based on ECC Recommendations and ETSI requirements for the 1518 MHz DL band edge

- Option 2: Further discuss out-of-band blocking requirements

Recommended WF:

- Agree Option 1.

Issue 2-11: Draft running CR for TS 36.102

Tentative Agreement:

- Postpone discussion of CRs to later meetings when agreement is reached on system parameters and general requirements.

# SAN RF Requirements

Issue 4-1: Channel Bandwidth for eMTC NTN

Tentative Agreement:

- For the Extended L-band, use SAN channel bandwidth and transmission bandwidth configuration for eMTC NTN operation defined in Table 2.2-1 below as a starting point, and further discuss to align with UE.

Table 2.2-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

Issue 4-2: Channel Bandwidth for NB-IoT NTN

Tentative Agreement:

- For the Extended L-band, use SAN channel bandwidth and transmission bandwidth configuration for NB-IoT NTN operation should defined in Table 2.2-2 as a starting point and further discuss to make sure it is aligned with agreements on the UE side.

Table 2.2-2: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Issue 4-3: Draft running CR for TS 36.108

Tentative Agreement:

- Postpone discussion of CRs to later meetings when agreement is reached on system parameters and general requirements.