3GPP TSG-RAN WG4 Meeting #106bis-e                                                        R4-2306004

Online, April 17th – 26th, 2023

Title:	Reply LS on interference modelling for duplex evolution

Response to:	R1-2302087

Release:	Rel-18

Work Item:	FS_NR_duplex_evo

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact person:	Wang, He (Jackson), h0809.wang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

## 1	Overall description

For RAN1 LS R1-2302087, RAN4 thanks RAN1 for further sharing the agreements and working assumption for Rel-18 NR duplex evolution study. For the following agreements and working assumption regarding the interference modelling for SBFD operation, RAN4 has discussed and conclude the reply as follows:

- Updated TR skeleton for RAN4 part of TR 38.858 is approved in R4-2302887.

- Regarding RAN1 Agreement-3 in R1-2302087,

  - Based on RAN1’s understanding on the co-site gNB-gNB adjacent-channel CLI modelling (i.e., reusing similar method as co-site inter-sector co-channel inter-subband CLI modeling), RAN4 would like to provide the understanding on αadj, co-site-x (i.e., the interference suppression capability of co-site inter-sector adjacent-channel CLI between the aggressor sector x and the victim sector) as

10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS

in which

    - The interference suppression capability of co-site inter-sector CLI from three different adjacent-channel sectors could be assumed as the same. i.e. αadj,co-site-1 = αadj,co-site-2 = αadj,co-site-3

    - How to interpret the noise figure model (i.e., noise figure over average total input power, captured in RAN4 LS R4-2302885) and its relationship with ACSBS:

      - ACS should be applied, and the noise figure model should also be applied to the total of all RX power.

    - ACSBS value range achievable by typical FR1 gNB implementation:

      - The range from [46 or 50]dB as lower bound to [62]dB as upper bound is proposed in RAN4.

      - The above range is proposed by considering both noise figure model and ACSBS in the simulation, in which the noise figure model has captured the non-linearity of RF components and ACSBS refer to baseband suppression.

    - The value of ICSBS by typical FR1 gNB implementation:

      - Based on the similar baseband suppression, the same range of ACS values can be achievable for ICS.

        - Lower bound is 46dB.

        - Upper bound is [62]dB.

    - For the spatial isolation of adjacent-channel inter-sector CLI, the following values have been proposed for macro BS in RAN4:

      - FR1: from (62+X)dB to (93+X)dB with (75+X)dB being typical value.

      - FR2-1: from (75+X)dB to (98+X)dB with (88+X)dB being typical value.

      - For both FR1 and FR2-1: X which can be in the range of [0~25] is added to the inter-sector isolation agreed for co-channel inter-sector interference, because of additional spacing between adjacent-channel antennas.

      - Note: The additional spatial isolation X can be different between FR1 and FR2-1.

      - Note: Companies has proposed that isolating materials between adjacent channel antennas and RF interference cancellation and/or beam nulling can provide additional spatial isolation.

      - Note: There is no consensus on the achievable performance on the value of X, and the feasibility of isolating materials. RAN4 will further evaluate and update to RAN1 if needed.

- Regarding RAN1 Agreement-4 in R1-2302087,

  - RAN4 confirm RAN1’s understanding on this model;

  - For the in-channel selectivity (or subband selectivity) ICSUE:

    - ICSUE = 33dB for FR1 and ICSUE = 23dB for FR2

      - Above values are for simulation usage purpose and only based on the assumption of same bandwidth between interference and wanted signals

      - Above values derived based on single company’s measurement data.

      - This agreement has no impact on RAN4 specified minimum RF requirements

  - Clarify that the IBE is applied per UE transmitter (with the per transmitter output power assumed), which is a reasonable interpretation for most circumstances that can be used in RAN1:

    - If total TX power (Pt) is equally allocated among the Nt transmitter for UE:

      - At one TX antenna, the in-band emission will be Pt – 10*log10(Nt) – IBE

      - For the total impact from all TX antennas, the in-band emission should be Pt – IBE.

- Regarding RAN1 Working Assumption in R1-2302087,

  - RAN4 confirm RAN1’s understanding on the UE-UE co-channel channel model in the working assumption.

  - RAN4 confirm the same response for IBE and ICSUE to Agreement-4 can be reused here.

Furthermore, RAN4 made the following agreement on noise figure model (i.e., noise figure over average total input power, captured in RAN4 LS R4-2302885):

- For typical FR1 WA BS implementation, companies have different views on sub-band filter, and based on companies’ proposal on the implementation with sub-band filter, the blocking performance can be improved around 10 dB. The following values of A and B can apply to the noise figure model if sub-band filter adopted.

  - The values of A, B, C and D:

    - A = -35dBm

    - B = -17dBm

    - C = 5dB

    - D = 14dB

- Note: RAN4 has not reach consensus on the implementation feasibility of sub-band filter. RAN4 will further evaluate and update to RAN1 if needed.

- For FR1 MR BS class, the following noise figure model can be used as starting point:

  - (A, B, C, D) = (-38, -20, 10, 19)

    - Point A is suggested to follow the in-band blocking requirement as specified in TS 38.104, i.e. -38dBm.

    - Point C is suggested to follow the assumed noise figure, i.e. 10dB.

- For FR1 LA BS class, the following noise figure model can be used as starting point:

  - (A, B, C, D) = (-35, -17, 13, 22)

    - Point A is suggested to follow the in-band blocking requirement as specified in TS 38.104, i.e. -35dBm.

    - Point C is suggested to follow the assumed noise figure, i.e. 13dB.

- For FR2-1 BS, the following noise figure model can be used as starting point:

  - The values of A, B, C and D:

    - A = [-58] dBm

    - B = [-40] dBm

    - C = 10dB

    - D = [19] dB

RAN4 made the following agreement on UE noise figure model:

- UE noise figure for simulation purpose only:

  - 9dB for FR1

  - 10dB for FR2-1

Additionally, RAN4 made the following agreement on UE ACLR performance

- Improved TX modelling for adjacent-channel interference simulation:

  - FR1: UE ACLR is modelled as 30 dB at UE maximum TX power, and improves 1dB per 1dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 40 dB.

  - FR2-1: UE ACLR is modelled as 24 dB at UE maximum TX power, and improves 1dB per 1dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 34 dB.

## 2	Actions

To RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to consider above replies in the future discussion.

## 3	Dates of next RAN WG 4 meetings

TSG RAN WG4 Meeting #107			22nd – 26th May, 2023   	    		Incheon, Korea

TSG RAN WG4 Meeting #108			21st – 25th August, 2023   	    	Toulouse, France