3GPP TSG-RAN WG4 Meeting # 106bis-e		R4-2306465

Online, April 17 – April 26, 2023

Title:	LS on non-simultaneous UL and DL from different two bands during UL CA

Response to:	NONE

Release:	Rel-18

Work Item:	          NR_700800900_combo_enh-Core

Source:	RAN4

To:	RAN2

Cc:	NONE

Contact Person:

#### Name:	Hiromasa Umeda

E-mail Address:	<hiromasa.umeda@nokia.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN4 has discussed feasibility of UL CA of CA_n5-n8 where n5 DL and n8 UL frequency ranges partially overlap. As one of the possible solutions, non-simultaneous n5 DL and n8 UL operation for UL CA of CA_n5-n8 was proposed. The solution leads to following two states as CA.

(UL, DL) = (n5, n5+n8)@t1, (n5+n8, n8)@t2 as shown in below Figure 1 (from Figure 5.1.1-1 in TR 38.872)

Figure 1: Possible CA_n5-n8 UE architecture and operation diagram for semi-full-duplex CA

RAN4 has concluded that the latter state of (UL, DL) = (n5+n8, n8) would potentially require some RAN2 specifications changes since the number of CCs in uplink is larger than that in downlink. Hence, RAN4 would like to ask RAN2 followings (see the details on TR 38.872). Based on RAN2 feedback, RAN4 will make a decision on whether to specify relevant requirements on this solution or not.

- Whether or not it is feasible for dual UL CA of CA_n5-n8 to take a state of (UL, DL) = (n5+n8, n8) with the existing RAN2 specifications.

- Additionally, RAN4 would like to ask whether RAN2 can enable this kind of configurations with necessary possible changes in RAN2 specs within the Rel-18 timeframe, if requested later by RAN4 at Q3 at the earliest.

2. Actions: RAN4 respectfully requests RAN2 to share their views on the questions.

3. Dates of Next RAN4 Meetings:

TSG RAN WG4 Meeting #107	 22 May – 26 Apr 2023	     Incheon, Korea

TSG RAN WG4 Meeting #108			21 August – 25 August 2023		Toulouse, France