# 3GPP TSG-RAN WG4 Meeting # 106-Bis-e 	R4-2306657

e-Meeting, Apr. 2023

Title:	Reply LS on configured Tx power for STxMP in FR2

Response to:	(R1-2205639) LS on UE power limitation for STxMP in FR2

Source:	RAN4

To:	RAN1

Contact Person:

Name	: Sumant Iyer

E-mail Address	: sumanti _at_ qti_dot_qualcomm_dot_com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 would like to thank RAN1 for LS on UE power limitation for STxMP in FR2 and further clarify the highlighted aspect.

Question 3: In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class?

Question 4: If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can/shall be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)?

Some definitions for ‘conditions’ below:

- The total EIRP in any one direction refers to the aggregated EIRP of all beams over all panels in that direction.

- The existing EIRP limitation refers to the max. EIRP in the power class definition of the UE for single band operation.

- The existing TRP limitation refers to the max. TRP in the power class definition of the UE for single band operation.

RAN4 would like to confirm to RAN1 the conditions to be simultaneously met by a UE for STxMP:

- The total EIRP over all panels cannot exceed the existing EIRP limitation.

- The total TRP over all panels cannot exceed the existing TRP limitation.

- The sum of per-panel EIRP power limitation for STxMP can be greater than the existing EIRP limitation.

  - See condition #1, which must always be obeyed.

  - In cases where the beams for each panel have low enough spatial overlap, this clause allows EIRP for each panel to individually approach the existing EIRP limitation.

  - RAN4 has not completed spec. definition work for ‘per panel EIRP power limitation’.

- No explicit ‘per-panel’ TRP limit is deemed necessary.

2. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #107	May. 2023         		Incheon, KR

TSG-RAN WG4 Meeting #108	Aug. 2023         		Toulouse, FR