3GPP TSG-RAN WG4 Meeting #107 	R4-2307021

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	pCR for 36.181 Introduction of SAN demodulation requirements for IoT-NTN

Agenda item:	9.7.7.2

Document for:	Approval

Introduction

NB-IoT and eMTC operations were identified as two key enablers of connectivity in remote areas. In some of these areas, limited or no-cellular infrastructure might be available. Because of this, 3GPP has introduced IoT over NTN (Non-terrestrial networks) as potential solution to provide ubiquitous connectivity in such conditions [1]. Currently, the RAN plenary has approved the work item whose objective is to develop requirements for NB-IoT and eMTC operation over NTN [2]. The WID indicates that the discussions should leverage previous work for NR over NTN and IoT as long as applicable.

The worksplit for ‘NB-IoT/eMTC core & performance requirements for NTN’ was agreed at RAN4 #106 regarding TS 36.181 as follows:

| Clause number | Clause name | Company name |
| --- | --- | --- |
| TBD | Conducted performance requirements - General | Nokia |
| TBD | Performance requirements for PUSCH |  |
| TBD | Performance requirements for PUCCH |  |
| TBD | Performance requirements for PRACH |  |
| TBD | Performance requirements for NPUSCH format 1 | Huawei |
| TBD | Performance requirements for NPUSCH format 2 |  |
| TBD | Performance requirements for NPRACH |  |
| TBD | Radiated performance requirements - General | Nokia |
| TBD | Performance requirements for PUSCH |  |
| TBD | Performance requirements for PUCCH |  |
| TBD | Performance requirements for PRACH |  |
| TBD | Performance requirements for NPUSCH format 1 | Huawei |
| TBD | Performance requirements for NPUSCH format 2 |  |
| TBD | Performance requirements for NPRACH |  |
| TBD | Fixed Reference Channels for PUSCH transmission | Nokia |
| TBD | PRACH Test preambles | Nokia |
| TBD | Fixed Reference Channels for NPUSCH format 1 transmission | Huawei |
| TBD | Measurement of Performance Requirements | Huawei |

Within this contribution, we present the Nokia contributions for the Work-Split regarding TS 36.181 for discussion and approval at RAN4 #107.

Furthermore, within this contribution we include updates following the first round at RAN4 #106 bis-e

- References

- RP-211601, WID on NB-IoT/eMTC support for NTN, Mediatek Inc & Eutelsat S.A, TSG RAN #92e, June 2021

- RP-220938, New WID on NB-IoT/eMTC core & performance requirements for NTN, Mediatek Inc, TSG RAN #95e March 2022

- R4-2302933, WF for SAN demodulation requirements for IoT over NTN, RAN4 WG 4 #106, Athens, Mar 2023.

Appendix (Text Proposal)

# <Start of change 1>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36 108: "Space Access Node (SAN) radio transmission and reception".

[3]	ITU-R Recommendation M.1545, "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[4]	ITU-R recommendation SM.328: "Spectra and bandwidth of emissions".

[5]	ITU-R recommendation SM.329: "Unwanted emissions in the spurious domain ".

[6]	IEC 60721-3-3 (2002): "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Section 3: Stationary use at weather protected locations".

[7]	IEC 60721-3-4 (1995): "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Section 4: Stationary use at non-weather protected locations".

[8]	IEC 60068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold".

[9]	IEC 60068-2-2 (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat".

[10]	IEC 60068-2-6 (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)".

[11]	3GPP TR 25.942: "RF system scenarios".

[12]	3GPP TS 36.211: Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”

[13]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks".

<End of change 1>

# <Start of change 2>

<End of change 2>

## 8.1	General

Performance requirements are specified for a number of test environments and multipath channel classes.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for a SAN supporting carrier aggregation are defined in terms of single carrier requirements. The requirements only apply to those measurement channels that are supported by the SAN.

For SAN with receiver antenna diversity the required SNR shall be applied separately at each antenna port.

In tests performed with signal generators a synchronization signal may be provided, from the SAN to the signal generator, to enable correct timing of the wanted signal.

For tests in clause 8 the transmitter may be off.

# <Start of change 3>

## 8.2	Performance requirements for PUSCH

### 8.2.1	Performance requirements of PUSCH in multipath fading propagation conditions transmission on single antenna port for coverage enhancement

#### 8.2.1.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

The tests for CEModeA defined in Clause 8.2.1 are applicable only to the space access nodes supporting coverage enhancement configured with CEModeA. The tests for CEModeB defined in Clause 8.2.1 are applicable only to the the space access nodes supporting coverage enhancement configured with CEModeB.

A test for a specific channel bandwidth is only applicable if the SAN supports it. For a SAN supporting FDD multiple channel bandwidths, only the tests for the lowest and the highest FDD channel bandwidths supported by the SAN are applicable.

#### 8.2.1.2	Minimum Requirement

The minimum requirement is in TS 36.108 [2] subclause [8.2.1].

#### 8.2.1.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

#### 8.2.1.4	Method of test

##### 8.2.1.4.1	Initial Conditions

Test environment:	Normal, see subclause [D.2].

RF channels to be tested:	M; see subclause [4.7].

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN antenna connectors for diversity reception via a combining network as shown in Annex [I.3.2.]

##### 8.2.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth, defined in Table 8.2.1.4.2-1.

Table 8.2.1.4.2-1: AWGN power level at the SAN input

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -92.7dBm / 1.08MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in Table 8.2.1.4.2-2. The index of the narrowband is set to 0. For reference channels using resource blocks less than 6, the resource blocks shall be allocated from the lowest number within the indicated narrowband.

Table 8.2.1.4.2-2: Test parameters for testing PUSCH

| Parameter | CEMode A | CEMode B |
| --- | --- | --- |
| Maximum number of HARQ transmissions | 4 | 2 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 | FDD: 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Duration of PUSCH segment transmission (Nsegmentprecompensation) | Same duration as repetition length | Same duration as repetition length |
| Frequency hopping | OFF | OFF |
| Note 1: Guard period shall be created according to TS36.211, 5.2.5 [12] |  |  |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex B.

4)	Adjust the equipment so that required SNR specified in Table 8.2.1.5-1 to 8.2.1.5-2 is achieved at the SAN input.

5)	The test signal generator sends frames, and the receiver tries to demodulate the content. The frames are sent with certain frequency offsets as described below in Table 8.2.1.4.2-3 as well as certain timing offsets as described below in Table 8.2.1.4.2-4.

Table 8.2.1.4.2-3: Frequency Offsets for testing PUSCH

| Channel | No. Repetitions | Tx duration in one segment | Accumulated Frequency Offset (Hz) |
| --- | --- | --- | --- |
| CEmode A | 8 | 8ms | 4 |
| CEmode B | 256 | 256ms | 128 |

Table 8.2.1.4.2-4: Timing Offsets for testing PUSCH

| Channel | No. Repetitions | Tx duration in one segment | Step size Δt |
| --- | --- | --- | --- |
| CEmode A | 8 | 8ms | [0.01] µs per subframe |
| CEmode B | 256 | 256ms | [0.01] µs per subframe |

6)	For each of the reference channels in Table 8.2.1.5-1 to 8.2.1.5-2 applicable for the access node, measure the throughput, according to annex E.

#### 8.2.1.5	Test Requirement

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.1.5-1 for CEMode A tests and not be below the limits for the SNR levels specified in Table 8.2.1.5-2 for CEMode B tests.

Table 8.2.1.5-1 Minimum requirements for PUSCH, 1.4 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Number of PUSCH repetitions | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode A | 8 | [NTN-TDLA100-5] | A5-2 | 70% | TBD |
| 1 | 2 | Mode A | 8 | [NTN-TDLA100-5] | A5-2 | 70% | TBD |
| 1 | 1 | Mode A | 8 | [NTN-TDLC5-5] | A5-2 | 70% | TBD |
| 1 | 2 | Mode A | 8 | [NTN-TDLC5-5] | A5-2 | 70% | TBD |

Table 8.2.1.5-2 Minimum requirements for PUSCH, 1.4 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Number of PUSCH repetitions | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode B | 256 | [NTN-TDLA100-5] | A5-1 | 70% | TBD |
| 1 | 2 | Mode B | 256 | [NTN-TDLA100-5] | A5-1 | 70% | TBD |
| 1 | 1 | Mode B | 8 | [NTN-TDLC5-5] | A5-1 | 70% | TBD |
| 1 | 2 | Mode B | 8 | [NTN-TDLC5-5] | A5-1 | 70% | TBD |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in [Annex G].

<End of change 3>

# <Start of change 4>

## 8.3	Performance requirements for PUCCH

### 8.3.1	ACK missed detection for PUCCH format 1a transmission on single antenna port for coverage enhancement

#### 8.3.1.1	Definition and applicability

The performance requirement of PUCCH for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK for the configured PUCCH transmission repetitions when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK for the configured PUCCH transmission repetitions when the signal is present.

The test is applicable only to space access nodes supporting coverage enhancement. A test for a specific channel bandwidth is only applicable if the SAN supports it.

#### 8.3.1.2	Minimum Requirement

The minimum requirement is in TS 36.108 [2] subclause [8.3.9.1] and [8.3.9.2].

#### 8.3.1.3	Test purpose

The test shall verify the receiver’s ability to detect ACK under multipath fading propagation conditions for a given SNR.

#### 8.3.1.4	Method of test

##### 8.3.1.4.1	Initial Conditions

Test environment:	Normal, see subclause [D.2].

RF channels to be tested:	M; see subclause [4.7]

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN antenna connectors for diversity reception via a combining network as shown in Annex [I.3.2].

##### 8.3.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth defined in Table 8.3.1.4.2-1.

Table 8.3.1.4.2-1: AWGN power level at the SAN input

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -89.7 dBm / 1.08MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 36.211 [12].

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex B.

4)	Adjust the equipment so that the SNR specified in Table 8.3.1.4.2-1 is achieved at the SAN input during the ACK transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.1.4.2-1. The PUCCH signals are sent with certain timing and frequency offsets as described below. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 8.3.1.4.2-1: Test signal pattern for PUCCH format 1a demodulation tests

PUCCH test signals are sent with a frequency offset as described below in Table 8.3.1.4.2-2 as well as a timing offset as described below in Table 8.3.1.4.2-3.

Table 8.3.1.4.2-2: Frequency Offsets for testing PUCCH

| Channel | No. Repetitions | Tx duration in one segment | Accumulated Frequency offset (Hz) |
| --- | --- | --- | --- |
| PUCCH format 1a | 8 | 8ms | 4 |

Table 8.3.1.4.2-3: Timing Offsets for testing PUCCH

| Channel | No. Repetitions | Tx duration in one segment | Step size Δt |
| --- | --- | --- | --- |
| PUCCH format 1a | 8 | 8ms | [0.01] µs per subframe |

#### 8.3.1.5	Test Requirement

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in Table 8.3.1.2.1-1.

Table 8.3.1.2.1-1 Minimum requirements for single user PUCCH format 1a, 1Tx

| Number of TX antennas | Number of RX antennas | Channel Bandwidth | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Repetitions | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1.4MHz | Normal | [NTN-TDLA100-5] | 8 | TBD |
| 1 | 2 | 1.4MHz | Normal | [NTN-TDLA100-5] | 8 | TBD |
| Note 1 Frequency Hopping : OFF Note 2 Guard period shall be created according to TS36.211 [12], 5.2.5. |  |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

<End of change 4>

# <Start of change 5>

## 8.4	Performance requirements for PRACH

### 8.4.1	PRACH false alarm probability and missed detection

#### 8.4.1.1	Definition and applicability

The performance requirement of PRACH for preamble detection is determined by the two parameters: total probability of false detection of the preamble (Pfa) and the probability of detection of preamble (Pd). The performance is measured by the required SNR at probability of detection, Pd of 99%. Pfa shall be 0.1% or less.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For NTN-TDLA100-5, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 2.08us. The strongest path for the timing estimation error refers to the strongest path (i.e. average of the delay of all paths having the same highest gain = 310ns for ETU) in the power delay profile.

The test preambles for normal mode are listed in table [A.6-1.]

#### 8.4.1.2	Minimum Requirement

The minimum requirement is in TS 36.108 [2] subclause [8.4.1.1] and [8.4.2.1].

#### 8.4.1.3	Test purpose

The test shall verify the receiver’s ability to detect PRACH preamble under multipath fading propagation conditions for a given SNR.

#### 8.4.1.4	Method of test

##### 8.4.1.4.1	Initial Conditions

Test environment:	Normal, see subclause [D.2.]

RF channels to be tested:	[M; see subclause 4.7]

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN antenna connectors for diversity reception via a combining network as shown in [Annex I.3.1] or [Annex I.3.2] as applicable.

##### 8.4.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth.

Table 8.4.1.4.2-1: AWGN power level at the SAN input

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -89.7 dBm / 1.08MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in Annex A.

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex B.

4)	Adjust the frequency offset of the test signal according to Table 8.4.1.5-1.

5)	Adjust the equipment so that the SNR specified in Table 8.4.1.5-1 is achieved at the SAN input during the PRACH preambles.

6) 	The test signal generator sends a preamble and the receiver tries to detect the preamble. This pattern is repeated as illustrated in figure 8.4.1.4.2-1. The preambles are sent with certain timing estimation errors as described below. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 8.4.1.4.2-1: PRACH preamble test pattern

PRACH prambles are sent with a timing estimation error of 2.08us.

#### 8.4.1.5	Test Requirement

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in Table 8.4.1.5-1.

Table 8.4.1.5-1: PRACH missed detection requirements for coverage enhancement (PRACH frequency hopping OFF)

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 1 | NTN-TDLA100-5 | 270 Hz | 8 | TBD | TBD | - | - |
|  |  |  |  | 16 | - | - | TBD | TBD |
|  | 2 | NTN-TDLA100-5 | 270 Hz | 8 | TBD | TBD | - | - |
|  |  |  |  | 16 | - | - | TBD | TBD |
| Note 1: Under fading channels, the PRACH detection performance may be significantly different with different PRACH Configuration Indexes. The requirements in this table are defined based on the simulation results with PRACH Configuration Indexes (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3 respectively. |  |  |  |  |  |  |  |  |

<End of change 5>

# <Start of change 6>

## 11.1	General

### 11.1.0	Scope and definitions

Radiated performance requirements are specified for a number of test environments and multipath channel classes.

Unless stated otherwise, performance requirements apply for a single carrier only. Radiated performance requirements for a SAN supporting carrier aggregation are defined in terms of single carrier requirements. The requirements only apply to those measurement channels that are supported by the SAN.

The radiated performance requirements for SAN are limited to two demodulations branches as described in clause 11.1.1. Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the number of polarizations supported by the SAN, with the required SNR applied separately per polarization.

NOTE 1: SAN can support more than 2 demodulation branches, however radiated conformance testing can only be performed for 1 or 2 demodulation branches.

For tests in clause 11 the transmitter may be off.

In tests performed with signal generators a synchronization signal may be provided, from the SAN to the signal generator, to enable correct timing of the wanted signal.

For radiated performance requirements the SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the subframe on a RIB.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists.

Radiated performance requirements are specified at the RIB, based on one or two demodulation branches.

### 11.1.1	Demodulation branches

Radiated performance requirements are only specified for up to 2 demodulation branches.

If the SAN uses polarization diversity and has the ability to maintain isolation between the signals for each of the demodulation branches, then radiated performance requirements can be tested for up to two demodulation branches (i.e. 1RX or 2RX test setups). When tested for two demodulation branches, each demodulation branch maps to one polarization.

If the SAN does not use polarization diversity then radiated performance requirements can only be tested for o a single demodulation branch (i.e. 1RX test setup).

### 11.1.2	Applicability rule

<To be discussed, if applicability rules are needed, or to directly in “definition and applicability.”>

## 11.2	Radiated performance requirements for PUSCH

### 11.2.1	Performance requirements of PUSCH in multipath fading propagation conditions transmission on single antenna port for coverage enhancement

#### 11.2.1.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

The tests for CEModeA defined in Clause 8.2.1 are applicable only to the space access nodes supporting coverage enhancement configured with CEModeA. The tests for CEModeB defined in Clause 8.2.1 are applicable only to the the space access nodes supporting coverage enhancement configured with CEModeB.

A test for a specific channel bandwidth is only applicable if the SAN supports it. For a SAN supporting FDD multiple channel bandwidths, only the tests for the lowest and the highest FDD channel bandwidths supported by the SAN are applicable.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause [11.1.2].

#### 11.2.1.2	Minimum Requirement

The minimum requirement is in TS 36.108 [2] subclause [8.2.1].

#### 11.2.1.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions emulated in OTA test chamber, for a given SNR.

#### 11.2.1.4	Method of test

##### 11.2.1.4.1	Initial Conditions

Test environment:	Normal, see subclause [D.2].

RF channels to be tested:	[M]; see subclause [4.7].

Direction to be tested:	OTA REFSENS receiver target reference direction (see [D.54 in table 4.6-1]).

##### 11.2.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.3].

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.3]. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in Table 11.2.1.4.2-2. The index of the narrowband is set to 0. For reference channels using resource blocks less than 6, the resource blocks shall be allocated from the lowest number within the indicated narrowband.

Table 11.2.1.4.2-2: Test parameters for testing PUSCH

| Parameter | CEMode A | CEMode B |
| --- | --- | --- |
| Maximum number of HARQ transmissions | 4 | 2 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 | FDD: 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Duration of PUSCH segment transmission (Nsegmentprecompensation) | Same duration as repetition length | Same duration as repetition length |
| Frequency hopping | OFF | OFF |
| Note 1: Guard period shall be created according to TS36.211, 5.2.5 [12] |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.1.5.1 and 11.2.1.5.2 for SAN respectively, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 11.2.1.4.2-1.

Table 11.2.1.4.2-1: AWGN power level at the SAN input

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -92.7 - ΔOTAREFSENS [dBm] / 1.08 MHz |

9)	If RX diversity is not supported, ensure the polarisation match is achieved among test antenna(s) and the SAN under test, in order to maximize the power at the SAN receiver.

10)	For reference channels applicable to the SAN, measure the throughput.

#### 11.2.1.5	Test Requirement

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in Table 11.2.1.5-1 for CEMode A tests and not be below the limits for the SNR levels specified in Table 11.2.1.5-2 for CEMode B tests.

Table 11.2.1.5-1 Minimum requirements for PUSCH, 1.4 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Number of PUSCH repetitions | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode A | 8 | [NTN-TDLA100-5] | A5-2 | 70% | TBD |
| 1 | 2 | Mode A | 8 | [NTN-TDLA100-5] | A5-2 | 70% | TBD |
| 1 | 1 | Mode A | 8 | [NTN-TDLC5-5] | A5-2 | 70% | TBD |
| 1 | 2 | Mode A | 8 | [NTN-TDLC5-5] | A5-2 | 70% | TBD |

Table 11.2.1.5-2 Minimum requirements for PUSCH, 1.4 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Number of PUSCH repetitions | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode B | 256 | [NTN-TDLA100-5] | A5-1 | 70% | TBD |
| 1 | 2 | Mode B | 256 | [NTN-TDLA100-5] | A5-1 | 70% | TBD |
| 1 | 1 | Mode B | 8 | [NTN-TDLC5-5] | A5-1 | 70% | TBD |
| 1 | 2 | Mode B | 8 | [NTN-TDLC5-5] | A5-1 | 70% | TBD |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in [Annex G].

## 11.3	OTA performance requirements for PUCCH

### 11.3.1	Performance requirements for PUCCH format 1A

#### 11.3.1.2	ACK missed detection

##### 11.3.1.2.1	Definition and applicability

The performance requirement of PUCCH format 1 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

The test is applicable only to space access nodes supporting coverage enhancement. A test for a specific channel bandwidth is only applicable if the SAN supports it.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause [11.1.2].

##### 11.3.1.2.2	Minimum Requirement

The minimum requirement is in TS 36.108 [2] subclause [8.3.9.1] and [8.3.9.2].

##### 11.3.1.2.3	Test purpose

The test shall verify the receiver's ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

##### 11.3.1.2.4	Method of test

###### 11.3.1.2.4.1	Initial Conditions

Test environment: Normal, see [Annex B.2.]

RF channels to be tested for single carrier: [M; see clause 4.9.1]

Direction to be tested: OTA REFSENS receiver target reference direction (see [D.44 in table 4.6-1]).

###### 11.3.1.2.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in [annex D.7].

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [D.7]. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 36.211 [12], and according to additional test parameters listed in table 11.3.1.2.4.2-1.

Table 11.3.1.2.4.2-1: Test Parameters

| Parameter | Value |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBS – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in [annex G.2].

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.3.1.2.5.1, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.3.1.2.4.2-2.

Table 11.3.1.2.4.2-2: AWGN power level at the SAN input

| Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 1.4 | -89.7 – ΔOTAREFSENS dBm / 1.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in [D.43 in table 4.6-1] and clause [10.1]. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

8) 	The signal generator sends a test pattern with the pattern outlined in figure 11.3.1.2.4.2-1. The PUCCH signals are sent with certain timing and frequency offsets as described below. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 11.3.1.2.4.2-1: Test signal pattern for PUCCH format 1a demodulation tests

PUCCH test signals are sent with a frequency offset as described below in Table 11.3.1.2.4.2-3 as well as a timing offset as described below in 11.3.1.2.4.2-4.

Table 11.3.1.2.4.2-3: Frequency Offsets for testing PUCCH

| Channel | No. Repetitions | Tx duration in one segment | Accumulated Frequency offset (Hz) |
| --- | --- | --- | --- |
| PUCCH format 1a | 8 | 8ms | 4 |

Table 11.3.1.2.4.2-4: Timing Offsets for testing PUCCH

| Channel | No. Repetitions | Tx duration in one segment | Step size Δt |
| --- | --- | --- | --- |
| PUCCH format 1a | 8 | 8ms | [0.01] µs per subframe |

#### 11.3.1.2.5	Test Requirement

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of correctly detected ACK bits shall be larger than 99% for the SNR listed in table 11.3.1.2.5-1.

Table 11.3.1.2.5.1-1: Required SNR for PUCCH format 1a, 1Tx

| Number of TX antennas | Number of RX antennas | Channel Bandwidth | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Repetitions | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1.4MHz | Normal | [NTN-TDLA100-5] | 8 | TBD |
| 1 | 2 | 1.4MHz | Normal | [NTN-TDLA100-5] | 8 | TBD |
| Note 1 Frequency Hopping: OFF Note 2 Guard period shall be created according to TS36.211 [12], 5.2.5. |  |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

## 11.4	OTA performance requirements for PRACH

### 11.4.1	PRACH false alarm probability and missed detection

#### 11.4.1.1	Definition and applicability

The performance requirement of PRACH for preamble detection is determined by the two parameters: total probability of false detection of the preamble (Pfa) and the probability of detection of preamble (Pd). The performance is measured by the required SNR at probability of detection, Pd of 99%. Pfa shall be 0.1% or less.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For NTN-TDLA100-5, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 2.08us. The strongest path for the timing estimation error refers to the strongest path (i.e. average of the delay of all paths having the same highest gain = 310ns for ETU) in the power delay profile.

The test preambles for normal mode are listed in table [A.6-1.]

#### 11.4.1.2	Minimum requirement

The minimum requirement is in TS 36.108 [2] subclause [8.4.1.1] and [8.4.2.1].

#### 11.4.1.3	Test purpose

The test shall verify the receiver's ability to detect PRACH preamble under static conditions and multipath fading propagation conditions for a given SNR.

#### 11.4.1.4	Method of test

##### 11.4.1.4.1	Initial conditions

Test environment: Normal, see clause [D.2.]

RF channels to be tested for single carrier: [M, see clause 4.9.1.]

Direction to be tested: OTA REFSENS receiver target reference direction (see [D.44 in table 4.6-1]).

##### 11.4.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [D.7].

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [D.7]. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameter msg1-FrequencyStart is set to 0.

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

7)	Adjust the AWGN generator, according to the SCS and channel bandwidth. The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.4.1.4.2-1.

Table 11.4.1.4.2-1: AWGN power level at the SAN input

| Channel bandwidth (MHz) | AWGN power level |
| --- | --- |
| 1.4 | -89.7 - ΔOTAREFSENS dBm / 1.08MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |

8)	Adjust the frequency offset of the test signal according to table 11.4.1.5-1.

9)	Adjust the equipment so that the SNR specified in table 11.4.1.5-1 is achieved at the SAN input during the PRACH preambles.

10) The test signal generator sends a preamble and the receiver tries to detect the preamble. This pattern is repeated as illustrated in figure 11.4.1.4.2-1. The preambles are sent with certain timing offsets as described below. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 11.4.1.4.2-1: PRACH preamble test pattern

PRACH prambles are sent with a timing estimation error of 2.08us

#### 11.4.1.5	Test requirement

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in table 11.4.1.5-1.

Table 11.4.1.5-1: PRACH missed detection requirements for coverage enhancement (PRACH frequency hopping OFF)

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 1 | NTN-TDLA100-5 | 270 Hz | 8 | TBD | TBD | - | - |
|  |  |  |  | 16 | - | - | TBD | TBD |
|  | 2 | NTN-TDLA100-5 | 270 Hz | 8 | TBD | TBD | - | - |
|  |  |  |  | 16 | - | - | TBD | TBD |
| Note 1: Under fading channels, the PRACH detection performance may be significantly different with different PRACH Configuration Indexes. The requirements in this table are defined based on the simulation results with PRACH Configuration Indexes (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3 respectively. |  |  |  |  |  |  |  |  |

<End of change 6>

# <Start of change 7>

# A.5	Fixed Reference Channels for performance requirements (QPSK 1/3)

Table A.5-1 FRC parameters for performance requirements (QPSK 1/3)

| Reference channel | A5-1 | A5-2 |
| --- | --- | --- |
| Allocated resource blocks | 1 | 6 |
| DFT-OFDM Symbols per subframe | 12 | 12 |
| Modulation | QPSK | QPSK |
| Code rate | 1/3 | 1/3 |
| Payload size (bits) | 104 | 600 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Coded block size including 12bits trellis termination (bits) | 396 | 1884 |
| Total number of bits per sub-frame | 288 | 1728 |
| Total symbols per sub-frame | 144 | 864 |

# A.6	PRACH Test preambles

Table A.6-1 Test preambles for coverage enhancement

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 13 | 22 | 32 |
| 1 | 167 | 22 | 2 |
| 2 | 167 | 22 | 0 |
| 3 | 0 | 22 | 0 |

<End of change 7>

# <Start of change 8>

# Annex B (Normative): Propagation condition

# B.1	Static propagation condition

The propagation for the static performance measurement is an Additive White Gaussian Noise (AWGN) environment. No fading or multi-paths exist for this propagation model.

# B.2	Multi-path fading propagation conditions

The multipath propagation conditions consist of several parts:

-	A delay profile in the form of a "tapped delay-line", characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency.

## B.2.1	Delay profiles

The delay profiles are simplified from the TR 38.811 [13] TDL models. The simplification steps are described in TS 38.108 D.2 [2].

### B.2.1.1	Delay profiles for FR1

The delay profiles for FR1 are selected to be representative of low, medium and high delay spread environment. The resulting model parameters are specified in table B.2.1.1-1 and the tapped delay line models are specified in tables B.2.1.1-2 ~ B.2.1.1-4.

Table B 2.1.1-1: Delay profiles for NR channel models

| Model | Number of channel taps | Delay spread (r.m.s.) | Maximum excess tap delay (span) | Delay resolution |
| --- | --- | --- | --- | --- |
| NTN-TDLA100 | 3 | 100 ns | 285 | 5ns |
| NTN-TDLC5 | 2 | 5 ns | 60 | 5ns |

Table B.2.1.1-2: NTN-TDLA100 (DS = 100 ns)

| Tap # | Delay (ns) | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.7 | Rayleigh |
| 3 | 285 | -6.5 | Rayleigh |

Table B.2.1.1-3: NTN-TDLC5 (DS = 5 ns)

| Tap # | Delay (ns) | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |

## B.2.2	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., NTN-TDLA<DS>-<Doppler> or NTN-TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table B.2.2-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment.

Table B.2.2-1: Channel model parameters for FR1

| Combination name | Tapped delay line model | Maximum Doppler frequency |
| --- | --- | --- |
| [NTN-TDLA100-5] | NTN-TDLA100 | [5] Hz |
| [NTN-TDLC5-5] | NTN-TDLC5 | [5] Hz |

<End of change 8>