3GPP TSG-RAN WG4 Meeting #107 	R4-2307022

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on SAN demodulation

Agenda item:	9.7.7.2

Document for:	Discussion

Introduction

NB-IoT and eMTC operations were identified as two key enablers of connectivity in remote areas. In some of these areas, limited or no-cellular infrastructure might be available. Because of this, 3GPP has introduced IoT over NTN (Non-terrestrial networks) as potential solution to provide ubiquitous connectivity in such conditions [1]. Currently, the RAN plenary has approved the work item whose objective is to develop RF and RRM requirements for NB-IoT and eMTC operation over NTN [2]. The WID indicates that the discussions should leverage previous work for NR over NTN and IoT as long as applicable.

The current paper discusses how some of the demodulation aspects in uplink need to be considered for this current WI, as they are specific for NB-IoT over NTN.

Specifically, this paper discusses the open issues from RAN4 #106-bis-e [3].

Discussion

Open Issue Discussion

The demodulation work in RAN4 for the WID on IoT over NTN (LTE_NBIOT_eMTC_NTN) started at RAN4 #105, with discussions regarding UE demodulation aspects and some general concepts [4]. The following open issues are found on the way forward (WF) from RAN4 #106-bis-e:

## Sub-topic 3-1: PUSCH requirements Way forward: Updated simulation assumptions for PUSCH CE mode A (for alignment purpose) - Assume TxDuration is same as the number of repetitions Parameters; CE Mode A. Max number of HARQ transmissions; 4. RV sequences; 0, 2, 3, 1, 0, 2, 3, 1. Number of PUSCH repetitions; 8. Frequency hopping; OFF. Duplex mode; FDD only. Number of Tx antennas; 1. Number of Rx antennas; 1 and 2. Propagation channel; Option 1: NTN-TDLC5-5 Option 2: NTN-TDLA100-5. FRC; TS36.104 A3-2. System BW; 1.4MHz only. Fraction of maximum throughput; 70%. Way forward: Simulation assumption for PUSCH CE mode B (for alignment purpose) - Assume TxDuration is same as the number of repetitions Parameters; CE Mode B. Max number of HARQ transmissions; 2. RV sequences; 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1. Number of PUSCH repetitions; 256. Frequency hopping; OFF. Duplex mode; FDD only. Number of Tx antennas; 1. Number of Rx antennas; 1 and 2. Propagation channel; Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5. FRC; TS36.104 A3-1. System BW; 1.4MHz only. Fraction of maximum throughput; 70%. ## Sub-topic 4-1: NPUSCH format 1 requirements Way forward: Simulation assumptions for NPUSCH format 1 requirements (for alignment purpose) Number of TX antennas; Number of RX antennas; Subcarrier spacing; Number of allocated subcarriers; Propagation conditions; FRC; Repetition number; Fraction of maximum throughput. 1; 1 and 2; 3.75KHz; 1; Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1; TS36.104 A16-1; 4; 70%. 1; 1 and 2; 15KHz; 12; Option 1: NTN-TDLC5-1 Option 2: NTN-TDLA100-1; TS36.104 A16-5; 16; 70%.

As agreed at RAN4#106-bis-e both options for propagation conditions would remain open for NPUSCH and PUSCH requirements, with simulations being presented at RAN4#107 for discussion and ultimate inclusion within the specifications.

Nokia note that both propagation conditions emulate different deployment options for NTN deployments, with NTN-TDLA100 channels representing a more rural deployment, and NTN-TDLA100 representing a deployment scenario which could be in an urban setting.

Therefore, Nokia propose that both propagation conditions are presented for requirements in TS 36.108 and TS 36.181.

For PUSCH and NPUSCH requirement, NTN-TDLA100 channels representing a more rural deployment, and NTN-TDLC5 represents a deployment scenario which could be in an urban setting.

For PUSCH requirements, RAN4 shall have requirement for both propagation conditions (NTN-TDLC5-5 and NTN-TDLA100-5), which shall be included in TS 36.108 and TS 36.181.

For NPUSCH requirements, RAN4 shall have requirement for both propagation conditions (NTN-TDLC5-1 and NTN-TDLA100-1), which shall be included in TS 36.108 and TS 36.181.

Radiated requirements for TS 36.181

As part of the Work Split as updated at RAN4#106-bis-e, Nokia have undertaken to write the Radiated requirements for TS 36.181, several queries and discussion matters have been raised through this activity that will require discussion at RAN4#107.

PUSCH Applicability Rules

If TS 38.181 is taken as a basis for Applicability rules for PUSCH within TS 36.181 applicability rules would be focused on differing subcarrier spacings, channel bandwidths and configurations. As subcarrier spacing and channel bandwidth are fixed at 15 kHz SCS and 1.4 MHz with LTE NTN we believe PUSCH applicability rules may be required for CE Mode A and CE Mode B.

We believe that some PUSCH applicability rules may be needed for TS 36.181, specifically around different configurations.

RAN4 to decide whether PUSCH format (CE A vs. B) applicability rules are required for TS 36.181.

Should a PUSCH applicability rule be required for CE Mode A and Mode B we propose the below wording

##### 11.1.3.2.3 Applicability of requirements for different configurations Unless otherwise stated, PUSCH requirement tests shall apply only for the mapping type declared to be supported (see [D.100] in table [4.6-1]). If both mapping type A and type B are declared to be supported, the tests shall be done for either type A or type B; the same chosen mapping type shall then be used for all tests.

If PUSCH format applicability rule inclusion is agreed, follow NR NTN and use the following wording in a new “Applicability of requirements for different configurations” section: 
“Unless otherwise stated, PUSCH requirement tests shall apply only for the mapping type declared to be supported (see [D.100] in table [4.6-1]). If both mapping type A and type B are declared to be supported, the tests shall be done for either type A or type B; the same chosen mapping type shall then be used for all tests.”.

PUCCH Applicability Rules

If TS 38.181 is taken as a basis for Applicability rules for PUCCH within TS 36.181 applicability rules would be focused on differing formats, subcarrier spacings, channel bandwidths, configuration, and multi-slot. As these are fixed at PUCCH Format 1a, 15 kHz SCS, 1.4 MHz for PUCCH with LTE NTN with LTE NTN we believe PUSCH applicability rules may be required for multi-slot.

We believe that some PUCCH applicability rules are needed for TS 36.181, specifically around multi-slot

RAN4 to decide whether PUCCH multi-slot applicability rules are required for TS 36.181

Should a PUCCH applicability rule be required for multi-slot we propose the below wording

##### 11.1.3.3.5 Applicability of requirements for multi-slot PUCCH Unless otherwise stated, multi-slot PUCCH requirement tests shall apply only if the SAN supports it (see [D.105] in table [4.6-1]).

If PUCCH multi-slot applicability rule inclusion is agreed, follow NR NTN and use the following wording in a new “Applicability of requirements for multi-slot PUCCH” section: 
“Unless otherwise stated, multi-slot PUCCH requirement tests shall apply only if the SAN supports it (see [D.105] in table [4.6-1])”.

PRACH Applicability Rules

If TS 38.181 is taken as a basis for Applicability rules for PRACH within TS 36.181 applicability rules would be focused on differing configurations, subcarrier spacings, and channel bandwidths. As subcarrier spacing and channel bandwidth is fixed at 1.4 MHz and 15 kHz SCS for PRACH with LTE NTN we believe PUSCH applicability rules may be required for different formats.

We believe that some PRACH applicability rules may be needed for TS 36.181, specifically around different formats

RAN4 to decide whether PRACH format applicability rules are required for TS 36.181

Should a PRACH applicability rule be required for different formats we propose the below wording.

##### 11.1.3.4.1 Applicability of requirements for different formats Unless otherwise stated, PRACH requirement tests shall apply only for each PRACH format declared to be supported (see [D.102] in table [4.6-1]).

If PRACH format applicability rule inclusion is agreed, follow NR NTN and use the following wording in a new “Applicability of requirements for different formats” section: 
“Unless otherwise stated, PRACH requirement tests shall apply only for each PRACH format declared to be supported (see [D.102] in table [4.6-1]).

Test Tolerance on TS 36.181

Whilst developing the pCR for TS 36.181 we note that within the work split no test tolerance activity is specified. Whereas this will be explicitly included in clauses relating to conducted and radiated performance. We believe this should be included in TS 36.181.

RAN4 shall introduce Test Tolerances for conducted and radiated performances within TS 36.181

Format and Wording for inclusion of Timing and Frequency Offset in TS 36.181

Whilst developing the pCR for TS 36.181 we note the discrepancy in the format with whicn the Frequency and Timing offsets are presented for PUSCH, PUCCH and PRACH.

Within PRACH this is described as a simple sentence, whereas within PUSCH and PUCCH this is a table

Specifically for the Frequency offset in PUSCH and PUCCH whilst the number of repetitions and Tx duration in one segment is useful we note this could be considered supplementary information which is not required for the test. Therefore we propose that this is discussed at RAN4#107.

RAN4 to decide format for inclusion of frequency and timing offsets for PUSCH and PUCCH for TS 36.181.

Conclusion

Within this paper we have presented a view on the SAN demodulation.

Specifically, in the paper, the following Observations and Proposals were made:

Observation 1: NTN-TDLA100 channels representing a more rural deployment, and NTN-TDLA100 represents a deployment scenario which could be in an urban setting.

Proposal 1: For PUSCH requirements, RAN4 shall have requirement for both propagation conditions (NTN-TDLC5-5 and NTN-TDLA100-5), which shall be included in TS 36.108 and TS 36.181.

Proposal 2: For NPUSCH requirements, RAN4 shall have requirement for both propagation conditions (NTN-TDLC5-1 and NTN-TDLA100-1), which shall be included in TS 36.108 and TS 36.181.

Observation 2: We believe that some PUSCH applicability rules may be needed for TS 36.181, specifically around different configurations.

Proposal 3: RAN4 to decide whether PUSCH format (CE A vs. B) applicability rules are required for TS 36.181.

Proposal 4: If PUSCH format applicability rule inclusion is agreed, follow NR NTN and use the following wording in a new “Applicability of requirements for different configurations” section:  “Unless otherwise stated, PUSCH requirement tests shall apply only for the mapping type declared to be supported (see [D.100] in table [4.6-1]). If both mapping type A and type B are declared to be supported, the tests shall be done for either type A or type B; the same chosen mapping type shall then be used for all tests.”.

Observation 3: We believe that some PUCCH applicability rules are needed for TS 36.181, specifically around multi-slot

Proposal 5: RAN4 to decide whether PUCCH multi-slot applicability rules are required for TS 36.181

Proposal 6: If PUCCH multi-slot applicability rule inclusion is agreed, follow NR NTN and use the following wording in a new “Applicability of requirements for multi-slot PUCCH” section:  “Unless otherwise stated, multi-slot PUCCH requirement tests shall apply only if the SAN supports it (see [D.105] in table [4.6-1])”.

Observation 4: We believe that some PRACH applicability rules may be needed for TS 36.181, specifically around different formats

Proposal 7: RAN4 to decide whether PRACH format applicability rules are required for TS 36.181

Proposal 8: If PRACH format applicability rule inclusion is agreed, follow NR NTN and use the following wording in a new “Applicability of requirements for different formats” section:  “Unless otherwise stated, PRACH requirement tests shall apply only for each PRACH format declared to be supported (see [D.102] in table [4.6-1]).

Proposal 9: RAN4 shall introduce Test Tolerances for conducted and radiated performances within TS 36.181

Proposal 10: RAN4 to decide format for inclusion of frequency and timing offsets for PUSCH and PUCCH for TS 36.181.

- References

- RP-211601, WID on NB-IoT/eMTC support for NTN, Mediatek Inc & Eutelsat S.A, TSG RAN #92e, June 2021

- RP-220938, New WID on NB-IoT/eMTC core & performance requirements for NTN, Mediatek Inc, TSG RAN #95e March 2022

- R4-2305965, WF on IoT NTN SAN demodulation requirements, RAN4 WG 4 #106-bis-e, Electronic Meeting, 17 April ‒ 26 April 2023

- R4-2220278, WF on IoT-NTN UE demodulation and CQI reporting requirements, MediaTek Inc., RAN WG4 #105, Toulouse, Nov. 2022

- 3GPP TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”, 3GPP, Release 17.2.0