3GPP TSG-RAN WG4 Meeting #107 	R4-2307024

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on PDSCH Demodulation Requirements for 8Rx

Agenda item:	8.5.3.1.2

Document for:	Discussion

Introduction

This document extends the discussion on the 8Rx UE demodulation and CSI requirements introduced in RAN4 #105 summarized in [1], discussions were continued at RAN4 #106 where the way forward was summarized in [3].

The agreements reached during the last meeting regarding both 8 RX and CSI requirements are captured on the WFs [2] and [8]

The major open topics being:

- PDSCH requirements - SDR requirements - CQI requirements

This paper presents Nokia’s views on the open issues related to the 8Rx UE PDSCH demodulation, extending the previous discussion introduced in [7], and [9]. We also discuss the critical outcome include two different MCS requirements in case of 2 CWs to model real deployment cases for 8 Rx.

This discussion is supported by simulation results provided in [5].

The following items were agreed at RAN4#106-bis-e regarding PDSCH requirements [8], whilst a number of issues remain open, which will be discussed in this contribution.

Issue 2-1: Maximum number of OFDM symbols (maxLength) for DL - Single-symbol for Rank not larger than 4, double-symbol for Rank larger than 4. Issue 2-2: Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx - Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 Issue 2-6: N1 and N2 configuration for 8Tx case - Use (N1,N2) = (4,1), (O1, O2) = (4,1) Issue 2-8: Additional test for two MCS configuration for Rank 8 test - Don’t introduce rank8 test with two different MCS

Discussion

Within this paper, we discuss the parameters defined for simulation and definition of requirements for PDSCH within RAN4#106 and refined at RAN4#106-bis-e, furthermore we make a series of observations and recommendations based on Nokia’s activities thus far.

Discussion within this contribution will be focussed on open issues following the WF identified at RAN4 #106-bis-e [8].

Issue 2-3: Propagation conditions for Rank 2 test

Following RAN4 #106-bis-e two options were presented in the WF for propagation conditions for Rank 2 tests, these were as follows.

- Option 1: TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium B - Option 2: Only TDLC300-100 ULA Medium B

As agreed at RAN4 #106 [3], PDSCH requirements will be defined in different correlation correlations and configurations, specifically, ULA Medium B for Rank 2 and as was agreed both TDLC 300-100 and TDLA 30-10.

TDLA 30-10 supports the FWA use cases as defined within the WI and should be defined for Rank 2, as they are for Rank 4 and Rank 8.

Furthermore, as our simulations in our companion simulation TDoc [5] show, all MCS choices are able to provide a 70% TPUT at a feasible SNR.

Prior agreements from RAN4#106 include both TDLC 300-100 and TDLA 30-10.

TDLA 30-10 requirements are defined for all other Ranks within 8 Rx

TDLC300-100 propagation conditions provide feasible test points for all MCS.

We propose that Option 1 shall be taken forward, which includes both TDLC 300-100 and TDLA 30-10 ULA Medium B.

Issue 2-4: MCS and antenna correlation for Rank 2 test

Following RAN4 #106-bis-e two sets of two options were presented in the WF for MCS for Rank 2 tests (corresponding to the two different channel propagation conditions), these were as follows.

- Proposals for TDLC300-100 ULA Medium B - Option 1: MCS 2, 7 (Table 2) - Option 2: MCS 19 (Table 1) - Proposals for TDLA30-10 ULA Medium B (if agreed) - Option 1: MCS 13, 26 (Table 2) - Option 2: MCS 19 (Table 1)

Option 2 for TDLC300-100 ULA Medium B does provide testable results, as is shown in the companion TDoc [5] therefore in these propagation and correlation conditions this provides a reasonable MCS, however we are happy to also use a lower MCS.

However, in TDLA30-10 ULA Medium B MCS 19 (Table 1) did provide a reasonable testable result therefore this could be an option for this channel.

MCS 19 (Table 1) does provide a testable and reasonable result in TDLC 300-100 ULA Medium B conditions.

We propose that either option 1 MCS 2, 7 (Table 2) or option 2 MCS 19 (Table 1) shall be taken forward for TDLC300-100 ULA Medium B, with all MCSs providing a reasonable test point.

MCS 19 (Table 1) does provide a reasonable testable result in TDLA 30-10 ULA Medium B conditions.

We propose that either option 1 MCS 13,26 (Table 2) or option 2 MCS 19 (Table 1) shall be taken forward for TDLA30-10 ULA Medium B, with all MCSs providing a reasonable test point.

Issue 2-5: MCS for Rank 4 test

Following RAN4 #106-bis-e two sets of two options were presented in the WF for MCS for Rank 4 tests, these were as follows.

- Option 1: MCS 13 and MCS26 (Table 1) - Option 2: Only MCS17 (Table 1)

As discussed at RAN4#106-bis-e, Nokia would prefer Rank 4 to have performance specified for a 64QAM modulation scheme at a reasonable testing point. Both option 1 (with MCS 26) and option 2 (with MCS 17) meet this requirement.

Both option 1 (with MCS 26) and option 2 (with MCS 17) provide 64QAM modulation options for defining performance.

We propose that either option is suitable however, to provide diversity of modulation order and max throughput RAN4 shall use option 1 with MCS 13 and MCS 26.

Issue 2-7: Single MCS configuration for Rank 8 test

Following RAN4 #106-bis-e two sets of two options were presented in the WF for MCS for Rank 8 tests, these were as follows.

- Option 1: MCS13 - Option 2: MCS17

As discussed at RAN4#106-bis-e, Nokia, both MCS 13 and MCS 17 are suitable MCS choices for testing Rank 8; noting that most companies following the 2nd round at RAN4#106-bis-e opted for MCS 13, and as this provides a reasonable performance, we are happy to agree that this MCS for use for performance definition.

Both MCS 13 and MCS 17 provide reasonable performance based on simulation, therefore either could be chosen to define the performance of Rank 8 tests.

Most companies opted for MCS 13 at Round 2 of RAN4#106-bis-e.

We propose that MCS 13 shall be used for Rank 8 Performance requirements.

Summary of MCS, Channel Correlation and Propagation Conditions for PDSCH

In summary Table 1 provides an overview of all the proposed MCS, Channel Correlation and Propagation Conditions for PDSCH following RAN4 #106-bis-e, those highlighted in green and bold are those which Nokia has a preference for, and which subsequently we have provided results for in our companion TDoc [5].

Table 1 : Summary of MCS, Channel Correlation and Propagation Conditions for PDSCH

| Rank | MCS | Channel Correlation | Propagation Condition |
| --- | --- | --- | --- |
| 2 | 2 (Table 2) | ULA Medium B | TDLA 30-10 |
|  |  |  | TDLC 300-100 |
|  | 7 (Table 2) |  | TDLA 30-10 |
|  |  |  | TDLC 300-100 |
|  | 19 (Table 1) |  | TDLA 30-10 |
|  |  |  | TDLC 300-100 |
| 4 | 13 (Table 1) | ULA Low | TDLA 30-10 |
|  | 17 (Table 1 |  |  |
|  | 26 (Table 1) |  |  |
| 8 | 13 (Both CW) (Table 1) |  |  |
|  | 17 (Both CW) (Table 1) |  |  |

Conclusion

In this paper, we have assessed the current direction of the 8Rx WI, specifically PDSCH UE Demodulation performance. We have provided recommendations where agreements need to be made and requirements defined. Furthermore, we have provided updated results and analysis based on prior agreements at RAN4 #1060-bis-e in order to specify the performance of PDSCH demodulation.

Specifically, in the paper, the following Observations and Proposals were made:

Observation 1: Prior agreements from RAN4#106 include both TDLC 300-100 and TDLA 30-10.

Observation 2: TDLA 30-10 requirements are defined for all other Ranks within 8 Rx

Observation 3: TDLC300-100 propagation conditions provide feasible test points for all MCS.

Proposal 1: We propose that Option 1 shall be taken forward, which includes both TDLC 300-100 and TDLA 30-10 ULA Medium B.

Observation 4: MCS 19 (Table 1) does provide a testable and reasonable result in TDLC 300-100 ULA Medium B conditions.

Proposal 2: We propose that either option 1 MCS 2, 7 (Table 2) or option 2 MCS 19 (Table 1) shall be taken forward for TDLC300-100 ULA Medium B, with all MCSs providing a reasonable test point.

Observation 5: MCS 19 (Table 1) does provide a reasonable testable result in TDLA 30-10 ULA Medium B conditions.

Proposal 3: We propose that either option 1 MCS 13,26 (Table 2) or option 2 MCS 19 (Table 1) shall be taken forward for TDLA30-10 ULA Medium B, with all MCSs providing a reasonable test point.

Observation 6: Both option 1 (with MCS 26) and option 2 (with MCS 17) provide 64QAM modulation options for defining performance.

Proposal 4: We propose that either option is suitable however, to provide diversity of modulation order and max throughput RAN4 shall use option 1 with MCS 13 and MCS 26.

Observation 7: Both MCS 13 and MCS 17 provide reasonable performance based on simulation, therefore either could be chosen to define the performance of Rank 8 tests.

Observation 8: Most companies opted for MCS 13 at Round 2 of RAN4#106-bis-e.

Proposal 5: We propose that MCS 13 shall be used for Rank 8 Performance requirements.

- References

- RP-221496 “Revised WID Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”

- R4-2220152, “Topic summary for [105][326] RF_FR1_enh2_Demod”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106, Athens, Greece, 27 February - 03 March, 2023

- R4-2304097, “PDSCH Requirements for 8Rx UE performance”, Nokia, Nokia Shanghai Bell, RAN WG4 #106-bis-e, Online, April 17 – April 26, 2023

- R4-2307025 “Simulation results for 8Rx UE demodulation”, Nokia, Nokia Shanghai Bell, RAN4 #107, Incheon, Korea, May 22 – 26, 2023

- TS 38.211 v17.4.0, Technical Specification Group Radio Access Network; NR; Physical channels and modulation (Release 17), December 2022

- R4-2300612, Discussion on 8Rx UE demodulation and CSI requirements, Nokia, Nokia Shanghai Bell, RAN4 #106, Athens, Greece, February 27th ‒ March 3rd, 2023.

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106-bis-e, Online, April 17 – April 26, 2023

- R4-2304097 “PDSCH Requirements for 8Rx UE performance”, RAN4 #106-bis-e, Electronic Meeting, April 17 – 26, 2023.