3GPP TSG-RAN WG4 #107	R4-2307049

May 22th ‒ May 26th, 2023

Incheon, KR

Agenda item:	9.3.3

Source: 	SWR, EBU

Title: 	UE RF requirements for 5G Broadcast

Document for:	Discussion

# Introduction

In the scope of Release 18, the main objective of the work item [1] under discussion in RAN4 is to identify one or more bands within the UHF frequency range enabling 5G broadcast operation and to establish the corresponding UE RF requirements. This contribution responds to the WF document of the RAN4#106bis-e [2].

# Discussion

After extensive discussion on the band definition for 5G broadcast in the UHF frequency range and corresponding UE RF requirements, RAN4#106bis-e decided on the following way forward [2] for the UE RF part of WI [1]:

2.1	Specification of channel bandwidth

Channel bandwidth of 10 MHz for the UE has been assumed, but there are differing views on whether this detail needs to be included in the specification.

-	Option 1: Include 10 MHz channel bandwidth and 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth (a la R4-2305824)

-	Option 2: Do not include 10 MHz channel bandwidth as this is an implementation detail.  The specification shall only mention 6, 7, and 8 MHz 5G terrestrial broadcast bandwidths

-	Option 3: Include 10 MHz channel bandwidth, but instead of referring to 6, 7, and 8 MHz 5G terrestrial broadcast bandwidth, refer to PMCH-bandwidth of 30, 35, and 40 RB’s as specified in 36.331.

Way forward:  FFS these options

2.2	Refsens

-	The 470 – 698 MHz band and/or the 612 – 652 MHz band is specified with

Option 1.  6, 7, and 8 MHz PMCH for reference sensitivity

Option 2.  10 MHz PDSCH for reference sensitivity

Way forward:  FFS these options

2.3	ACS

Option 1:  33 dB (for all 6, 7, and 8 MHz bandwidths)

Option 2:  29 dB (for all 6, 7, and 8 MHz bandwidths)

Option 3:

| Channelization | Power in transmission bandwidth configuration | PInterferer | ACS |
| --- | --- | --- | --- |
| 10 MHz (baseline) | REFSENS + 14 dB | REFSENS + 45.5 | 33 dB |
| 6 MHz | REFSENS + 14 dB | REFSENS + 28.5 | [16] dB |
| 7 MHz | REFSENS + 14 dB | REFSENS + 30.0 | [17.5] dB |
| 8 MHz | REFSENS + 14 dB | REFSENS + 35.0 | [22.5] dB |

Way forward:  FFS these options

# Proposals on WF

3.1	Specification of channel bandwidth

Option 2 is the preferred one. The term "channel bandwidth“ needs to be defined properly. For 5G Broadcast the carrier bandwidth is 6, 7 or 8 MHz in order to fit into the underlying channel raster of the UHF band 470-694/698 MHz. In all cases the CAS symbol is only 5 MHz wide. The only point where 10 MHz comes into play is the bandpass filter of the UE which according to the WID should be reused. Furthermore, in the WID [1], it is clearly expressed that the objective is not to specify a 10MHz channel bandwidth but how to implement the UE RF requirements for 6/7/8 MHz.

Option 1 and 3 are essentially the same, there is no difference apart from wording. However, both options do not the reflect the situation correctly.

Proposal:	Use Option 2

3.2	Refsens

Option 2 is not applicable for 5GBC. The occupied bandwidth of a 5GBC carrier is 6, 7 and 8 MHZ for PMCH and 5 MHz for PDSCH. Therefore, it does not make sense to use 10 MHz for PDSCH.

Proposal:	Use Option 1

3.3	ACS

Option 1 is the preferred one. In the light of reusing existing UE RF requirements as much as possible, 33 dB could be an acceptable compromise. However, a value of 33 dB already imposes severe constraints on network deployment for 5G Broadcast. Relaxation of the ACS value will induce substantial network deployment effort and hence driving network costs. Annex 1 contains a more elaborate analysis of the situation.

Proposal:	Use Option 1

# Conclusion

It is proposed to approve the following options of the WF document [2]:

Specification on channel bandwidth:	Option 2

Refsens:				Option 1

ACS:					Option 1

# Reference

- RP-220518, “Revised WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2”, EBU, Qualcomm., 3GPP TSG RAN Meeting #95e, Electronic Meeting, March 17 - 23, 2022

- R4-2306566, “WF on UE RF requirements for 5G broadcast” 3GPP RAN4 Meeting, online, April 17th ‒ April 26th, 2023

# Annex 1

ACS specification of 5G Broadcast for 6, 7 and 8 MHz carriers

A1.1	Introduction

The way forward [2] for the UE RF part of WI [1] proposed by 3GPP TSG-RAN WG4 Meeting #106bis-e outlines the following options for the ACS specifications:

- Option 1: 33 dB (for all 6, 7, and 8 MHz bandwidths)

- Option 2: 29 dB (for all 6, 7, and 8 MHz bandwidths)

- Option 3:

| Channelization | Power in transmission bandwidth configuration | PInterferer | ACS |
| --- | --- | --- | --- |
| 10 MHz (baseline) | REFSENS + 14 dB | REFSENS + 45.5 | 33 dB |
| 6 MHz | REFSENS + 14 dB | REFSENS + 28.5 | [16] dB |
| 7 MHz | REFSENS + 14 dB | REFSENS + 30.0 | [17.5] dB |
| 8 MHz | REFSENS + 14 dB | REFSENS + 35.0 | [22.5] dB |

RAN4 invited the participating companies to provide further studies for all the options.

A1.2	Proposal

The ACS capability of the UE is estimated assuming a conventional LTE 10 MHz channel filter design, but subjected to 6, 7, and 8 MHz channelization used for broadcast. The (worst case) 6 MHz scenario as reported in Figure 1 has been considered. Both wanted and interfering signals having SCS= 1.25 kHz have been used while keeping the receiver channel filter at 10 MHz.

As required in TS 136.101, a small offset  Δ =SCS/2 is ideally applied to introduce loss of orthogonality (FFT leakage). Also, the absolute signal levels are ideally assumed to be much greater than the thermal noise level, in order to neglect the latter.

As already known, the considered arrangement with 10 MHz filter causes a portion of interfering signal to get into the filter passband. This interfering component, consisting in Nc subcarriers, is suppressed after the FFT. However, the FFT leakage from the interferer is still present. FFT leakage has been calculated by summing up Nc shifted sinc functions:

Roughly, the FEC decoding with code rate R is able to recover up to R*100% interfered subcarriers. An individual subcarrier can be considered interfered if it experiences a local SNR value below the in-channel SNR requirement. This has been adopted as a mean to estimate the system threshold in terms of signal-to-interference ratio, at a given MCS code rate R.

Hence, to assess the UE’s ACS, for the given SCS and Nc, the FFT leakage has been calculated and plotted in the frequency range [-6 MHz, 0], with the same power spectral density (PSD) level as the wanted signal (co-located scenario); then, it has been ideally increased until system threshold is reached, based on the above criterion. The ACLR contribution, assumed to be that of the interfering transmitter OOB emission mask, are always negligible w.r.t FFT leakage.

Figure 1: Interfering scenario for ACS assessment

Figure 2: Graphical evaluation of the system threshold and ACS.

As shown in Figure 2, the red curve represents the FFT leakage while the purple one the ACLR mask. MCS 12, R=0.42 has been assumed as a typical 5GBC use case, corresponding to an in-channel SNR requirement of about 12dB. The above-mentioned criterion requires that 0.42*100%=42% of carriers to be over SINR threshold: in terms of band, this means 5.4 MHz*0.42=2.27 MHz. From the graph, we identify the system threshold as -47dB. Therefore, -47-(-12)= -35dB. In other words, “in order to bring the receiver to the out-of-service threshold, the interferer can be increased by 35dB”. The latter value is then an approximate estimation of the ACS.

In a real terrestrial propagation environment, it may happen that, due to frequency-selective fading, the wanted signal experiences attenuation while the adjacent interferer does not. Thus, the apparently satisfactory ACS value can actually be substantially reduced Also, two (left and right) adjacent interferers are possibly present, leading to a few dB more demanding protection.