3GPP TSG-RAN WG4 Meeting # 107	R4-2307059

# Incheon, KR, May 22 – May 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	n71 PC2 RSD for 25, 30 and 35MHz symmetrical ULDL CBW

Agenda Item:	7.22.2	UE RF requirements	[HPUE_NR_FR1_FDD_R18]

Document for:	Information

# Introduction

In RAN#99, new symmetrical UL/DL 25MHz and 30MHz CBWs are requested for Band n71 in [1]. The impact of these requests were discussed in RAN4#106b-e in [4] and resulted in two WF [2, 3] for the impacted WIs for PC3 and PC2 n71 requirements. Separately, the proponent for these new symmetrical CBWs expressed the need to also cover 35MHz symmetrical UL/DL. Given the dependency between the PC2 RSD and PC3 REFSENS, the PC2 RSD is proposed in the “Adding new channel BWs support to existing NR bands” AI in [6]. In this contribution, we provide the proposed PC2 RSD for information.

# Discussion

In RAN#99 a new request was made for the support of symmetrical UL/DL 25MHz and 30MHz CBW for Band n71 in [1].

In RAN4#106b-e, contribution [4] identified the need to cover the related PC2 RSD and other impacted requirements like NS-35 and CA_n71-n85 BCS4/5

After further discussions in RAN4#106b-e, the proponent for the new symmetrical UL/DL CBW also expressed the need for supporting symmetrical UL/DL 35MHz CBW for Band n71.

There are conflicting agreements on which WI, AI should handle the PC2 RSD:

- WF [2] in “Adding new channel BWs support to existing NR bands” says:

  - For Release 18 completeness the PC2 1Tx and 2Tx RSDs should be specified for the optional symmetrical UL/DL 25MHz and 30MHz case in the High power UE for FR1 for FDD single band(s) with power class 2 WI.

- WF [3] in “High power UE for FR1 for FDD single band(s) with power class 2” says:

  - On n8 25MHz: Cover PC2 for n8 25MHz in FDD PC2 HPUE WI, but this does not prevent/delay completion for other potential completed CBWs.

  - On n71 25/30MHz and other potential new CBWs of other FDD bands: Keep the relevant PC2 work within the “Adding new channel BWs support to existing NR bands” WI.

Given that:

- WF [3] suggests, except for n8, to handle PC2 RSD related to new requested CBW in “Adding new channel BWs support to existing NR bands” as a generic approach.

- There are identified issues on how to handle the requirement for the same DL CBW with two UL CBW.

- The RSD is on top of PC3 REFSENS that is agreed in “Adding new channel BWs support to existing NR bands” WI.

We provide our PC2 PSD proposal here and provide the proposed RSD values for information to the “High power UE for FR1 for FDD single band(s) with power class 2”.

Observation: In the “Adding new channel BWs support to existing NR bands”, the following handling proposal is made: Since PC2 RSD is on top of PC3 REFSENS that must be agreed upon in the “Adding new channel BWs support to existing NR bands” WI, we propose to agree PC3 REFSENS and PC2 RSD in the “Adding new channel BWs support to existing NR bands” WI to avoid the “chicken and egg” issue. The agreements can be captured in the respective BigCRs.

## PC2 25, 30 and 35MHz symmetrical UL/DL CBW RSD

Based on the averaged PC3 REFSENS from [4,5], the 1TX and 2TX RSDs can be re-calculated from the measurements shown in [4]. Similarly, the 35MHz case can be estimated based on the observation that the interference is the same as that for the 30MHz case. The evaluated values are provided in the table below.

Table 4: RSD values for 1Tx and 2Tx PC2 symmetrical UL/DL CBW

| TX configuration | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) |
| --- | --- | --- | --- |
| 1Tx | 2.8 | 3.0 | [3.0] |
| 2Tx | 5.9 | 6.1 | [6.1] |

Similar to what is proposed for REFSENS, we propose to capture the RSD requirements for the optional symmetrical UL/DL CBW in 38.101-1 related tables together with the baseline requirement (still pending agreement in the PC2 FDD WI) with the appropriate notes. The proposed tables use the current averaged values for the baseline requirement in brackets. Note that in some cases, the RSD for the symmetrical UL/DL CBW is lower than that for the baseline case. This is explained by the averaging of different values, but more accurately by the fact that the symmetrical UL/DL CBW REFSENS is much worse than the baseline REFSENS.

Observation: In the “Adding new channel BWs support to existing NR bands”, the following proposal on PC2 1Tx and 2Tx RSDs is presented:

- 1Tx and 2Tx RSDs for the symmetrical UL/DL 25, 30 and 35MHz CBW are specified in Table 5 and Table 6 below (yellow highlight). Other requirements are in brackets and correspond to the average of proposals. These need to be formally agreed in the related PC2 FDD WI (blue highlight).

- Notes are added to clarify applicability to UEs supporting the optional symmetrical UL/DL bandwidths by signaling the associated channelBWs-UL IE.

- 35MHz RSDs shall not be captured in 38.101-1, until this new CBW is formally approved in RAN.

- Note that RSD is on top of REFSENS, thus RSD may need to be recalculated if the agreed REFSENS is different from the one proposed in this contribution.

Table 5: Reference Sensitivity Degradation from PC3 to PC2 for n71 for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | [0.5] | [0.9] | [0.9] | [2.2] | [2.4] 2.82 | [2.5] 3.02 | [2.9] [3.02] | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |

Table 6: Reference Sensitivity Degradation from PC3 to PC2 for n71 for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | [1.1 | [1.1 | [1.7] | [5.5] | [5.9] 5.92 | [6.2] 6.12 | [6.5] [6.12] | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, we provide the PC2 RSD values proposed for the optional symmetrical UL/DL CBW cases in the “Adding new channel BWs support to existing NR bands” AI for information.

Observation: In the “Adding new channel BWs support to existing NR bands”, the following handling proposal is made: Since PC2 RSD is on top of PC3 REFSENS that must be agreed upon in the “Adding new channel BWs support to existing NR bands” WI, we propose to agree PC3 REFSENS and PC2 RSD in the “Adding new channel BWs support to existing NR bands” WI to avoid the “chicken and egg” issue. The agreements can be captured in the respective BigCRs.

Observation: In the “Adding new channel BWs support to existing NR bands”, the following proposal on PC2 1Tx and 2Tx RSDs is presented:

- 1Tx and 2Tx RSDs for the symmetrical UL/DL 25, 30 and 35MHz CBW are specified in Table 5 and Table 6 below (yellow highlight). Other requirements are in brackets and correspond to the average of proposals. These need to be formally agreed in the related PC2 FDD WI (blue highlight).

- Notes are added to clarify applicability to UEs supporting the optional symmetrical UL/DL bandwidths by signaling the associated channelBWs-UL IE.

- 35MHz RSDs shall not be captured in 38.101-1, until this new CBW is formally approved in RAN.

- Note that RSD is on top of REFSENS, thus RSD may need to be recalculated if the agreed REFSENS is different from the one proposed in this contribution.

Table 5: Reference Sensitivity Degradation from PC3 to PC2 for n71 for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | [0.5] | [0.9] | [0.9] | [2.2] | [2.4] 2.82 | [2.5] 3.02 | [2.9] [3.02] | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |

Table 6: Reference Sensitivity Degradation from PC3 to PC2 for n71 for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | [1.1 | [1.1 | [1.7] | [5.5] | [5.9] 5.92 | [6.2] 6.12 | [6.5] [6.12] | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |

# References

- RP-230756 Revised Basket WID: Core part: Adding new channel BWs support to existing NR bands, Ericsson, RAN#99

- R4-2306545 WF on adding new channel bandwidths support to existing NR bands, Ericsson, RAN4#106b-e

- R4-2306544 WF on FDD HPUE Basket WIs, China Unicom, RAN4#106b-e

- R4-2304559 Impact of 25 and 30MHz ULDL CBW for n71, Skyworks Solutions Inc., RAN4#106b-e

- R4-2304349 On RF requirements for 25MHz and 30MHz UL CBW for n7, Apple, RAN4#106b-e

- R4-2307060 n71 REFSENS and RSD for 25 30 and 35MHz symmetrical ULDL CBW, Skyworks Solutions Inc., RAN4#107