3GPP TSG-RAN WG4 Meeting # 107	R4-2307060

# Incheon, KR, May 22 – May 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	n71 REFSENS and RSD for 25, 30 and 35MHz symmetrical ULDL CBW

Agenda Item:	7.26.2.1	Single band requirements	[NR_bands_R18_BWs-Core]

Document for:	Approval

# Introduction

In RAN#99, new symmetrical UL/DL 25MHz and 30MHz CBW are requested for Band n71 in [1]. The impact of these requests were discussed in RAN4#106b-e in [4] and this resulted in two WF [2, 3] for the impacted WIs for PC3 and PC2 n71 requirements. Separately, the proponent for these new symmetrical CBWs expressed the need to also cover 35MHz symmetrical UL/DL. In this contribution, we provide our input on PC3 REFSENS and PC2 RSD based on [4, 5], and also provide preliminary input to the 35MHz case.

# Discussion

In RAN#99, a new request was made for the support of symmetrical UL/DL 25MHz and 30MHz CBW for Band n71 in [1].

In RAN4#106b-e, contribution [4] identified the need to cover the related PC2 RSD and other impacted requirements, such as NS-35 and CA_n71-n85 BCS4/5.

After further discussions in RAN4#106b-e, the proponent for the new symmetrical UL/DL CBW also expressed the need for supporting symmetrical UL/DL 35MHz CBW for Band n71.

There are conflicting agreements on which WI, AI should handle the PC2 RSD:

- WF [2] in “Adding new channel BWs support to existing NR bands” says:

  - For Release 18 completeness the PC2 1Tx and 2Tx RSDs should be specified for the optional symmetrical UL/DL 25MHz and 30MHz case in the High power UE for FR1 for FDD single band(s) with power class 2 WI.

- WF [3] in “High power UE for FR1 for FDD single band(s) with power class 2” says:

  - On n8 25MHz: Cover PC2 for n8 25MHz in FDD PC2 HPUE WI, but this does not prevent/delay completion for other potential completed CBWs.

  - On n71 25/30MHz and other potential new CBWs of other FDD bands: Keep the relevant PC2 work within the “Adding new channel BWs support to existing NR bands” WI.

Given that:

- WF [3] suggests, except for n8, to handle PC2 RSD related to new requested CBW in “Adding new channel BWs support to existing NR bands” as a generic approach.

- There are identified issues on how to handle the requirement for the same DL CBW with two UL CBW.

- The RSD is on top of PC3 REFSENS that is agreed in “Adding new channel BWs support to existing NR bands” WI.

We provide our PC2 PSD proposal here and provide the proposed RSD values for information to the “High power UE for FR1 for FDD single band(s) with power class 2”.

Proposal: Since PC2 RSD is on top of PC3 REFSENS that must be agreed upon in the “Adding new channel BWs support to existing NR bands” WI, we propose to agree PC3 REFSENS and PC2 RSD in the “Adding new channel BWs support to existing NR bands” WI to avoid the “chicken and egg” issue. The agreements can be captured in the respective BigCRs.

## PC3 REFSENS

To analyse the REFSENS issues for the new UL channel bandwidths, Figure 1 describes the IMD landscape for the wanted allocated RBs and their image for the different UL CBW and their overlap to the related DL CBWs. Table 1 provides more accurate calculations for the worst case IMD.

Figure 1: IMD landscape for the different UL configurations

Table 1: IMD interference for various UL/DL CBW

| DL CBW | UL CBW | IMD order | overlap | comment | UL CBW | IMD order | overlap | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [MHz] | [MHz] |  | [MHz] |  | [MHz] |  | [MHz] |  |
| 25 | 20 | IMD5 | 13.7 | partial IMD5 spectrum with peak in DL BW | 25 | IMD3 | 1.895 | small IMD3+IMD5 |
| 30 | 20 | IMD5 | 16.13 |  | 30 | IMD3 | 10.8 | Full IMD3 spectrum in DL BW |
| 35 | 20 | IMD5 | 18 | just missed IMD3+full IMD5 | 35 | IMD3 | 10.8 |  |

Observation for the baseline 20MHz max UL CBW:

- 20MHz DL CBW is overlapped by partial IMD5 and partial IMD7 of 20MHz UL CBW

- 25MHz DL CBW is overlapped by full IMD5 and partial IMD7 of 20MHz UL CBW

- 30MHz DL CBW is overlapped by partial IMD3, full IMD5 and partial IMD7 of 20MHz UL CBW

- 35MHz DL CBW is overlapped by partial IMD3, full IMD5 and partial IMD7 of 20MHz UL CBW

Observation for the optional symmetrical UL/DL CBW:

- 25MHz DL CBW is overlapped by partial IMD3 and almost full IMD5 of 25MHz UL CBW

- 30MHz DL CBW is overlapped by full IMD3 and partial IMD5 of 25MHz UL CBW

- 35MHz DL CBW is overlapped by full IMD3 of 35MHz UL CBW

- It should be noted that with IMD3 being significantly stronger than IMd5, the 30MHz and 35MHz CBW cases experience a very similar interference from their respective UL.

### PC3 25 and 30MHz symmetrical UL/DL CBW REFSENS

In RAN4#106b-e, the contributions [4, 5] provided input for PC3 REFSENS for the 25 and 30MHz as follows:

[4]: -76.6 dBm and -67.3 dBm can be used for 25MHz and 30MHz CBW respectively with 20RB0 UL allocation.

[5]: -75.9 dBm and -69.9 dBm for 25 MHz and 30 MHz channels, respectively.

As discussed earlier and as shown in Figure 1, the 25MHz and 30MHz cases have different overlap of IMD3 and IMD5 which can explain the difference between values in [4, 5]. The higher REFSENS degradation from 25MHz to 30MHz in [4] compared to [5], can be explained by a stronger IMD3 content. This is because the 30MHz CBW encompasses the peak of IMD3, while 25MHz does not, and there is a different ratio between IMD3 and IMD5. These differences are reasonable for various implementations, and thus averaging the values can provide ample coverage for different UEs.

Observation: Since the values are very close it is fair to investigate averaging the data as was suggested in [2]. Using averaged linear power, this gives -76.2 dBm and -68.4 dBm for 15kHz SCS 25MHz and 30MHz CBW, respectively. The same delta versus the 20MHz maximum UL CBW can be applied for the 30kHz REFSENS, resulting in -76.3 dBm and -68.3 dBm for 30kHz SCS 25MHz and 30MHz CBW, respectively.

### PC3 35MHz symmetrical UL/DL CBW REFSENS

As discussed earlier and as shown in Figure 1, the UL interference levels in 30MHz and 35MHz DL CBW are very similar, allowing to derive an estimate for 35MHz REFSENS. Based on measurements in [4], and adding correction to fit the averaged RFESENS for the 30MHz case, the calculated de-sense is 20dB. This means that the receiver noise is dominated by the interfering power, and thus the 35MHz REFSENS is the same than for 30MHz at -68.4 dBm.

Observation: Since the UL interference is the same for 30MHz and 35MHz UL/DL and the REFSENS is dominated by the de-sense (>20dB), the 30MHz and 35MHz 15kHz SCS REFSENS are the same at -68.4 dBm and -68.5 dBm for 30kHz SCS.

### Capturing the baseline asymmetrical and optional symmetrical REFSENS

For Release 18, the UE shall support the current 20MHz maximum UL CBW case as a baseline as agreed upon in [2]. However, UEs are also allowed to support all/any of the new symmetrical UL/DL CBWs and signal it accordingly with the channelBWs-UL IE. The most direct method to capture the new optional symmetrical UL/DL CBW requirement is to add a second line with a proper note.

Proposal on PC3 REFSENS and UL configuration:

- REFSENS and UL configuration for the symmetrical UL/DL 25, 30 and 35MHz CBW are specified in Table 2 and Table 3 below (yellow highlight). Other requirements are already in 38.101-1.

- Notes are added to clarify applicability to UEs supporting the optional symmetrical UL/DL bandwidths by signaling the associated channelBWs-UL IE.

- 35MHz REFSENS shall not be captured in 38.101-1, until this new CBW is formally approved in RAN.

Table 2: Two antenna port reference sensitivity QPSK PREFSENS for n71

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 | -84.1 -76.29 | -82.5 -68.49 | -80.7 [-68.49] |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.4 | -84.2 -76.39 | -82.6 -68.59 | -80.8 [-68.59] |  |  |  |
| NOTE 9: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |  |

Table 3: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | Duplex Mode |
| n71 | 15 | 25 | 251 | 201 | 201 | Note 5 201,6 | Note 5 201,6 | Note 5 [201,6] | FDD |
|  | 30 |  | 121 | 101 | 101 | Note 5 101,6 | Note 5 101,6 | Note 5 [101,6] |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. |  |  |  |  |  |  |  |  |  |

## PC2 25, 30 and 35MHz symmetrical UL/DL CBW RSD

Based on the averaged PC3 REFSENS from [4,5], the 1Tx and 2Tx RSDs can be re-calculated from the measurements shown in [4]. Similarly, the 35MHz case can be estimated based on the observation that the interference is the same as that for the 30MHz case. The evaluated values are provided in the table below.

Table 4: RSD values for 1Tx and 2Tx PC2 symmetrical UL/DL CBW

| TX configuration | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) |
| --- | --- | --- | --- |
| 1Tx | 2.8 | 3.0 | [3.0] |
| 2Tx | 5.9 | 6.1 | [6.1] |

Similar to what is proposed for REFSENS, we propose to capture the RSD requirements for the optional symmetrical UL/DL CBW in 38.101-1 related tables together with the baseline requirement (still pending agreement in the PC2 FDD WI) with the appropriate notes. The proposed tables use the current averaged values for the baseline requirement in brackets. In some cases, the RSD for the symmetrical UL/DL CBW is lower than for the baseline case. This is explained by the averaging of different values, but more accurately by the fact that the symmetrical UL/DL CBW REFSENS is much worse than that for the baseline REFSENS.

Proposal on PC2 1Tx and 2Tx RSDs:

- 1Tx and 2Tx RSDs for the symmetrical UL/DL 25, 30 and 35MHz CBW are specified as per Table 5 and Table 6 below (yellow highlight). Other requirements are in brackets and these correspond to the average of the proposals. These need to be formally agreed upon in the related PC2 FDD WI (blue highlight).

- Notes are added to clarify applicability to UEs supporting the optional symmetrical UL/DL bandwidths by signaling the associated channelBWs-UL IE.

- 35MHz RSDs shall not be captured in 38.101-1 until this new CBW is formally approved in RAN.

- Note that the RSD is on top of REFSENS. Thus, RSD may need to be recalculated if the agreed REFSENS is different from the one proposed in this contribution.

Table 5: Reference Sensitivity Degradation from PC3 to PC2 for n71 for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | [0.5] | [0.9] | [0.9] | [2.2] | [2.4] 2.82 | [2.5] 3.02 | [2.9] [3.02] | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |

Table 6: Reference Sensitivity Degradation from PC3 to PC2 for n71 for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | [1.1 | [1.1 | [1.7] | [5.5] | [5.9] 5.92 | [6.2] 6.12 | [6.5] [6.12] | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, we evaluated the 25MHz and 30MHz symmetrical UL/DL CBW REFSENS and RSD for Band n71, and estimated the same for 35MHz. This resulted in the following proposals.

Proposal: Since PC2 RSD is on top of PC3 REFSENS that must be agreed upon in the “Adding new channel BWs support to existing NR bands” WI, we propose to agree PC3 REFSENS and PC2 RSD in the “Adding new channel BWs support to existing NR bands” WI to avoid the “chicken and egg” issue. The agreements can be captured in the respective BigCRs.

Proposal on PC3 REFSENS and UL configuration:

- REFSENS and UL configuration for the symmetrical UL/DL 25, 30 and 35MHz CBW are specified in Table 2 and Table 3 below (yellow highlight). Other requirements are already in 38.101-1.

- Notes are added to clarify applicability to UEs supporting the optional symmetrical UL/DL bandwidths by signaling the associated channelBWs-UL IE.

- 35MHz REFSENS shall not be captured in 38.101-1, until this new CBW is formally approved in RAN.

Table 2: Two antenna port reference sensitivity QPSK PREFSENS for n71

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 | -84.1 -76.29 | -82.5 -68.49 | -80.7 [-68.49] |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.4 | -84.2 -76.39 | -82.6 -68.59 | -80.8 [-68.59] |  |  |  |
| NOTE 9: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |  |

Table 3: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | Duplex Mode |
| n71 | 15 | 25 | 251 | 201 | 201 | Note 5 201,6 | Note 5 201,6 | Note 5 [201,6] | FDD |
|  | 30 |  | 121 | 101 | 101 | Note 5 101,6 | Note 5 101,6 | Note 5 [101,6] |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. |  |  |  |  |  |  |  |  |  |

Proposal on PC2 1Tx and 2Tx RSDs:

- 1Tx and 2Tx RSDs for the symmetrical UL/DL 25, 30 and 35MHz CBW are specified as per Table 5 and Table 6 below (yellow highlight). Other requirements are in brackets and these correspond to the average of the proposals. These need to be formally agreed upon in the related PC2 FDD WI (blue highlight).

- Notes are added to clarify applicability to UEs supporting the optional symmetrical UL/DL bandwidths by signaling the associated channelBWs-UL IE.

- 35MHz RSDs shall not be captured in 38.101-1 until this new CBW is formally approved in RAN.

- Note that the RSD is on top of REFSENS. Thus, RSD may need to be recalculated if the agreed REFSENS is different from the one proposed in this contribution.

Table 5: Reference Sensitivity Degradation from PC3 to PC2 for n71 for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | [0.5] | [0.9] | [0.9] | [2.2] | [2.4] 2.82 | [2.5] 3.02 | [2.9] [3.02] | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |

Table 6: Reference Sensitivity Degradation from PC3 to PC2 for n71 for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | [1.1 | [1.1 | [1.7] | [5.5] | [5.9] 5.92 | [6.2] 6.12 | [6.5] [6.12] | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: UEs supporting the optional symmetrical UL/DL bandwidths shall meet this requirement |  |  |  |  |  |  |  |  |  |  |

# References

- RP-230756 Revised Basket WID: Core part: Adding new channel BWs support to existing NR bands, Ericsson, RAN#99

- R4-2306545 WF on adding new channel bandwidths support to existing NR bands, Ericsson, RAN4#106b-e

- R4-2306544 WF on FDD HPUE Basket WIs, China Unicom, RAN4#106b-e

- R4-2304559 Impact of 25 and 30MHz ULDL CBW for n71, Skyworks Solutions Inc., RAN4#106b-e

- R4-2304349 On RF requirements for 25MHz and 30MHz UL CBW for n7, Apple, RAN4#106b-e