3GPP TSG-RAN WG4 Meeting # 107										R4-2307067

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.28.1

Source: 	Fujitsu

Title: 	Discussion on the enhancement for SAR issue mitigation

Document for:	Approval

# 1. Introduction

In RAN4#106bis-e meeting, the discussion for the enhancement for SAR issue mitigation for FR1 was conducted but the progress was limited. In this contribution, we share our thought on this issue from the viewpoint of gNB vendor.

# 2. Discussion

Firstly, the issue we are discussing several meetings is clarified. For avoiding the violation of SAR limitation, current spec assumes the following two solutions by reducing the UE transmission power.

- P-MPR

- Changing the requirements of power class to default power class (called “PC fallback”)

For P-MPR, the UE calculates the RF exposure and then reduce the transmission power if it exceeds the SAR limitation. It is up to UE how much the transmission power is reduced because it is closely related to the UE implementation. For PC fallback, the UE shall apply all requirements for the default power class if the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle. One important thing is that there is no function/signaling to report from UE to gNB about P-MPR value or PC fallback occurrence. It means that the UE can reduce its transmission power but do not need to report it to gNB according to the current specs.

Observation 1: It is the fact that the UE can reduce its transmission power considering the SAR limit but do not need to report it to gNB.

Next, the impact on not reporting the transmission power reduction to gNB is analyzed. For P-MPR, it is difficult to analyze the range of P-MPR value because it is fully up to the UE implementation. On the other hand, for PC fallback, it can be easily analyzed. It is up to 6dB considering the change of maximum transmission power from PC1.5 to default power class. It means that gNB schedulers are imposed to conduct the resource allocation by using the wrong information of UE transmission power with 6dB higher than actual if the PC fallback occurs. It impacts on not only the network performance but also the stability of gNB scheduler, such as the increase of HARQ retransmission. Especially the occasion of using P-MPR or PC fallback scheme may increase because of the introduction of the function of increasing UE power high limit for CA and DC. It should be avoided to reduce the benefit of introducing it due to the lack of information related to the transmission power reduction at gNB side.

Observation 2: The transmission power reduction without reporting it to gNB impacts on the network performance and the stability of gNB scheduler because gNB schedulers are imposed to conduct the resource allocation by using the wrong information related to UE transmission power.

In the last meeting, there are some comments on PC fallback scheme, such as it is practically not used in the field or almost all the UEs uses P-MPR scheme so far. It might be true, but our concern is that the PC fallback scheme has already been specified. It means that there is a possibility that UE implements the duty cycle function in the future. As explained in Observation 2, we have concerned about the impacts on the network performance and the stability of gNB scheduler if its function will be introduced. Fortunately, the possible solutions we have discussed in several meetings can solve the issue. It is straightforward to study its enhancement if they can address the issue.

Observation 3: There is a possibility that UE implements the duty cycle function in the future because it has already been specified. It is straightforward to study its enhancement if the possible solutions can address the issue.

Observation

RAN1 discussed advantages and disadvantages of solutions included in R1-2302270 (R4-2303701) on enhancements to realize increasing UE power high limit for CA and DC. Pros and cons of the inclusion in the PHR report of at least one of the following quantities have been analyzed for different reporting mechanisms, triggers, and reporting periodicities:

- ∆PPowerClass

- Power class

- P-MPR

- Start and length of evaluation period for power class fallback

- Estimated duration of power class fallback

- Estimated duration over which UE can sustain Pcmax before additional P-MPR is required

- Sustainable duty cycle to prevent a fallback

- Energy/power availability

Note: Discussion is still ongoing, and its full current content can be found in Section 2.1.2 of R1-2303924.

From the above observations, RAN4 should continue the discussion on the possible solutions. One important thing is that RAN1 has to complete the discussion at RAN1#114 meeting, held at Aug. 2023 according to the allocated TU information. According to the RAN1 observation in the last meeting as follows, the progress of RAN1 discussion is limited and it will not move forward without the input from RAN4 [1].

Considering the current situation, we think that RAN4 should not waste the RAN1 discussion time by not sending the reply LS to RAN1. Without it at May meeting, it is difficult to have the time for discussing it in RAN1. If RAN4 can conclude that the impact on gNB scheduling with wrong information is negligible, the reason should be clarified. And it also should be informed to RAN1 because that information is helpful for RAN1 discussion.

Proposal 1: RAN4 should not waste the RAN1 discussion time by not sending the reply LS. If RAN4 can conclude that the impact on gNB scheduling with wrong information is negligible, the reason should be clarified and informed to RAN1.

# 3. Conclusions

In this contribution, we share our observations and proposals for the enhancement for SAR issue mitigation as follows.

Observation 1: It is the fact that the UE can reduce its transmission power considering the SAR limit but do not need to report it to gNB.

Observation 2: The transmission power reduction without reporting it to gNB impacts on the network performance and the stability of gNB scheduler because gNB schedulers are imposed to conduct the resource allocation by using the wrong information related to UE transmission power.

Observation 3: There is a possibility that UE implements the duty cycle function in the future because it has already been specified. It is straightforward to study its enhancement if the possible solutions can address the issue.

Proposal 1: RAN4 should not waste the RAN1 discussion time by not sending the reply LS. If RAN4 can conclude that the impact on gNB scheduling with wrong information is negligible, the reason should be clarified and informed to RAN1.

# 4. Reference

- Draft Report of 3GPP TSG RAN WG1 #112bis-e v0.1.0