3GPP TSG RAN WG4 #107 meeting 	 	R4-2307121

Incheon, Republic of Korea, 22nd – 26th May, 2023

Agenda Item:	8.5.1.3.1

Source:	Meta Ireland

Title:	Discussion on the remaining open issues for lower MSD capability signalling

Document for:	Approval

- Introduction

In RAN4 #106 meeting, RAN4 concluded that lower MSD is feasible [1] and RAN approved the updated objectives in the WI [2] for FR1 RF further enhancements. Especially, RAN2 WG was involved to specify the lower MSD capability signalling. In RAN4 #106BIS-e meeting, the remaining open issues will be continually discussed and addressed in 2nd phase discussion in the WI based on the approved the WF [3] as follow:

Sub-Topic 1-1: Conditions to indicate the lower MSD capability - Option 1: For the purpose of MSD improvement, if the minimum requirement for a given REFSENS exception case falls into the interval of MSD ≤ Thi dB, the actual MSD should be at least one-level lower (i.e., actual MSD ≤ Thi-1 dB) in order for the UE to report the low-MSD capability. If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case (Samsung, Xiaomi, Nokia, AT&T, Skyworks, HW) - If UE reports the lower MSD capability, the reported MSD value should be improved at least by TBD dB against a specified MSD - Option 2: For the purpose of MSD improvement, if the minimum requirement for a given REFSENS exception case falls into the interval of MSD ≤ Thi dB, the actual MSD should be at least one-level lower (i.e., actual MSD ≤ Thi-1 dB) in order for the UE to report the low-MSD capability. If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case (QC, OPPO, vivo) - If UE reports the lower MSD capability, the reported MSD value should be improved at least by TBD dB against a specified MSD - Option 3: Others (Meta, MediaTek, Apple) - WF Further discuss the listed options. Issue 1-1-2: Whether CBW of aggressor UL and victim DL should be reported for lower MSD capability - Option 1: Yes (CHTTL, ZTE) - Option 2: No (Samsung, Xiaomi, HW, vivo, Meta, Skyworks, QC, Apple, OPPO, AT&T) - Option 3: Discuss together with conformance test (Samsung, Nokia) - WF FFS in next meeting. Sub-Topic 1-4: Candidate MSD thresholds - Option 1: 0~15dB (Nokia, AT&T) - Option 2: 0~20dB (HW, Samsung, Skyworks, Meta, vivo, OPPO, Xiaomi) - Option 3: 0~[25/27]dB (LGE, QC) - Option 4: improved MSD in granularity of 1dB (MTK, ZTE) - WF Further discuss the listed options in next meeting. Sub-Topic 1-6: Conformance test for lower MSD - Option 1: For cross band isolation, in terms of lower MSD capability (Samsung, QC, Meta) - If UE supports the specified worst-case configuration which corresponds to the largest MSD, it should be chosen to verify lower MSD capability - If UE does not support the specified worst-case configuration. FFS on the test configuration for lower MSD - Alt 1: The worst-case configuration the UE supports itself is chosen as test configuration (→ But may lead to new test point against the exiting specified test point for conformance test) - Alt 2: Others - Option 1a: For cross band isolation, in terms of lower MSD capability (Samsung) - UE supports the specified worst-case configuration which corresponds to the largest MSD, this configuration is selected as test configuration for verifying both existing specified MSD and lower MSD capability → No additional test point needed for lower MSD compared with existing specified MSD - UE does not support the specified worst-case configuration, but support the second test configuration (if introduced) which is an optionally defined one to address operator’s demand, the second configuration is selected as test configuration for verifying both existing specified MSD and lower MSD capability → No additional test point needed for lower MSD compared with existing specified MSD - UE does not support any of the specified configurations, the worst-case configuration the UE supported itself for this band combination should be chosen as test configuration for verifying both existing specified MSD and lower MSD capability → No additional test point needed for lower MSD compared with existing specified MSD. - Option 2: (Skyworks, Meta, [HW], Apple, Xiaomi, AT&T, QC) - A UE signalling the optional lower MSD capability should not have more or different conformance test points than a legacy UE without lower MSD capability, only the test limits should be impacted - When a UE signals a lower MSD threshold for a given MSD type, the currently applicable inter-band worst case MSD tests are performed, and the limit is modified to the signalled threshold value instead of the MSD value in 3.101-1 or 38.101-3. - If other MSD test points exits for the same MSD type (for example H3 on top of worst case H2 or IMD3 on top of worst case IMD2…), the test is also performed with the limit modified to the signalled threshold value instead of the MSD value in 3.101-1 or 38.101-3. - If the UE fails the test, the conformance test will have to be passed again either: - With the UE now passing the test with the same threshold after HW/SW modifications - With the UE passing the test with the higher threshold signalled - With the UE passing the normal test without the lower MSD capability - It should be noted that there are worst case MSD test points cases in 38.101-1 or 38.101-3 that some UE cannot pass as: - They do not support the lowest channel UL CBW (very rare) for all 1UL and 2UL IMD test - They do not support the largest channel UL CBW for the 1UL cross-band MSD case: - In many cases, there is a second cross-band MSD test point that uses a lower UL CBW that a majority of UEs would support; in this case, this test point is used with the signalled threshold value as the limit - In the rare case where a UE would not support the UL CBW of all the cross-band MSD test points, the UE is tested with the largest CBW it supports and uses the signalled threshold value as the limit - The worst-case MSD test point is not valid for the support frequency range in a given region - For this case, there is usually a second MSD test point that can be measured in any applicable region; in this case, this test point is used with the signalled threshold value as the limit. - Option 3: No new test configurations (points) be set for lower MSD compared to current MSD requirements (vivo, OPPO) - Option 4: Others - WF FFS in next meeting. Sub-topic 1-7: Other approaches for lower MSD capability reporting - Option 1: Report the ratio of MSD reduction to Tx power reduction (HW, Meta) - Option 2: Report MSD = 0 dB region report (Nokia) - Option 3: single-bit low-MSD indicator for a UE that all MSD types for this BC have been improved to above a threshold (HW, vivo, Samsung, Meta, [OPPO]) - Option 3a: A joint solution of one bit low MSD indication per BC with the per victim band per MSD type per band combination signaling, one bit low MSD indication can be used if all MSD types for this BC have been improved to above a threshold. (CHTTL) - Option 3b: Single bit indication of small MSD capability (MSD <= [3]dB) can be reported to apply the improved MSD level by the high order IMD/harmonic problems (Meta) - Option 4: Others (Skyworks, Ericsson, QC, Apple, ZTE, Xiaomi, AT&T, MTK) - WF FFS in next meeting.

In this paper, we propose how to solve the open issues and propose the detail reported lower MSD threshold range, MSD granularity based on the general lower MSD capability and the small MSD capability.

- Signalling for lower MSD capability

  - Conditions to indicate the lower MSD capability

In the last RAN4 meeting, some vendors requested to define the additional level compared to MSD threshold for actual improved MSD value as follow “the actual MSD should be at least one-level lower (i.e., actual MSD ≤ Thi-1 dB) in order for the UE to report the low-MSD capability.”

However, the actual MSD value is the relative MSD value difference from the existing MSD requirements for the specific CA/DC band combinations in TS 38.101-1 and TS38.101-3.

From our perspective, it is quite difficult to define the absolute level with 1dB from the MSD threshold to report the lower MSD capability signalling. If UE reports 3dB granularity for lower MSD capability in [4], the index ‘2’ means that the actual MSD belongs to 3 < Actual MSD ≤ 6. So, we think that the actual MSD improvement can be in the from 3dB to 6dB range. But, if RAN4 accept the proponent proposal in [4], the proposal can be interpreted as follow

“The actual MSD value shall be located from 3dB to 5dB only for the UE reported with index 2. The 5.5dB MSD improvement do not allowed to report the MSD capability with index 2”

But this is not true, the actual MSD improvement can belong to 3 < Actual MSD ≤ 6 dB when the UE reported with index 2 for lower MSD capability. The 5.5dB MSD improvement also can be passed the MSD conformance test.

Therefore, we propose that RAN4 only allow the condition to indicate the lower MSD capability to define the minimal condition for MSD capability reporting as follow

Proposal #1: UE could indicate Lower MSD capability for a band combination as long as one kind of MSD from one victim band is improved. Additionally, it is unnecessary to report the Lower MSD values in case the specified MSD itself is small or the MSD improvement is not significant. The small MSD improvement will be discussed in different sub-topic based on the specific band combinations.

  - Reporting information for lower MSD Capability

    - Channel bandwidth information of the aggressor UL and the victim DL

For the information of the aggressor UL CBW and the victim DL CBW, RAN4 would not support all the supported channel bandwidths for the lower MSD capability of all the CA/DC band combinations and the corresponding CBW combination sets. So, the specific UL/DL CBW information is not necessary since the network controller would not expect any different behaviour according to the CBW information.

If the individual NW behaviour is assumed based on the specific UL/DL CBW information, then RAN4 can report information of the aggressor UL CBW and the victim DL CBW. But it should be clearly understood and requested with reasonable evidence.

Proposal #2: The aggressor UL and victim DL CBW information is not necessary since the network controller will not expect any different behaviour according to the CBW information. If RAN4 makes consensus on the necessity of the UL/DL CBW information based on the reasonable evidence and explicit NW behaviour, then RAN4 can report the CBW information of the aggressor UL and victim DL CBW.

  - Candidate MSD thresholds for all MSD types

In MSD evaluation results in TR38.881 [4], the expected MSD improvements in the evaluation results from interested companies [4] showed that the maximum 20 dB MSD enhancements were feasible for the all the MSD types compared to existing MSD requirements in TS38.101-1 and TS38.101-3. Based on these evaluation results, we propose to report the lower MSD capability as a compromise.

Table 1: MSD reporting bitmap for lower MSD capability

| Bit map | MSD range (i.e. Thresholds) | Note |
| --- | --- | --- |
| 000 | - | Not supported the lower MSD capability. Only apply the existing MSD requirements in TS38.101-1 and TS38.101-3. |
| 001 | 3 dB | 0 ≤ Actual MSD ≤ 3 |
| 010 | 6 dB | 3 < Actual MSD ≤ 6 |
| 011 | 9 dB | 6 < Actual MSD ≤ 9 |
| 100 | 12 dB | 9 < Actual MSD ≤ 12 |
| 101 | 15 dB | 12 < Actual MSD ≤ 15 |
| 110 | 18 dB | 15 < Actual MSD ≤ 18 |
| 111 | 21 dB | 18 < Actual MSD ≤ 21 |

Proposal #3: RAN4 introduces multiple MSD thresholds using 3-bit MSD reporting bitmap in Table 1 to apply lower capability for all CA/DC band combinations according to the different MSD types and different order.

  - Conformance test for lower MSD

Generally, we are fine with the option 2 for the conformance test aspect for lower MSD capability signalling.

Especially, we prefer to keep the current CA/DC UE conformance test’s burden for the lower MSD capability UE on the related conformance perspectives such as test time and the number of test cases. Also, prefer to keep the current test configuration in the existing MSD requirements and do not define the additional minimum requirements in TS38.101-1 and TS38.101-3 for lower MSD capability UE. RAN4 can add the note in the MSD requirements which was already proposed in [5] as follows.

NOTE X: If the UE supports the lower MSD capability, then a reported MSD threshold shall be tested and verified as the REFSENS exception requirements with the appropriate test point in each reference sensitivity exceptions in section 7.3A.4, 7.3A.5 and 7.3A.6.

If the lower MSD capable UE did not pass the MSD conformance test, then the UE can re-test with the relaxed MSD threshold or same MSD threshold with the updated UE development. Also, the UE can decide not to report the lower MSD capability after the conformance test results.

The specific test configuration and requirements can be treated as case by case of the specific CA/DC band combinations if the existing MSD requirements are not possible to apply the MSD test configuration for the CA/DC band combinations.

Proposal #4: For the conformance test aspect, meta supports the option 2 in the sub-topic 1-6 [3]. In Particular, we prefer not to define the explicit MSD requirements for lower MSD capability in TS38.101-1 and TS38.101-3.

  - Other approaches for lower MSD capability reporting

For the lower MSD capability, we think that the additional small MSD capability reporting is needed and separately RAN4 can treat the small MSD capability. In our perspective, the small MSD capability (MSD <= [3] dB) reporting is also useful to apply the actual improved MSD level by high order IMD/harmonic problems (e.g., H5, IMD5) and cross band isolation issue. Even though the improved MSD level is quite small, the MSD enhancement is also important to keep the legacy UE performance in single carrier to overcome the self-interference problems.

Observation #1: A bit indication of small MSD capability (MSD <= [3] dB) can be useful to apply the improved MSD level by the high order IMD/harmonic problems (e.g. 5th IMD, 5th Harmonic) in the inter-band CA/DC band combinations.

Based on the observation, RAN4 can discuss separately between general lower MSD capability for all type MSD sources & orders and small MSD capability. The latter MSD capability is related to the specific CA/DC band combinations to overcome the high order IMD/harmonic interference problems and crossband isolations.

Based on our analysis, we propose the 2-bits MSD reporting as follows.

Table 2: MSD reporting bitmap for lower MSD capability when a bit indication of small MSD capability is enabled (MSD <= [3]dB)

| Bit map | MSD range (i.e. Thresholds) | Note |
| --- | --- | --- |
| 00 | - | Not supported the lower MSD capability. Only apply the existing MSD requirements in TS38.101-1 and TS38.101-3. |
| 01 | [1] dB | 0 ≤ Actual MSD ≤ [1] |
| 10 | [2] dB | 1 < Actual MSD ≤ [2] |
| 11 | [3] dB | 2 < Actual MSD ≤ [3] |

Proposal #5: Single bit indication and 2-bits MSD reporting bitmap for the small MSD capability (MSD <= [3] dB) can be considered to apply the improved MSD level by the high order IMD/harmonic problems.

- Conclusions

In this contribution, we provided our view on how to report the MSD levels, how to apply the MSD thresholds for the different MSD source, different order within the same MSD source and different power classes. Also, we prefer to consider the general MSD capability and small-MSD capability separately.  If RAN4 considers the indication of small MSD capability (MSD <= [3] dB), the reported MSD thresholds in Table 2 are considered to apply actual improved MSD levels for the CA/DC band combination. Furthermore, in the conformance test aspect, we prefer to keep the current test burden of the inter-band CA/DC UE for the lower MSD capability UE. Based on our analysis for the remaining open issues, we proposed as follow

Proposal #1: UE could indicate Lower MSD capability for a band combination as long as one kind of MSD from one victim band is improved. Additionally, it is unnecessary to report the Lower MSD values in case the specified MSD itself is small or the MSD improvement is not significant. The small MSD improvement will be discussed in different sub-topic based on the specific band combinations.

Proposal #2: The aggressor UL and victim DL CBW information is not necessary since the network controller will not expect any different behaviour according to the CBW information. If RAN4 makes consensus on the necessity of the UL/DL CBW information based on the reasonable evidence and explicit NW behaviour, then RAN4 can report the CBW information of the aggressor UL and victim DL CBW.

Proposal #3: RAN4 introduces multiple MSD thresholds using 3-bit MSD reporting bitmap in Table 1 to apply lower capability for all CA/DC band combinations according to the different MSD types and different order.

Proposal #4: For the conformance test aspect, meta supports the option 2 in the sub-topic 1-6 [3]. In Particular, we prefer not to define the explicit MSD requirements for lower MSD capability in TS38.101-1 and TS38.101-3.

Proposal #5: Single bit indication and 2-bits MSD reporting bitmap for the small MSD capability (MSD <= [3] dB) can be considered to apply the improved MSD level by the high order IMD/harmonic problems.

Reference

[1] R4-2303695, “WF on study for lower MSD,” Huawei

[2] RP-230753, “Revised WID on Further RF requirements enhancement for NR and EN-DC in FR1,” Huawei

[3] R4-2306590, “WF on lower MSD,” Huawei

[4] TR38.881: Study on lower MSD for inter-band CA/EN-DC/DC combinations

[5] R4-2300206, “Lower MSD capability signalling,” Meta

[6] R4-2218298, “On lower MSD capability signaling,” Meta Ireland

[7] TS38.101-1

[8] TS38.101-3