3GPP TSG-RAN WG4 Meeting #107	R4-2307142

Incheon, Korea, May 22 – May 26, 2023

Agenda Item:	8.22.3

Source:	CAICT

Title:	Discussion on Interoperability and testability aspects for AI/ML

Document for:	Discussion

1	Introduction

In last RAN4 meeting, interoperability and testability aspects for AI/ML was discussed and following agreements had been made [1].

Agreement: Reference block diagrams for 1-sided model and 2-sided model are to be further studied, - Logical block diagrams in R4-2305051 can be used as reference - AI/ML model control in TE may not be applicable in specific use cases - Diagrams are to be taken as logical blocks - Further study, whether test dataset should be defined for each test - DUT can be either UE or gNB - “TE” may mean test equipment as used in conformance testing today, but if RAN4 requirements are used as part of model monitoring it may be more generic to refer to the testing methodology. Agreement: - RAN4 to study the following issues for the 2-sided model test framework - Common assumptions for proposals of the reference decoder / encoder (and the paired encoder/ decoder) for tester - Definition and derivation procedure of intermediate KPI for decoder evaluation and selection - Data collection/generation for decoder evaluation, and the common assumptions/environment needed for data collection/generation - How to minimize the impact of possible variations/differences in the reference decoder/ reference encoder design/implementation on UE/ gNB performance verification - The impact of reference decoder/ encoder for testing complexity to UE/gNB performance verification, and the advantage/disadvantage analysis of high/low complexity decoders. - Other aspects are not precluded, companies are invited to bring contribution detailing any other aspects that should be considered - FFS whether any reference for the encoder/ decoder needs to be considered given that the encoder/decoder performance is to be tested - Take into account RAN1 discussions and conclusions on interoperability and training for 2-sided - Reference Decoder for test implementation for 2-sided models in the UE performance tests - Following options should be studied for the reference decoder for test implementation in the UE performance tests - Option 1: reference decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained - Option 2: reference decoder is provided by the vendor of the decoder so that the encoder and decoder are jointly designed and trained - Option 3: The reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The reference decoder(s) are partially specified and captured in RAN4 spec. - Option 5: Option 1, 2, 3 or 4 depending on the test - Option 6: Test decoder is specified and captured in RAN4 and is provided by test environment vendor. The encoder and decoder can be jointly trained. - Other options can be discussed depending on companies’ inputs - Reference decoder defined for the tester in the UE performance tests should not limit the implementation of different models at the network side - Reference Encoder for test implementation for 2-sided models in the gNB performance tests - Following options should be studied for the reference encoder for test implementation in the gNB performance tests - Option 1: reference encoder is provided by the vendor of the decoder under test so that the encoder and decoder are jointly designed and trained - Option 2: reference encoder is provided by the vendor of the encoder so that the encoder and decoder are jointly designed and trained - Option 3: The reference encoders are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The reference encoders are partially specified and captured in RAN4 spec. - Option 5: Option 1, 2, 3 or 4 depending on the test - Option 6: Test encoder is specified and captured in RAN4 and is provided by test environment vendor. The encoder and decoder can be jointly trained. - Other options can be discussed depending on companies’ inputs - Reference decoder defined for the tester in the gNB performance tests should not limit the implementation of different models at the UE side.Further discuss the difference between reference encoder/decoder and test encoder/decoder. Agreement: - RAN4 to investigate how to define performance requirements/tests for the following candidate procedures: - model/functionality monitoring - model/functionality selection - model/functionality activation/deactivation/switching/fallback - FFS whether data collection should be considered - FFS whether model update/transfer/delivery should be considered

In this contribution, we share our views on reference block diagram and reference encoder/decoder.

2	Discussion

## 2.1 Reference block diagram

The reference block diagrams for one-sided and two-sided AI/ML model given in R4-2305051 are presented in Figure 1.

(a)

(b)

Figure 1 Reference block diagram for (a) one-sided and (b) two-sided AI/ML model

AI/ML model control: The content of model control part needs clarification since it was not defined in current discussion. In our thinking, this part shall include model select/switch/fallback/activation/deactivation and model update, which relates to the LCM procedure. Model delivery/transfer could also be added into the reference block diagram as a separate block, considering the support of collaboration level z and the possibility of implementing one-sided model with model training on one side and model inference on a different side.

Test dataset/data generator: Test dataset is used to verify the inference performance of the model under particular conditions (e.g., scenarios/configurations/sites). Different dataset may be used for various conditions. Therefore, constructing test dataset is an important functionality in the test to ensure that the inference performance of an AI/ML model can be properly verified.

Using the same generation method and/or configuration as training dataset to construct test dataset could verify the achievable performance of AI/ML, while using a different method and/or configuration could verify the generalization performance of AI/ML.

Performance verification: In current diagram, the performance verification is conducted on TE side. We think this part is also possible to be conducted on DUT side. This may depend on the specific use case. Intermediate KPI and/or performance KPI could be considered for performance verification.

Proposal 1: Further clarifications for different elements of reference block diagram should be included.

## 2.2 Reference encoder/decoder

In last meeting, several options for reference encoder/decoder for test implementation had been given.

Option 1/2

The intention of option 1 and 2 is to have a jointly designed and trained two-sided model which is possible to achieve the optimal performance of AI/ML. The difference between two options lies on from which side the reference model is provided, DUT (option 1) or tester (option 2). In our thinking, option 1 does not require tester to develop its own model, but it requires tester to be able to run multiple model types considering various implementations of different DUTs. This may face the issues like, less efficiency or even incompatibility when implementing the model given from DUT, which may deteriorate the performance of AI/ML. Option 2 can relieve the issue by involving tester into model design and training. Some offline efforts may be needed among different vendors. Propriety of models is another major concern.

Option 3/4

Option 3 and 4 is development-friendly since explicit information of reference model is captured. This can avoid discrepancy among implementations and provide consistent performance towards different DUTs. The disadvantage is less future-proof considering the rapid progress of AI technology. Moreover, reaching a consensus on common reference model would also need more discussions and evaluations.

Option 6

Option 6 can be viewed as a mixture of option 1/2 and 3/4. In our thinking, specifying some information of reference model could ease the development for TE vendor while leave some free space for implementation. This could also relieve the incompatibility issue since DUT can develop corresponding encoder/decoder based on the same structure or type with TE vendor. A possible concern is implementation discrepancy between different TE vendors and between TE vendor and NW/chip/terminal vendor, which would lead to performance inconsistency. Moreover, option 6 may also be less future-proof as option 3/4.

From our perspective, option 3/4 can be taken as starting for reference model design. Evaluation results and progress in RAN1 could be taken as reference for determination of the model. Further discussion is needed on the extent of the reference model to be specified. The manner of joint design and training could be pursued with continuing efforts inside and outside of 3GPP. If certain consensus could be achieved among industry, update to reference model could be involved in future release.

Proposal 2: Following aspects could be considered when designing reference encoder/decoder in RAN4: 1) minimize the impact of implementation discrepancy; 2) ensure sufficient future-proof of model design; 3) performance efficiency of reference encoder/decoder.

3	Conclusion

In this contribution, we share our initial views on reference block diagram and reference encoder/decoder with following proposals:

Proposal 1: Further clarifications for different elements of reference block diagram should be included.

Proposal 2: Following aspects could be considered when designing reference encoder/decoder in RAN4: 1) minimize the impact of implementation discrepancy; 2) ensure sufficient future-proof of model design; 3) performance efficiency of reference encoder/decoder.

References

RAN4_106bis-e_Main_Session_report_v09_Apr_26_EoM