3GPP TSG-RAN WG4 Meeting #107                                          R4-2307146

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	On UE MSD requirements for FWA UE supporting 3T for two bands

Agenda Item:	7.29.3.2

Document for:	Approval

Introduction

MSD requirements for FWA UE supporting 3Tx were discussed in last meeting. The issues to be further analyzed are recorded in the WF [1]. The issues can be categorized to two kinds, one is for PC2 band combination, and the other one is for PC1.5 band combination.

For PC2 combinations with 3Tx in two bands, the main issue is whether the existing MSD requirements for cross band isolation, harmonic mixing as well as IMD for PC2 with 1Tx at each constituent band could be reused. In our view, though a unified framework is simplified and attractive, which should be based on some detailed analysis.

In this contribution, we would provide further analysis and calculation based on the WF and our preliminary consideration in last meeting [2].

Discussion

Parameters used for MSD analysis

As discussed in RAN4#106 meeting, for the band combinations proposed in the WI, at least the highlighted MSD requirements for 3T listed in the table below should be studied.

Table 0: Proposed band combinations in the WI

| Band combination in WID | MSD evaluation status |
| --- | --- |
| CA_n28A-n41A | - No new MSD will be defined |
| CA_n28A-n78A | - No new harmonic MSD will be defined - New PC2 IMD5 MSD and PC2 Receive harmonic mixing MSD should be defined |
| CA_n8A-n78A | - No new harmonic MSD will be defined - New PC2 IMD4 MSD should be defined |
| CA_n41A-n71A | - No new harmonic MSD will be defined - New PC2 IMD4 MSD should be defined |
| CA_n41A-n77A | - New PC2 harmonic mixing MSD should be defined - New PC2 Cross band isolation MSD should be defined |
| CA_n26A-n78A | - No new harmonic MSD will be defined - New PC2 IMD4 MSD should be defined |
| DC_3A_n78A | - No new harmonic MSD will be defined - New PC2 harmonic mixing MSD and PC2 IMD2/4 MSD should be defined |
| DC_40A_n78A | - New PC2 harmonic mixing MSD should be defined - New PC2 cross band isolation MSD should be defined |

A unique framework on specifying the MSD requirements for 3T combinations with same power class would be preferable, but it still depends on some analysis. Here we use CA_n3-n78 as an example to study the MSD and to see whether the observation and possible conclusion could be extended to other 3T band combinations.

The parameters utilized for CA_n3-n78 MSD analysis are listed in Table 1.

Table 1: Parameters for MSD analysis (CA_n3-n78)

| Parameter | value |
| --- | --- |
| IP2: Antenna switch (dBm) | 115 |
| IP2: Duplexer (dBm) | 100 |
| IP2: PA forward (dBm) | 28 |
| IP2: PA reverse (dBm) | 38 |
| IP2: LNA (dBm) | 6 |
| Antenna isolation (dB) | 10 |
| PCB isolation (dB) | 65 |
| Diplexer isolation (dB) | 15 |
| L-H diplexer IL (dB) | 0.5 |
| M-H diplexer IL (dB) | 1 |
| Antenna switch IL (dB) | 1 |
| Duplexer IL (dB) | 4 |
| Band n3 Tx-Rx duplexer rejection (dB) | 50 |
| Cross band isolation (n3, n78) | 50 |
| n3 Channel BW (MHz) | 5 |

PC2 IMD MSD

Table 2 to 4 are the basis for the further discussion on MSD for CA_n3-n78, in which Table 2 and 3 give the specific values for the main IMD producers in the band combination for main and diversity branches respectively, while Table 4 shows the final calculated MSD after MRC for PC3 band combination.

Table 2: IMD2 level to n3 Rx main path (PC3)

| Main Path | IMD2 |
| --- | --- |
| Diplexer (L-H) | -74 |
| Antenna switch | -71.5 |
| Diplexer (M-H) | -68.5 |
| n3 PA forward | -64.3 |
| n78 PA forward | -83.8 |
| n3 PA reversed | -97.5 |
| n78 PA reversed | -117 |
| Tx total | -60.6 |
| n3 LNA via filters | -71.8 |
| n3 LNA via PCB | -82.3 |
| Total IMD | -60.3 |

Table 3: IMD2 level to n3 Rx diversity path

| Diversity Path | IMD2 |
| --- | --- |
| From main path | -70.3 |
| Diplexer | -94.5 |
| Antenna switch | -94.5 |
| Duplexer | -78.5 |
| n3 LNA via antenna isolation | -90.8 |
| n3 LNA via PCB isolation | -84.3 |
| Total IMD | -69.5 |

Table 4: CA_n3-n78 MSD for n3 Rx due to IMD2 (PC3)

| MSD calculation parameters | Value |
| --- | --- |
| Main path IMD (dBm) | -60.3 |
| Diversity path IMD (dBm) | -69.5 |
| MSD after MRC (dB) | 26 |

Since Table 4 is for PC3 band combination, the maximum power is capped by the per BC power class. Traditionally, the power is equally split between the two constituent bands in the MSD calculation, in other words, 20dBm at each antenna connector for the two bands though the power class for single band could be 23dBm solely.

For the same band combination of power class 2, the similar approach is utilized for MSD calculation. Table 5 gives the MSD for PC2 CA_n3-n78, which is based on 23+23dBm PA configuration. The requirement in the spec is quite close to the value in the table (31.9dB in the spec) for this band combination. It is noted that when the PC2 MSD is introduced for the band combination, increase power limit feature is not considered.

Table 5: CA_n3-n78 MSD for n3 Rx due to IMD2 (PC2 1Tx@n78 w/o increase power limit)

| MSD calculation parameters | Value |
| --- | --- |
| Main path IMD (dBm) | -54.9 |
| Diversity path IMD (dBm) | -64 |
| MSD after MRC (dB) | 31.5 |

Observation 1: PC2 MSD in current specification for an inter-band CA band combination is defined based on 23+23dBm PA configuration, which has not considered increase power limit feature.

In case supporting increase power limit capability, which means the UE can solely transmit 26dBm at n78 and deliver 27.8dBm total output power for the CA band combination with 2Tx. We keep all the component linearity assumptions unchanged and just increase the power at band n78, it can be seen that MSD would be increased as well, though that is not reflected in the specification yet.

Table 6: CA_n3-n78 MSD for n3 Rx due to IMD2 (PC2 1Tx@n78 w/ increase power limit)

| MSD calculation parameters | Value |
| --- | --- |
| Main path IMD (dBm) | -52.2 |
| Diversity path IMD (dBm) | -61.2 |
| MSD after MRC (dB) | 34.2 |

Observation 2: PC2 MSD is different for UE supporting increase power limit feature and the difference is not reflected in the spec yet.

It is noticed that the issued actually was already discussed under the increase power limit WI. Finally, it was agreed in the group that no specific requirements would be defined for the case with 1x26dBm for one constituent band, and the test case is still based on 2x23dBm for a PC2 configuration.

Now we move to the interesting part of MSD for UE supporting 3T. According to the WID, n78 is implemented with PC2 2Tx. Let’s assume 2x23dBm PA configuration for n78. The main interference paths for the second PA for n78 are profiled in Figure 1.

Figure 1: IMD MSD for CA_n3-n78 with 1Tx or 2Tx @ n78

For the second PA of n78 in the diversity path, the PCB isolation could be further enlarged. In Table 1, 65dB PCB isolation is considered for 1Tx, here we could consider 70~75dB isolation. While for reverse IMD, the additional 10dB antenna isolation should be considered.

Table 7: IMD2 level to n3 Rx main path (PC2 1Tx)

| Main Path | IMD2 |
| --- | --- |
| Diplexer (L-H) | -68 |
| Antenna switch | -65.5 |
| Diplexer (M-H) | -62.5 |
| n3 PA forward | -58.3 |
| n78 PA forward | -77.8 |
| n3 PA reversed | -91.5 |
| n78 PA reversed | -111 |
| Tx total | -55.3 |
| n3 LNA via filters | -65.8 |
| n3 LNA via PCB | -76.3 |
| Total IMD | -60.3 |

Here we just give IMD2 impact to main path in Table 7. The IMD2 products at n3 via the additional interfering paths should be added together with existing forwarded and reversed IMD. As observed in Table 7, n3 PA forwarded IMD in the determining factor, for simplicity, we just consider the interference added in this path, i.e. the forward IMD considering the leaked interference from 2nd n78 PA with 5~10dB PCB isolation. 5dB is considered in the following calculation.

Table 8: CA_n3-n78 MSD for n3 Rx due to IMD2 (PC2 2Tx@n78)

| MSD calculation parameters | Value |
| --- | --- |
| Main path IMD (dBm) | -54.3 |
| Diversity path IMD (dBm) | -64 |
| MSD after MRC (dB) | 31.6 |

Observation 3: With rough estimation, due to additional PCB and antenna isolation, the IMD increase caused by the 2nd PA of n78 is minimal.

PC2 Harmonic mixing MSD & cross band isolation MSD

For harmonic mixing, we use CA_n41-n77 as an example. Table 8 and Table 9 are cited from the current specification with PC3 and PC2 aggressor respectively.

Table 8: Harmonic mixing MSD from a PC3 aggressor

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 6.3 | NOTE 2 | UL2/DL3 |

Table 9: Harmonic mixing MSD from a PC2 aggressor

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 8.8 | NOTE 3 | UL2/DL3 |

It is noted that the difference of harmonic mixing MSD between PC3 and PC2 aggressor is about 3dB, which would be different for combinations in the existing specification.

Figure 2: Harmonic mixing MSD for CA_n41-n77

The harmonic mixing interfering path is depicted in the right above figure. Besides the harmonic mixing product for the diversity branch, there also has the leaked Tx signal from the high band via the antenna isolation, but the generated harmonic mixing en route this path would be quite smaller. With simple calculation, the harmonic mixing products would be doubled if only the interferences without mutual impact from the other antenna branch considered. Though the result calculated by 2x23dBm and 1x26dBm for the high TDD band could be slightly different, the level could be comparable.

Observation 4: For harmonic mixing, the MSD caused by 2Tx or 1Tx with same power class for the high aggressor band is comparable.

Regarding cross band isolation, for the proposed combinations in Table 0, only two combinations should consider this kind of MSD, i.e. CA_n41A-n77A and DC_40A_n78A.

Use CA_n41A-n77A as an example, the current specification has cross band isolation MSD requirements for PC3, PC2 and PC1.5 aggressor separately. Specifically, for the TDD band combination, the cross-band isolation MSD occurred for single UL operation only, while for PC2, UE could be implemented as both 2x23dBm or 1x26dBm, the requirements should consider different implementation already and that is line with the current HPUE basket status. Thus, if MSD for PC2 aggressor is specified or to be specified, the requirement should be the same for both 1T and 2T for the aggressor band.

Observation 5: Cross band isolation requirements for some band combination could already consider different implementation for the same power class. In other words, the requirements should be applicable for both 1T and 2T for the aggressor band with same power class.

PC1.5 MSD due to 2UL IMD

So far, we only have one PC1.5 band combination in the WI, i.e. CA_n41A-n71A. For this specific band combination, only IMD4 MSD should be analysed. The specification has requirements for PC3 and PC2 IMD for the combination already. As discussed in section 2.2, the IMD 2T MSD requirements for PC2 are based on 2x23dBm assumption. For the 3T band combination of PC3@n71 1Tx + PC1.5@n41 2Tx, the MSD should be further studied, and no existing requirement could be reused.

Regarding the MSD test configuration discussed in last meeting, i.e.

| MSD test configuration | PC3 FDD band | PC1.5 TDD band |
| --- | --- | --- |
| Option 1 | 23 dBm | 27.8 dBm |
| Option 2 | 23 dBm | [23] dBm |

As observed in section 2.2, the interference from the second PA could be limited, hence the test based on 1Tx of the PC1.5 TDD band could reflect the MSD impact enough. Instead of the two options in last meeting, we propose to only consider the configuration as below.

| MSD test configuration | PC3 FDD band | PC1.5 TDD band |
| --- | --- | --- |
| Option 3 | 23 dBm | 26 dBm |

Observation 6: For IMD MSD test, use the fallback power class with 1T for the 2T band together with the other operating band could reflect the MSD impact enough.

Conclusion

This contribution provides our further consideration and analysis for the MSD requirements for FWA UE supporting 3T with two bands. Upon the analysis, we have the following observations and proposals.

Observation 1: PC2 MSD in current specification for an inter-band CA band combination is defined based on 23+23dBm PA configuration, which has not considered increase power limit feature.

Observation 2: PC2 MSD is different for UE supporting increase power limit feature and the difference is not reflected in the spec yet.

Observation 3: With rough estimation, due to additional PCB and antenna isolation, the IMD increase caused by the 2nd PA of n78 is minimal.

Observation 4: For harmonic mixing, the MSD caused by 2Tx or 1Tx with same power class for the high aggressor band is comparable.

Observation 5: Cross band isolation requirements for some band combination could already consider different implementation for the same power class. In other words, the requirements should be applicable for both 1T and 2T for the aggressor band with same power class.

Observation 6: For IMD MSD test, use the fallback power class with 1T for the 2T band together with the other operating band could reflect the MSD impact enough.

Proposal 1: For PC2 IMD MSD with 2 bands, the requirements specified based on 1Tx-1Tx UL configuration are applicable for 1Tx-2Tx UL configuration.

Proposal 2: For harmonic mixing and crossband isolation MSD with 2 bands, same MSD requirements can be applied for the band combination with either 1Tx or 2Tx in TDD aggressor band UL with same power class.

Proposal 3: For PC1.5 3T band combination, the following test configuration could be adopted:

| MSD test configuration | PC3 FDD band | PC1.5 TDD band |
| --- | --- | --- |
| Option 3 | 23 dBm | 26 dBm |

- References

[1] R4-2306555, WF on Rx requirements for 3Tx UE, Apple

[2] R4-2305565, On UE RF requirements for FWA UE supporting 3T for two bands, Huawei, HiSilicon