3GPP TSG-RAN WG4 Meeting #107                                          R4-2307148

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Further discussion on UE configured with two serving cells and with SUL in each cell

Agenda Item:	11.2

Document for:	Approval

Introduction

The issue for UE configured with two serving cells and with SUL in each cell was discussed in RAN #99. Finally, a RAN task was agreed and content of the task was listed as below.

- UL/SUL indicator field is excluded from a DCI format 0_X.

- Task RAN4 to assess the additional, if any, RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL; report to RAN#100 with the goal of striving for potential normative work supporting the case where a UE is configured with two serving cells, each with SUL

  - E.g., whether back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier could be supported without any switching period, or

  - E.g., whether it is only feasible to support such configuration in the UL Tx switching framework with UE capability based switching period

  - Example band combinations are referred to in RP-223553 (RP-230719)

  - Further check the status in RAN#100

The RAN task was discussed in last RAN4 meeting, and a WF captures the issues to be further clarified [1].

This contribution provides further analysis from both RAN4 spec impact as well as implementation impact for the task, and proposal based the analysis is also provided.

Discussion

Example band combination for the analysis

The following band combinations for two SUL cells are considered in [2].

Table 1: NR Inter-band CA with two SUL cells

| NR CA configuration | Uplink CA configuration or SUL configuration | Power Class |
| --- | --- | --- |
| CA_n41A-n95A_ n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A | PC3 |
| CA_n41A-n98A_ n79A-n95A | SUL_n41A-n98A SUL_n79A-n95A CA_n41A-n79A | PC3 |
| CA_n41A-n83A_ n79A-n98A | SUL_n41A-n83A SUL_n79A-n98A CA_n41A-n79A | PC3 |
| CA_n41A-n83A_ n79A-n95A | SUL_n41A-n83A SUL_n79A-n95A CA_n41A-n79A | PC3 |
| CA_n78C_n81A-n84A | SUL_n78A-n81A SUL_n78A-n84A CA_n78C | PC3 |
| CA_n78C_n80A-n84A | SUL_n78A-n80A SUL_n78A-n84A CA_n78C | PC3 |
| CA_n78C_n84A-n89A | SUL_n78A-n84A SUL_n78A-n89A CA_n78C | PC3 |

From the table, it is seen that in general the CA mechanism is utilized for the band combinations with two SUL cells. In other words, the combinations included in the WI have no difference from other combinations in a different basket or spectrum related WIs proposed by RAN4. Specifically, for the column of UL CA or SUL configurations, it just lists the configurations which should specify the RF requirements, as band combination specific requirements are always studied and specified configuration by configuration as usual, nothing new. And in RAN4 #106 meeting, the draft big CR for the combinations captured in the WID was endorsed, which only define the band combination specific requirements, e.g. delta Tib, Rib, supported BCS, etc. In this sense, we don’t think the WI NR CA band combinations with two SUL cells in Rel-18 has direct relationship to the ongoing WI multi-carrier enhancements discussed in RAN1.

Observation 1: The NR CA band combinations with two SUL cells in Rel-18 is a RAN4 spectrum related WI, which follows the usual study methodology for band combinations to specify the band combination specific requirements.

Though the WI itself has no direct relationship with the RAN1 discussion, to fulfil the RAN task, we would like to take a band combination in the WI, i.e. CA_n41A-n95A_ n79A-n98A, as an example to analyse the RAN4 spec impact and implementation impact for the band combination.

0us switching and capability-based switching

In current RAN4 specification, there are two kinds of switching relevant to SUL. One is 0us switching for SUL band combinations introduced in Rel-15, the other one is switching period capability introduced from Rel-16.

For Rel-15, SUL band combination means UE is configured with both NR UL and NR SUL carriers in a cell and the switching time between NR UL carrier and NR SUL carrier is 0 us. It is noticed that both SUL and NUL in such case is single carrier. The 0us switching is doable based on the assumption that UE has sufficient Tx chains and adjustable PLLs, with which, the Tx chain and frequency can be prepared in advance for the two bands in the combination. It should be noted that such 0us switching is not just limited to SUL band combination for two bands with single carrier in each band from implementation perspective.

Observation 2: 0us switching is for 1 carrier->1 carrier switching with two-band pair involved. For both switching from or switching to, there is only 1 carrier on transmission. For such case, the 0us switching is doable for a band pair by the proper UE implementation and is not limited to band combinations with SUL.

For 0us switching capable UE, there is no specific RAN4 time mask requirements and UE just needs to meet the general time mask requirements.

Observation 3: 0us switching has no specific RAN4 switching requirements.

From Rel-16, the Tx switching evolved to a more advanced feature, i.e. 1T->2T switching in Rel-16 and 2T->2T switching in Rel-17, and more advanced feature for 3/4 bands switching in Rel-18. Nevertheless, the switching has two Tx chains occupied at least for one switching state, either switching from or switching to, or both. The intention of those UL Tx switching features with non-zero switching period is to maximize the usage of multiple spectrum for UE with only 2Tx chains, which would be more typical UE capability for smartphone form factor in the market.

Observation 4: Switching period capability-based Tx switching involves Tx switching with simultaneous transmission on 2Tx chains at least for one switching state, either switching from or switching to, or both.

Observation 5: Switching period capability-based Tx switching has time mask specified for both inter-band CA and SUL from Rel-16.

Cases for the RAN task

For RAN task, one example in the task is:

whether back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier could be supported without any switching period

Take CA_n41A-n95A_ n79A-n98A for instance. Cases for back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier for the example band combination can be considered as:

Case 1:

UE supports one of the SUL band in band combinations of CA_n41A-n95A_ n79A-n98A, e.g. n95 in cell 1 of SUL_n41A-n95A then switches to the other SUL band, e.g. n98 in cell 2 of SUL_n79A-n98A, or vice versa.

Figure 1: Tx switching between two SUL cells (SULband1->SULband2)

It is known that the switching time between SUL and NUL in the same cell is 0us in Rel-15, which means that the Tx chains and PLLs are available in advance for the SUL and NUL band in the band pair. As observed in section 2.2, 0us switching for the case that only one band is on transmission for a two-band pair is doable no matter it is a SUL band combination or a CA band combination.

Normally multiple Tx chains including PA as well corresponding filter are supported by the RFIC and frontend modules, which provides the possibility that Tx chain and PLL are ready for the switching in advance if the preparation could be done in the case with the knowledge of timely scheduling info. For this case, the switching period for SUL bands in two cells could still be 0us.

Case 2:

UE supports one of the SUL band in band combinations of CA_n41A-n95A_ n79A-n98A, e.g. n95 in cell 1 of SUL_n41A-n95A, then switches to the NUL band, e.g. n79, in cell 2 of SUL_n79A-n98A.

Figure 2: Tx switching between two SUL cells (SULband1->NULband2)

Similarly, for switching between SUL band and NUL band in different cells, since there is only one band on transmission, there is no difference compared to the case of Rel-15 SUL band combination from implementation perspective.

Observation 6: 0us switching between two SUL carriers and SUL carrier with non-corresponding NUL carrier for back-to-back transmission can be supported with proper UE implementation.

In addition, since two cells are involved in the switching, the existing requirements of inter-band CA are applicable. No specific RAN4 requirements are foreseen for UE supporting such kind of operation, which is up to NW scheduling.

Observation 7: 0us switching between two SUL carriers and SUL carrier with non-corresponding NUL carrier for back-to-back transmission has no RAN4 spec impact.

While for switching capability-based Tx switching framework, as tasked by RAN for the other example, i.e.

whether it is only feasible to support such configuration in the UL Tx switching framework with UE capability based switching period

Though the case is relatively complicated compared to the former example, as simultaneous transmission could occur on two RF chains, either on two bands or single band considering dualUL or switchedUL modes, the solution is straightforward, and actually the draft big CR for Tx switching time mask for 3/4 bands was endorsed in previous RAN4 meeting [3]. Though there are some remaining issues, but the main framework is quite stable, and the time mask requirement is also applicable to switching between two SUL carriers and SUL carrier with non-corresponding NUL carrier.

Observation 8: Switching period based Tx switching is an advanced switching feature, which covers 1T->2T, 2T->2T and 3/4 bands switching. No specific implementation effort is needed for UE supporting two SUL carriers and SUL carrier with non-corresponding NUL carrier compared to inter-band CA in terms of same Tx switching framework.

Observation 9: The time mask requirements in the endorsed CR can already cover the cases mentioned in the RAN task, and no additional specification impact is foreseen to support a UE configured with two serving cells, each with SUL.

New proposed UL configurations

When the WI of two SUL cells were proposed in RAN#98e, simultaneous SUL transmission was not added as a compromise. However, though the note on SUL configuration in RAN4 understanding is the same as no SUL simultaneous transmission just for RF requirements purpose, it seems some companies forcibly connected it to multi-carrier enhancement feature discussed in RAN1. Based on the RAN task, we think there is no unreasonable limitation on the supported UL configuration anymore as long as there are no additional implementation and specification impact. That’s the reason in the WF the assessment cases are extended to {SUL2,NUL2}, {SUL2,NUL1} and {SUL1,SUL2} with dualUL.

As discussed in section 2.3, we see no difference for UE supporting these UL configurations with simultaneous transmission. The reason is that SUL is just UL frequency range for a NR band. It is even simpler than the combinations with 3/4 NR bands for inter-band CA from standard perspective.

Observation 10: Tx switching frame work, i.e. switching period based solution, is also applicable for newly proposed UL configurations with simultaneous transmission on two bands.

Proposal 1: New UL configurations with two SUL bands and SUL band with non-corresponding NUL band should be added in a revised WID for two SUL cells.

With above analysis and observations, we think a reasonable conclusion from RAN4 perspective could be made and focus on how to support the clear operator demand and better utilize the precious spectrum. The proposed conclusion from RAN4 for the task is as follows:

For a UE configured with two serving cells, each with SUL, RAN4 studied the following cases discussed in RAN #99 plenary meeting:

- E.g., whether back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier could be supported without any switching period, or

- E.g., whether it is only feasible to support such configuration in the UL Tx switching framework with UE capability based switching period

To support 0us Tx switching for back-to-back transmission switching between SUL and SUL or between SUL and NUL for the band combinations with two simultaneous configured SUL cells:

- From implementation perspective, 0us switching period can be supported with proper UE implementation which is equipped with sufficient RF chains, which is the similar case as that for Rel-15 SUL band combination.

- From specification perspective, there is no additional RAN4 spec impact foreseen for back-to-back transmissions between SUL carrier and non-corresponding NUL carrier.

- So it is not the only feasible way to support such configuration by the UL Tx switching framework with UE capability based switching period.

To support non-zero Tx switching (i.e., so-called UE capability based switching period) for back-to-back switching between SUL and SUL or between SUL and NUL for the band combinations with two simultaneous configured SUL cells:

- From specification perspective, the non-zero Tx switching mechanism specified for 3/4 bands in Rel-18 multi-carrier operation WI can enable the back-to-back switching between SUL and SUL or between SUL and NUL for the band combinations with two SUL cells configured.

- From UE implementation perspective, the UE implementation with 2Tx which supports Tx switching mechanism specified for 3/4 bands in Rel-18 multi-carrier operation WI can support the back-to-back switching between SUL and SUL or between SUL and NUL for the band combinations with two SUL cells configured.

We thereby propose to approve a WF to record the RAN4 conclusion for the RAN task and report to RAN in #100 meeting with such a conclusion.

Proposal 2: It is proposed to conclude in RAN4 that from both implementation and spec impact perspective, with completion of CRs for two SUL cells band combinations and Tx switching for 3/4 bands, there is no additional work needed to support a UE configured with two serving cells, each with SUL.

Conclusion

This contribution provides further consideration on the issue for UE configured with two serving cells and with SUL in each cell. Based on the analysis, we have the following observations and proposal:

Observation 1: The NR CA band combinations with two SUL cells in Rel-18 is a RAN4 spectrum related WI, which follows the usual study methodology for band combinations to specify the band combination specific requirements.

Observation 2: 0us switching is for 1 carrier->1 carrier switching with two-band pair involved. For both switching from or switching to, there is only 1 carrier on transmission. For such case, the 0us switching is doable for a band pair by the proper UE implementation and is not limited to band combinations with SUL.

Observation 3: 0us switching has no specific RAN4 switching requirements.

Observation 4: Switching period capability-based Tx switching involves Tx switching with simultaneous transmission on 2Tx chains at least for one switching state, either switching from or switching to, or both.

Observation 5: Switching period capability-based Tx switching has time mask specified for both inter-band CA and SUL from Rel-16.

Observation 6: 0us switching between two SUL carriers and SUL carrier with non-corresponding NUL carrier for back-to-back transmission can be supported with proper UE implementation.

Observation 7: 0us switching between two SUL carriers and SUL carrier with non-corresponding NUL carrier for back-to-back transmission has no RAN4 spec impact.

Observation 8: Switching period based Tx switching is an advanced switching feature, which covers 1T->2T, 2T->2T and 3/4 bands switching. No specific implementation effort is needed for UE supporting two SUL carriers and SUL carrier with non-corresponding NUL carrier compared to inter-band CA in terms of same Tx switching framework.

Observation 9: The time mask requirements in the endorsed CR can already cover the cases mentioned in the RAN task, and no additional specification impact is foreseen to support a UE configured with two serving cells, each with SUL.

Observation 10: Tx switching frame work, i.e. switching period based solution, is also applicable for newly proposed UL configurations with simultaneous transmission on two bands.

Proposal 1: New UL configurations with two SUL bands and SUL band with non-corresponding NUL band should be added in a revised WID for two SUL cells.

Proposal 2: It is proposed to conclude in RAN4 that from both implementation and spec impact perspective, with completion of CRs for two SUL cells band combinations and Tx switching for 3/4 bands, there is no additional work needed to support a UE configured with two serving cells, each with SUL.

For a UE configured with two serving cells, each with SUL, RAN4 studied the following cases discussed in RAN #99 plenary meeting:

- E.g., whether back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier could be supported without any switching period, or

- E.g., whether it is only feasible to support such configuration in the UL Tx switching framework with UE capability based switching period

To support 0us Tx switching for back-to-back transmission switching between SUL and SUL or between SUL and NUL for the band combinations with two simultaneous configured SUL cells:

- From implementation perspective, 0us switching period can be supported with proper UE implementation which is equipped with sufficient RF chains, which is the similar case as that for Rel-15 SUL band combination.

- From specification perspective, there is no additional RAN4 spec impact foreseen for back-to-back transmissions between SUL carrier and non-corresponding NUL carrier.

- So it is not the only feasible way to support such configuration by the UL Tx switching framework with UE capability based switching period.

To support non-zero Tx switching (i.e., so-called UE capability based switching period) for back-to-back switching between SUL and SUL or between SUL and NUL for the band combinations with two simultaneous configured SUL cells:

- From specification perspective, the non-zero Tx switching mechanism specified for 3/4 bands in Rel-18 multi-carrier operation WI can enable the back-to-back switching between SUL and SUL or between SUL and NUL for the band combinations with two SUL cells configured.

- From UE implementation perspective, the UE implementation with 2Tx which supports Tx switching mechanism specified for 3/4 bands in Rel-18 multi-carrier operation WI can support the back-to-back switching between SUL and SUL or between SUL and NUL for the band combinations with two SUL cells configured.

- References

[1] R4-2306651, WF on RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL, CMCC

[2] RP-230719, Revised WID: NR CA band combinations with two SUL cells in Rel-18, CMCC, China Telecom

[3] R4-2303719, CR for 38.101-1: Time mask for switching across three or four uplink bands, China Telecom, NTT DOCOMO, Huawei, Hisilicon, CMCC, OPPO, ZTE