3GPP TSG-RAN WG4 Meeting #107	R4-2307157

Incheon, KR, May 22 – May 26, 2023

Source:	Huawei, Hisilicon

Title:	On ambiguity issue of the exact switching band pairs

Agenda item:			8.24.2

Document for:	Approval

# 1. Introduction

In the contribution, we discuss the ambiguity issue of the exact switching band pairs on Rel-18 multi-carrier Tx switching.

# 2. Discussion

In RAN4#105, the baseline UE assumption was agreed for the ambiguity issue when two Tx chains are switched between two different band pairs.

For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, and it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}, RAN4 agreed that: - As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively.

The reason to introduce the baseline UE assumption is that the network only schedules transmissions on switch-from bands and switched to bands, rather than indicating a Tx chain to switch in the exact band pair.

However, it was proposed to solve the ambiguity issue of the exact switching band pairs for the concurrent switching of two Tx chain in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}. The switching period resulting from the baseline assumption max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C} is related to four band pairs. In this case, the UE performs switching only in two band pairs and it is possible to have a shorter switching period than the baseline value.  The UE cannot utilize the spare resource to transmit uplink channel/signal as baseline.

Observation 1: The switching period resulting from the baseline assumption max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C} is related to four band pairs. In this case, the UE performs switching only in two band pairs and it is possible to have a shorter switching period than the baseline value.

In the last meeting, the WF [1] on Tx switching ambiguity issue was approved.

- RAN4 maintains the baseline assumption in Issue 1-2-3 agreed in R4-2220546 during RAN4#105. - Neither of the two Tx chains is expected to be used for transmission during the switching periods in Rel-18 - Encourage companies to study the benefit to resolve the switching pattern ambiguity issue when 2 Tx chains are switched between two different band pairs for one Tx switching instance - At least for single TAG - FFS whether it applies to dual TAG

There are two possible switching cases in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}:

Case #1: A-> C, B -> D   period#1

Case #2: A-> D, B -> C   period#2

If period#1, as the switching period of case#1, is longer than period#2, period #1 is the baseline max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. In Rel-18, the set of switching period value is {35μs, 140μs, 210μs}. The largest difference between period#2 and period #1 is up to 175μs, which is equivalent to 5 OFDM symbols for 30kHz subcarrier spacing as shown in Figure 1.

Therefore, there could be 35.7% more uplink resource than the baseline assumption in one slot that can be used if the UE can perform Tx switching with the two band pairs corresponding to shorter switching period.

- Case #1: A-> C, B -> D   period#1                 (b) Case #2: A-> D, B -> C   period#2

Figure 1 illustration of two possible switching cases in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}

If the UE is able to perform Tx switching with the band pairs A->D, B->C as Case#2 in Figure 1(b) that is corresponded to shorter switching period, the UE could utilize the symbol#8~ symbol#12 to transmit uplink signal/channels compared to Case#1.

Observation 2: If the UE is able to perform Tx switching with the band pairs that are corresponded to shorter switching period, the UE could utilize more resources to transmit uplink signal/channels compared to baseline.

During the discussion of Rel-18 Tx switching, we have introduced and have been trying to introduce several optional/advanced UE capability based on UE implementation to get better performance.

#### RAN4#105 Way forward: - For the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching.

#### RAN4#106bis - Further discuss the following options in the next meeting - Option 1: For the switching scenario without ambiguity, i.e., the two Tx chains are on the same band before or after switching (switch between {1T, 1T, 0T} and {0T, 0T, 2T}), introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1). - Tswitch_1 and Tswitch_2 are the switching periods of Tx chain #1 and Tx chain #2 respectively, and Tswitch_1 < Tswitch_2. - Option 1a: apply the same UE capability for this scenario and for the following scenario agreed in the previous meetings. (MTK, China Telecom, ZTE) - Option 2: Not introduce advanced optional UE ability

Observation 3: Rel-18 Tx switching introduces optional/advanced UE capability based on UE implementation to get better performance

If the UE is advanced enough and able to choose the pair of band pairs with shorter switching period, as an optional UE capability, the UE can report min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} for the two pairs of band pair as the switching period for {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}.

Proposal 1: For the parallel switching of two Tx chain in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}, in addition to the baseline UE assumption agreed in RAN4 #105, introduce optional UE capability with the switching period min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

# 3. Conclusions

This contribution presents the following observations and proposals:

Observation 1: The switching period resulting from the baseline assumption max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C} is related to four band pairs. In this case, the UE performs switching only in two band pairs and it is possible to have a shorter switching period than the baseline value.

Observation 2: If the UE is able to perform Tx switching with the band pairs that are corresponded to shorter switching period, the UE could utilize more resources to transmit uplink signal/channels compared to baseline.

Observation 3: Rel-18 Tx switching introduces optional/advanced UE capability based on UE implementation to get better performance.

Proposal 1: For the parallel switching of two Tx chain in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}, in addition to the baseline UE assumption agreed in RAN4 #105, introduce optional UE capability with the switching period min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

# 4. Reference

[1]	R4-2306660, “WF on Tx switching ambiguity issue”, MediaTek Inc, TSG RAN4 Meeting #106bis-e.