3GPP TSG-RAN WG4 Meeting #107	R4-2307161

Incheon, KR, May 22 – May 26, 2023

Source:	Huawei, Hisilicon

Title:	On the inconsistency of intra-band EN-DC configuration

Agenda item:			10.2.3

Document for:	Approval

# 1. Introduction

In the contribution, we discuss the inconsistency issue of intra-band EN-DC configuration.

# 2. Discussion

In RAN4#105, the LS[1] has been sent to ask RAN2 to check whether the interpretation of ‘both’ can be supported by RAN2 for Case 3 and Case 4 based on the conclusion of the approved WF[2].

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL, including

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: One of LTE carriers and the NR carrier are contiguous in DL, contiguous and non-contiguous are both supported in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

In RAN2’s reply LS[3], from the perspective of signalling design, the intention of ‘both’ in the legacy intraBandENDC-Support is to represent that the UE supports both contiguous and non-contiguous intra-band EN-DC for the same band entries as illustrated in [3] with scenario #4.

Table 1 intra-band EN-DC scenarios indicated ‘both’ by intraBandENDC-Support in [3]

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 4 | Both | Absent | - Contiguous/contiguous - Non-contiguous/Non-contiguous |

In this case, taking 48C+n48A as an example, the UE reporting ‘both’ with intraBandENDC-Support, the UE support two pairs UL/DL configurations as shown in Figure 1.

- Configuration #1: Contiguous/contiguous   DL DC_(n)48CA, UL DC_(n)48AA

- Configuration #2: Non-contiguous/Non-contiguous   DL DC_48C_n48A, UL DC_48A_n48A

Observation 1: In RAN2’s understanding, if the UE reports ‘both’ with intraBandENDC-Support for 48C+n48A, it supports

- Configuration #1: DL DC_(n)48CA, UL DC_(n)48AA, and

- Configuration #2: DL DC_48C_n48A, UL DC_48A_n48A.

Figure 1 UE supports both contiguous and non-contiguous intra-band EN-DC for the same band entries with ‘both’

In RAN2’s understanding, the legacy signalling intraBandENDC-Support cannot be interpreted to a configuration with different contiguity of UL and DL in case there would be forward backward compatibility issues considering signaling design. Thus a new signaling has to be introduced for UL contiguity for intra-band EN-DC configuration to indicate Case 3, with Scenario#2 in [3] as shown in Table 2.

Table 2 Case 3 indicated by intraBandENDC-Support-UL in [3]

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 2 | Absent (Contiguous) | Non-contiguous | - Contiguous/Non-contiguous - NOTE: “Case 3” in R2-2300060 (R4-2220837) |

Figure 2 UE supports Case 3 with ‘non-contiguous’ indicated by intraBandENDC-Support-UL

Observation 2: In RAN2’s understanding, Case 3 can be indicated by ‘Non-contiguous’ with intraBandENDC-Support-UL and absent with intraBandENDC-Support.

Actually, there are two pairs of UL/DL configuration for EN-DC band combination DC_48A_(n)48AA.

- Configuration #1: Contiguous/contiguous   DL DC_48A_(n)48AA, UL DC_(n)48AA.

- Configuration #2: Contiguous/Non-contiguous   DL DC_48A_(n)48AA, UL DC_48A_n48A.

For the configuration#1 in DC_48A_(n)48AA, it can be covered by the legacy signalling intraBandENDC-Support given that the contiguity of UL and DL is consistent in contiguous.

Table 3 DL DC_48A_(n)48AA, UL DC_(n)48AA indicated by intraBandENDC-Support-UL in [3]

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous |

Figure 3 UE supports DL DC_48A_(n)48AA, UL DC_(n)48AA as intra-band contiguous EN-DC configuration

Figure 4 UE supports DL DC_48A_(n)48AA, DC_48A_n48A with ‘non-contiguous’ indicated by intraBandENDC-Support-UL

Observation 3: DL DC_48A_(n)48AA, UL DC_(n)48AA is an intra-band contiguous EN-DC configuration according to RAN2 LS. And the configuration is in the table defined for mixed intra-band contiguous and non-contiguous EN-DC in RAN4 specification.

Proposal 1: The configuration of DL DC_48A_(n)48AA, UL DC_(n)48AA should be moved to the tables defined for intra-band contiguous EN-DC configuration Table 5.3B.1.2-1 and Table 5.5B.2-1 in 38.101-3.

For the configuration#2, the contiguity relationship between UL and DL is same as that in Case 3, so is the signalling indication in Table 2. To illustrate both the configurations, Case 4 is shown with Scenario#6 in [3].

Table 4 Case 4 indicated by intraBandENDC-Support-UL in [3]

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) |
| --- | --- | --- | --- |
| 6 | Absent (Contiguous) | Both | - Contiguous/Contiguous - Contiguous/Non-contiguous - NOTE: “Case 4” in R2-2300060 (R4-2220837) |

Observation 4: In RAN2’s understanding, Case 4 can be indicated by ‘both’ with intraBandENDC-Support-UL and absent with intraBandENDC-Support .

Observation 5: From RAN2’s perspective, the new signaling intraBandENDC-Support-UL has to be introduced to indicate Case 3 and Case 4.

In our view, the detail of the RAN2 signaling would not be reflected in RAN4 specification. It is RAN4’s task to clarify the band combination with configurations that need to be indicated by the new signaling intraBandENDC-Support-UL introduced in RAN2.

Proposal 2: RAN4 should clarify in 38.101-3 the EN-DC configurations in the following table that need to be indicated by the new signaling intraBandENDC-Support-UL introduced in RAN2.

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |
| DC_48A-(n)48AA | DC_48A_n48A |

In RAN2’s LS [3], ‘both’ indicated by either intraBandENDC-Support or intraBandENDC-Support-UL includes at least two EN-DC configurations. For example, Case 4 is the combination of Scenario#1 and Scenario#2.

Table 5 Configuration combination in Case 4 indicated by intraBandENDC-Support-UL in [3]

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) | Configuration for DC_48A_(n)48AA |
| --- | --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous | - DL DC_48A_(n)48AA - UL DC_(n)48AA. |
| 2 | Absent (Contiguous) | Non-contiguous | - Contiguous/Non-contiguous | - DL DC_48A_(n)48AA - UL DC_48A_n48A |

Observation 6: Case 4 includes two configurations for DC_48A_(n)48AA.

It is necessary for RAN4 to clarify each configuration, and ‘both’ would involve multiple combinations of configuration that is not required in RAN4 specification.

Proposal 3: Combinations of EN-DC configurations indicated by ‘both’ with intraBandENDC-Support or intraBandENDC-Support-UL are not required to be reflected in RAN4 specification as in the following table.

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A-(n)48AA | DC_(n)48AA DC_48A-n48A |

# 3. Conclusions

This contribution presents the following observations and proposals:

Observation 1: In RAN2’s understanding, if the UE reports ‘both’ with intraBandENDC-Support for 48C+n48A, it supports

- Configuration #1: DL DC_(n)48CA, UL DC_(n)48AA, and

- Configuration #2: DL DC_48C_n48A, UL DC_48A_n48A.

Observation 2: In RAN2’s understanding, Case 3 can be indicated by ‘Non-contiguous’ with intraBandENDC-Support-UL and absent with intraBandENDC-Support.

Observation 3: DL DC_48A_(n)48AA, UL DC_(n)48AA is an intra-band contiguous EN-DC configuration according to RAN2 LS. And the configuration is in the table defined for mixed intra-band contiguous and non-contiguous EN-DC in RAN4 specification.

Observation 4: In RAN2’s understanding, Case 4 can be indicated by ‘both’ with intraBandENDC-Support-UL and absent with intraBandENDC-Support .

Observation 5: From RAN2’s perspective, the new signaling intraBandENDC-Support-UL has to be introduced to indicate Case 3 and Case 4.

Observation 6: Case 4 includes two configurations for DC_48A_(n)48AA.

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | UE supports in DL / UL (if applicable) | Configuration for DC_48A_(n)48AA |
| --- | --- | --- | --- | --- |
| 1 | Absent (Contiguous) | Absent | - Contiguous/Contiguous | - DL DC_48A_(n)48AA - UL DC_(n)48AA. |
| 2 | Absent (Contiguous) | Non-contiguous | - Contiguous/Non-contiguous | - DL DC_48A_(n)48AA - UL DC_48A_n48A |

Proposal 1: The configuration of DL DC_48A_(n)48AA, UL DC_(n)48AA should be moved to the tables defined for intra-band contiguous EN-DC configuration Table 5.3B.1.2-1 and Table 5.5B.2-1 in 38.101-3.

Proposal 2: RAN4 should clarify in 38.101-3 the EN-DC configurations in the following table that need to be indicated by the new signaling intraBandENDC-Support-UL introduced in RAN2.

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |
| DC_48A_(n)48AA | DC_48A_n48A |

Proposal 3: Combinations of EN-DC configurations indicated by ‘both’ with intraBandENDC-Support or intraBandENDC-Support-UL are not required to be reflected in RAN4 specification as in the following table.

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A-n48A |

# 4. Reference

[1]	R4- 2220837, “LS on intraBandENDC-Support”, TSG RAN4 Meeting #105.

[2]	R4- 2220589, “WF on intra-band EN-DC support capability”, OPPO, TSG RAN4 Meeting #105.

[3]	R2- 2304431, “Reply LS on intraBandENDC-Support”, TSG RAN2 Meeting #121bis-e.