3GPP TSG-RAN WG4 Meeting #107                                           R4-2307172

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Simultaneous Rx/Tx analysis for CA_n40-n41

Agenda Item:	7.27.2

Document for:	Approval

Introduction

A new WI on simultaneous Rx/Tx was approved in RAN#97e, in which CA_n40A-n41A is listed as one band combination to be analysed [1]. This contribution provides cross band isolation MSD analysis for CA_n40-n41.

Discussion

Cross band isolation for CA_n40-n41

Companies has discussed the calculation of MSD value for CA_n40A-n41A for several meetings. And in RAN4#106-bis-e, the averaged MSD value with bracket for the band combination was captured in TR38.894. We propose to confirm the MSD value in the last meeting and remove the brackets.

Table 5.3.3.2-1a: PC3 Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A in TR38.894

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | [28.1] | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | [31.4] | ACLR2 |

Table 5.3.3.2-1b: PC2 Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A in TR38.894

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | [31.1] | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | [34.4] | ACLR2 |

Proposal 1: Confirm the MSD value for CA_n40A-n41A for PC3 and PC2 in TR38.894, and remove the brackets.

Capabilities related to CA_n40-n41

In RAN4#106bis-e, [2] proposed to decouple the UE capability of simultaneous Rx/Tx and UL MIMO/TxD/SRS antenna switching for CA_n40-n41, considering the frequency range of the two bands is too close for the band combination to meet design requirements of all antennas. In our observation, simultaneous Rx/Tx, as well as SRS Antenna switching and TxDiversity is a per-bandcombination capability. The support of UL MIMO can be reported per carrier. If simultaneous Rx/Tx is mandatorily supported for CA_n40-n41, it is possible for n40/n41 to report UL MIMO at the same time. But considering the MSD value defined for CA_n40-n41 is up to 30+dB, the requirement is not too hard to meet. Thus it is not required to decouple the capability of simultaneous Rx/Tx and UL MIMO/TxD or SRS antenna switching for CA_n40-n41.

BandCombination ::=                 SEQUENCE {

bandList                            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination               FeatureSetCombinationId,

ca-ParametersEUTRA                  CA-ParametersEUTRA                          OPTIONAL,

ca-ParametersNR                     CA-ParametersNR                             OPTIONAL,

mrdc-Parameters                     MRDC-Parameters                             OPTIONAL,

supportedBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL,

powerClass-v1530                    ENUMERATED {pc2}                            OPTIONAL

}

CA-ParametersNR ::=                 SEQUENCE {

dummy                                         ENUMERATED {supported}      OPTIONAL,

parallelTxSRS-PUCCH-PUSCH                     ENUMERATED {supported}      OPTIONAL,

parallelTxPRACH-SRS-PUCCH-PUSCH               ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxInterBandCA                   ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxSUL                           ENUMERATED {supported}      OPTIONAL,

diffNumerologyAcrossPUCCH-Group               ENUMERATED {supported}      OPTIONAL,

diffNumerologyWithinPUCCH-GroupSmallerSCS     ENUMERATED {supported}      OPTIONAL,

supportedNumberTAG                            ENUMERATED {n2, n3, n4}     OPTIONAL,

...

}

Observation 1: The MSD value defined for CA_n40-n41 is large and it is not too hard for the UE to meet the requirement.

Proposal 2: It is not required to decouple the capability of simultaneous Rx/Tx and UL MIMO/TxD or SRS antenna switching for CA_n40-n41.

[3] proposed to relax the 4Rx requirement of n41 if carrier aggregation is configured with band n40 and simultaneous Rx-Tx. Our initial concern is the transmission on band n40 may reuse the two antenna of band n41, and the MSD should be re-considered due to the insufficient isolation of the filter, rather than preventing band n41 from supporting 4Rx. It is mandatory for the band >2.6G to support 4Rx is RAN decision. However, the large value of CA_n40-n41 MSD has been defined, and the relaxation is also not required.

Proposal 3: It is not required to relax the 4Rx requirement of n41 if carrier aggregation is configured with band n40 and simultaneous Rx-Tx.

Conclusion

This contribution provides further simultaneous Rx/Tx analysis for CA_n40-n41 with the following observations and proposals.

Observation 1: The MSD value defined for CA_n40-n41 is large and it is not too hard for the UE to meet the requirement.

Proposal 1: Confirm the MSD value for CA_n40A-n41A for PC3 and PC2 in TR38.894, and remove the brackets.

Table 5.3.3.2-1a: PC3 Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A in TR38.894

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.1 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | ACLR2 |

Table 5.3.3.2-1b: PC2 Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A in TR38.894

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 31.1 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 34.4 | ACLR2 |

Proposal 2: It is not required to decouple the capability of simultaneous Rx/Tx and UL MIMO/TxD or SRS antenna switching for CA_n40-n41.

Proposal 3: It is not required to relax the 4Rx requirement of n41 if carrier aggregation is configured with band n40 and simultaneous Rx-Tx.

- References

[1] RP-222648, New WID Simultaneous RxTx inter-band combinations in Rel-18, Huawei, HiSilicon

[2] R4-2304871, Discussion simultaneous Rx/Tx inter-band combinations for CA_n40-n41, MediaTek Inc.

[3] R4-2304319, On simultaneous Rx-Tx for NR inter-band combinations, Apple.