3GPP TSG-RAN WG4 Meeting #107                                         R4-2307208

Incheon, Korea, May 22nd – May 26th, 2023

Source: 	Huawei, HiSilicon

Title: 	Clarification in UL-MIMO requirements

Agenda Item:	5.2.10.2

Document for:	Approval

Introduction

In RAN4#106 meeting, there was discussion regarding the ambiguity in UL-MIMO requirements, and CR R4-2300824 was agreed to address some issue. However, there are still some ambiguity remaining in the specification, and this document provides some thinking.

Discussion

In RAN4#106 meeting, CR R4-2300824 was agreed to clarify the single antenna port transmission is based on precoding matrix W=1. The changes are only for 6.2D and 6.2H. Similar changes need to be added to 6.3D, 6.3H, 6.4D, 6.4H, 6.5D and 6.5H to ensure the specification consistency.

Proposal 1: RAN4 to clarify single antenna port transmission is based on precoding matrix W=1 in 6.3D, 6.3H, 6.4D, 6.4H, 6.5D and 6.5H.

In sections 6.3D, 6.4D and 6.5D, when UE is scheduled for single antenna port transmission, the requirement refers back to 6.3, 6.4 or 6.5 without considering UE’s TxD capability. If UE supports TxD, the requirements in 6.3G, 6.4G and 6.5G shall apply.

Proposal 2: RAN4 to clarify requirements in 6.3G, 6.4G and 6.5G shall apply for single antenna port transmission if UE supports TxD.

Below is the example of how proposal 1 and proposal 2 are implemented in 6.3D.1

In RAN4#106 meeting, there was also discussion on the power measurement when ULFPTx requirement is verified with the PUSCH configuration in Table 6.2D.1-3. There were different opinions on whether the power should be measured as sum of both antenna connectors.

For ULFPTx mode-1, with the precoding matrix W =, both antenna connectors are expected to be used for Tx transmission, therefore the power measurement shall be the sum of both antenna connectors.

For ULFPTx mode-2 and mode-full power, with the precoding matrix W = or , only single antenna port is used. However the antenna virtualization method was supported in both RAN1 and RAN4. RAN1 even introduced new type of SRS resource set including SRS resources with different port number to support the antenna virtualization. Therefore the power measurement also need to be done as the sum of both antenna connectors to allow for different UE implementations.

Companies might want to confirm UEs supporting ULFPTx mode-2 or mode-full power do have a full power PA. This could be ensured by the test of single antenna port transmission in 6.2x, where the power class is verified against single antenna connector.

In summary, the power measurement when ULFPTx requirement is verified with the PUSCH configuration in Table 6.2D.1-3 should be done as sum of both antenna connectors.

Proposal 3: RAN4 to clarify requirements of ULFPTx with PUSCH configuration in Table 6.2D.1-3 are measured as sum of both antenna connectors.

Conclusion

In this contribution we discussed how to handle the ambiguity in UL-MIMO test cases. According to the analysis, we have the following proposals. The proposals are implemented in accompanying CR R4-2307209.

Proposal 1: RAN4 to clarify single antenna port transmission is based on precoding matrix W=1 in 6.3D, 6.3H, 6.4D, 6.4H, 6.5D and 6.5H.

Proposal 2: RAN4 to clarify requirements in 6.3G, 6.4G and 6.5G shall apply for single antenna port transmission if UE supports TxD.

Proposal 3: RAN4 to clarify requirements of ULFPTx with PUSCH configuration in Table 6.2D.1-3 are measured as sum of both antenna connectors.

Reference

[1] R4-2300823, Corrections to the section 6.2D, Qualcomm, RAN4#106

[2] R4-2300824, CR for 38.101-1 Corrections to the section 6.2D, Qualcomm, RAN4#106