3GPP TSG-RAN WG4 Meeting # 107	R4-2307229

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	8.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.877: Terms and Symbols

Document for:	Approval

# 1.	Introduction

The revised SID on NR BS RF requirement evolution was approved at TSG RAN#97-e [1]. The objective of this study item is to study certain aspects for FR2 multi-band BS, including the following aspects relating to FR2-1 multi-band BS:

    - Investigate whether a generic solution for all combinations within FR2-1 is possible and/or a solution for all or a part of the frequency range should be targeted

      - Frequency range 24-29 GHz which includes n257/n258/n261

      - Frequency range 37-48 GHz which includes n260/n259/n262

This contribution provides a text proposal to the mandatory clauses of Term and Symbols in TR 38.877 [2] based on those defined in TS 38.104 [3].

# 2.	Text proposal

<Start of change>

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

array element: subdivision of a passive antenna array, consisting of a single radiating element or a group of radiating elements, with a fixed radiation pattern

antenna array: group of radiating elements characterized by the geometry and the properties of the array elements

Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the BS channel bandwidth.

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain AAS BS antenna array, there may be more than one beam.

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

directivity: ratio of the radiation intensity in a given direction from the antenna to the radiation intensity averaged over all directions

NOTE:	If the direction is not specified, the direction of maximum radiation intensity is implied.

equivalent isotropic radiated power: in a given direction, the relative antenna gain of a transmitting antenna with respect to the antenna gain of an isotropic radiating element multiplied by the net power accepted by the antenna from the connected transmitter

NOTE:	For an AAS BS the EIRP can be seen as the equivalent power radiated from an isotropic radiating element, producing the same field intensity as the field intensity radiated in the declared beam pointing direction of the active antenna system being considered.

equivalent isotropic sensitivity: power level relative to an isotropic antenna that is required to be incident on the AAS BS array from a specified azimuth/elevation direction in order to meet a specified receiver sensitivity requirement

NOTE:	EIS is directly related to field-strength via free-space impedance and effective aperture antenna area. EIS is expressed as the receiver power that would be collected by an isotropic antenna if it were subject to a uniform field around the whole sphere as the AAS BS array experiences in the specified azimuth/elevation direction.

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements.

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

radiating element: basic building block of an array element characterized by its radiation properties

radiation pattern: angular distribution of the radiated electromagnetic field or power level in the far field region

radio distribution network: passive network which distributes radio signals generated by the active transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the active transceiver unit array.

NOTE:	The number of transmission outputs from the RDN should be greater than or equal to the number of transmission inputs for a single frequency.

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping.

single-band RIB: operating band specific RIB supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band RIB.

sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

total radiated power: is the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.  Total radiated power is defined in both the near-field region and the far-field region

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

FPBWhigh	Highest supported frequency, for which percentage bandwidth support was declared

FPBWlow	Lowest supported frequency, for which percentage bandwidth support was declared

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AA	Antenna Array

AAS	Antenna Array System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

ADC	Analog-Digital Converter

BB	Base Band

BS	Base Station

BW	Bandwidth

CFR	Crest Factor Reduction

CMOS	Complementary Metal-Oxide-Semiconductor

DAC	Digital-to-Analog Converter

DPD	Digital Pre-Distortion

EIRP	Effective Isotropic Radiated Power

EIS 	Equivalent Isotropic Sensitivity

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

IC	Integrated Circuit

IM	Inter-Modulation

LNA	Low Noise Amplifier

MCS	Modulation and Coding Scheme

MIMO	Multiple-Input Multiple-Output

NF	Noise Figure

N/PMOS	N/P-channel Metal-Oxide Semiconductor

NR	New Radio

OBUE	Operating Band Unwanted Emissions

OOB	Out-of-band

OTA	Over-The-Air

PA	Power Amplifier

PAE	Power Added Efficiency

PBW	Percentage Bandwidth

RB	Resource Block

RDN	Radio Distribution Network

RF	Radio Frequency

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RX	Receiver

SCS	Sub-Carrier Spacing

SOI	Silicon On Insulator

TX	Transmitter

TRP	Total Radiated Power

TRX	Transceiver

VGA	Variable Gain Amplifier

VSWR	Voltage Standing Wave Ratio

ZF	Zero Forcing

<End of change>

# References

[1]	RP-222501, “SID revision: NR BS RF requirement evolution”, Huawei, HiSilicon.

[2]	R4-2307753, “TR 38.877 v0.4.0”, Huawei, HiSilicon.

[3]	3GPP TS 38.104 v18.1.0: “NR; Base Station (BS) radio transmission and reception”.