3GPP TSG-RAN WG4 Meeting #107	                                                  R4-2307233

# Incheon, South Korea, May 22 – May 26, 2023

Source:		Nokia, Nokia Shanghai Bell

Title:	Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception

Agenda item:		8.8.2.2

Document for:	Approval

# Introduction

The revised work item on Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception was approved at TSG RAN#98e [1]. One of the objectives of this work item is to:

•	Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs.

Moreover, it is stated in the WID [1] that:

•	The legacy spherical coverage requirement for reception from a single direction will be kept

And that:

•	PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized.

The subject was discussed at TSG RAN4#106-bis-e and the WF was agreed [2].

# Discussion

In RAN4 #106-bis-e meeting, a common WF [2] for agenda items 5.8.2.1 and 5.8.2.2, i.e., ‘System parameter assumption, UE architecture and conditions of UE RF requirements’ and ‘UE RF requirements’ was approved, and the agreements related to the agenda item 5.8.2.2 are provided below.

This contribution provides Nokia’s further views on ‘UE RF requirements for FR2-1 multi-Rx chain DL reception’ topic for defining the RF requirements for FR2-1 multi-Rx chain DL reception from two directions for PC3 UEs.

  - Data bias in agreed TE assumptions

Agreement: - A TE system that uses a legacy 2-axis positioner as measurement setup for multi-Rx UE RF testing will introduce different measurement results for different UE orientations because test AoA pairs only lie along meridians of the UE centric grid. i.e., no test AoA pairs in other directions. - The work on the enhanced positioner, including motivation, conceptual solution, implementation aspects shall be recorded in the accompanying SI TR38.871 as record of having considered the technical aspects associated with the legacy positioner. - To balance pragmatic considerations against the goal of removing the different measurement results for different UE orientations, the enhanced positioner is not adopted in testing and the requirement is agreed to be framed based on the previously agreed legacy positioner. - The requirement applies for one of the FFS options below: Option 1: one or more UE-declared orientations in the positioner - FFS if UE orientation is packaged with an AoA separation angle as a pair. Option 2: one or more standard-specified UE orientations in the positioner. Option 3: the requirement applies for two fixed AoA configurations, the UE orientation can be declared (no capability needed) for each of these two directions.

The test conditions and angle definitions for three permitted device alignment for the default test condition is specified in clause C.2 of TR 38.810. Figure 1 details the three default alignment options.

Figure 1: Different DUT alignment options

Alignment options shown in Figure 1 are based on the test set up and based on the different orientations, test results can be different. Such biased measurement test results cannot be avoided as the recent agreement from the last RAN4 meeting was not to upgrade the test equipment (from 2D positioner to 3D) to remove the bias, and it is not practically feasible to test the UE for all possible UE orientations. Therefore, Option 2 is preferable and RAN4 to decide the UE orientation(s) for the test.

Observation 1: Spherical coverage measurement result can be different based on the different UE orientation.

Proposal 1: Reference UE orientation(s) has to be defined for the test.

  - UE RF requirement metric

#### Equation for metric Option 1: (R4-2304603) For a specific angular separation between 2 TRPs and a specific UE orientation under standardized DL power level which is equal between 2 TRPs, the result at each test point is constructed based on two AoA pairs containing that test point, i.e., AoA+ pair and AoA- pair. Overall result (probability to support 2TRP DL) is by averaging regional results. Option 2: (R4-2305750) The result is averaged over all tested AoA pairs. Weighting is products of the area weights for both AoAs in the pair (AW = sinθ1.sinθ2) and the average is further normalized by the sum of the weights (see figure to the right) used in the integration. Expression below is shown for + and – AoA offset ‘c’. Agreement: - Take option 1 as baseline - Option 2 and other options with complete theoretical explanations are not precluded. Companies are encouraged to investigate physical meaning of both options in future meetings

For multi-Rx RF test with go-no go method, every test point in a constant step size grid type is linked with two AoA pairs, +AoA pair and -AoA pair. This means that for a specific AoA separation and UE orientation, there will be two sets of test probe resulting in two independent outcomes for a given test point. To generate the test result at each test point, these two independent outcomes corresponding to +AoA and -AoA offset need to be treated independently.

Observation 2: For a fixed AoA separation and UE orientation, the test results corresponding to +AoA offset and -AoA offset will be independent.

Proposal 2: The test outcome corresponding to +AoA offset and -AoA offset should be treated independently to generate the result.

  - On area weight type for spatial average

On area weight type for spatial average Agreement: Either sine or Clenshaw-Curtis weights can be used for summing or averaging data collected on a lat-long grid. - In the simulation, sine is used as baseline for simplicity

The spatial arrangement of the test grid points in a constant step size grid type is non-uniform. Therefore, it is inaccurate to arrive at a decision based only on the number of test points complied with the requirements. Two methods to solve this issue of non unform distribution of grid points are specified in clause M.4.2 of TS 38.521-2.

<Start of extraction>

Table M.4.2.1-2: Samples and weights for the classical sin q×Dq weighting and Clenshaw-Curtis quadratures with 13 latitudes (Dq=15o)

| Classical sinq×Dq |  | Clenshaw-Curtis |  |
| --- | --- | --- | --- |
| q [deg] | Weights | q [deg] | Weights |
| 0 | 0 | 0 | 0.007 |
| 15 | 0.0678 | 15 | 0.0661 |
| 30 | 0.1309 | 30 | 0.1315 |
| 45 | 0.1851 | 45 | 0.1848 |
| 60 | 0.2267 | 60 | 0.227 |
| 75 | 0.2529 | 75 | 0.2527 |
| 90 | 0.2618 | 90 | 0.262 |
| 105 | 0.2529 | 105 | 0.2527 |
| 120 | 0.2267 | 120 | 0.227 |
| 135 | 0.1851 | 135 | 0.1848 |
| 150 | 0.1309 | 150 | 0.1315 |
| 165 | 0.0678 | 165 | 0.0661 |
| 180 | 0 | 180 | 0.007 |

<End of extraction>

It can be seen from the extraction above that the implementation based on sinϴ.Dϴ is ignoring the measurement points at the poles (0° and 180°) where sinϴ = 0. For that reason, it is not accurate to use sinϴ.Dϴ weights for spatial averaging. Better results can be obtained with Clenshaw-Curtis weights which do not disregard the points at the poles.

Observation 3: sinϴ.Dϴ weights ignore the measurement points at the poles (0° and 180°) where sinϴ = 0.

Proposal 3: Use Clenshaw-Curtis weights for spatial averaging instead of sinϴ.Dϴ weights.

  - AoA separation(s) for specifying the requirement

Agreement: Down-select to following options: Option 1: UE meets requirement at one AoA offset. FFS if AoA offset is by specification or by UE declaration Option 2: UE meets the requirement at two AoA offsets. One is selected from {30⁰, 60⁰, 90⁰} and another is selected from {120⁰, 150⁰, 180⁰}. FFS if AoA offsets are by UE declaration or by specification.

The simulation results presented in our companion contribution R4-2307232 shows that the SIR performance of the UE varies depending on the selected AoA offset. Considering the practical TRP (transmission and reception point) deployment scenarios and the fact that the incoming signal can reach UE from any direction on the sphere (based on the LoS/reflected path), Option 1 is not a preferred candidate. However, recreating an equivalent test set up for testing multi-Rx RF feature in an anechoic chamber is difficult. Furthermore, testing the UE for all possible AoA offsets can lead to increased test time. Therefore, we support Option 2 as a trade-off between test time and test accuracy.

Proposal 4: RAN4 to define separate RF requirements for different AoA offsets (one requirement for each AoA offset). During the test, the UE will choose test AoA offsets from {30⁰, 60⁰, 90⁰} and {120⁰, 150⁰, 180⁰} respectively.

# Conclusions

In this contribution, we make the following observations and conclusions:

Observation 1: Spherical coverage measurement result can be different based on the different UE orientation.

Observation 2: For a fixed AoA separation and UE orientation, the test results corresponding to +AoA offset and -AoA offset will be independent.

Observation 3: sinϴ.Dϴ weights ignore the measurement points at the poles (0° and 180°) where sinϴ = 0.

Proposal 1: Reference UE orientation(s) has to be defined for the test.

Proposal 2: The test outcome corresponding to +AoA offset and -AoA offset should be treated independently to generate the result.

Proposal 3: Use Clenshaw-Curtis weights for spatial averaging instead of sinϴ.Dϴ weights.

Proposal 4: RAN4 to define separate RF requirements for different AoA offsets (one requirement for each AoA offset). During the test, the UE will choose test AoA offsets from {30⁰, 60⁰, 90⁰} and {120⁰, 150⁰, 180⁰} respectively.

# References

[1]	RP-223527, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated.

[2]	R4-2306604 “WF on FR2-1 UERF requirements for FR2 multi-Rx”, Vivo, Apple