3GPP TSG-RAN WG4 Meeting # 107                                                                 R4-2307242

Incheon, 22 - 26 May, 2023

Title: 	MU associated with hand phantom

Source: 	Huawei, HiSilicon

Agenda item:	8.17.4

Document for:	agreement

# Introduction

In RAN4 #106bis, WF [1] suggests further study the feasibility of tests with hand phantom.

Issue 1-3-1: Feasibility of FR1 MIMO OTA test with hand phantom

Proposals:

- Proposal 1: Adjust positioning guidelines to ensure that the DUT and hand phantom can be encompassed in the 20 cm QZ in DMP.

- Proposal 2: Further discuss the MU assessment and CM impact with hand phantom based on the conclusion of the QZ size issue.

<Agreement>:

- FFS the feasibility and the proposals.

This contribution supplements the MU assessment in [2] by adding MU due to positioning uncertainty.

# Discussion

The random uncertainty term due to monoblock, clamshell and PDA used for testing in entry 11 of Table A.4.2-2 of [3] was not included in the MU table in [2]. Although the proposed MIMO OTA tests with hand phantom would likely use hand phantom for wide grip, the uncertainty values should be applicable to hand phantoms for wide grip. This contribution adds this term in the MU table below.

Proposal: review and endorse the MU table in R4-2307242 for MIMO OTA tests with hand phantoms.

| UID | Description of uncertainty contribution | Example value (410MHz<f≤3GHz) | Example value (3GHz <f≤7.125GHz) | Distribution of the probability | Std Uncertainty (410MHz<f≤3GHz) [dB] | Std Uncertainty (3GHz <f≤7.125GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 0 | 0 | U-Shaped | 0 | 0 |
| 2 | Measure distance uncertainty | 0 | 0 | Normal | 0 | 0 |
| 3 | Quality of quiet zone | 0.6 | 0.6 | Actual | 0.6 | 0.6 |
| 4 | Base Station simulator | 1.5dB | 2dB | Rectangular | 0.87 | 1.15 |
| 5 | Channel Emulator - absolute output power - output signal stability - output stability with temperature | 1.5dB 0.5dB 0.4dB | 1.5dB 0.5dB 0.4dB | Actual (normal- power; rect-stability) | 0.84 | 0.84 |
| 6 | Amplifier uncertainties | 0.7dB | 0.7dB | Rectangular | 0.4 | 0.4 |
| 7 | Random uncertainty | 0.2dB | 0.2dB | Normal | 0.12 | 0.12 |
| 8 | Throughput measurement: output level step resolution | 0.25dB | 0.25dB | Rectangular | 0.14 | 0.14 |
| 9 | DUT sensitivity drift | 0.2 | 0.2 | Rectangular | 0.12 | 0.12 |
| 10 | Signal flatness | 0 | 0 | Normal | 0 | 0 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.2 | 0.2 | U-Shaped | 0.14 | 0.14 |
| 12 | Reference antenna positioning misalignment | 0 | 0 | Normal | 0 | 0 |
| 13 | Quality of quiet zone | 0.6 | 0.6 | [Rectangular] | 0.35 | 0.35 |
| 14 | Total uncertainty of the Network Analyzer | 0.5 | 0.5 | Rectangular | 0.29 | 0.29 |
| 15 | Uncertainty of an absolute gain of the calibration antenna | 1 | 1 | Normal | 0.5 | 0.5 |
| 16 | Offset of the Phase Center of the Reference Antenna | 0 | 0 | Normal | 0 | 0 |
| 17 | Uncertainty related to the use of phantoms | 0.32 | 0.32 | Rectangular | 0.18 | 0.18 |
| 18 | Random uncertainty due to Monoblock, clamshell and PDA used for testing | 0.91 | 0.91 | Rectangular | 0.53 | 0.53 |

# Conclusions

This contribution makes the following proposal.

Proposal: review and endorse the MU table in R4-2307242 for MIMO OTA tests with hand phantoms.

# References

- R4-2305910 WF for Rel-18 MIMO OTA

- R4-2218071 MU for hand phantom in MIMO OTA tests

- TS 38.161 v17.1 User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements