3GPP TSG-RAN WG4 Meeting # 107                                                                 R4-2307245

Incheon, 22 - 26 May, 2023

Title: 	on TRP TRS tests under TxD, UL MIMO and CA

Source: 	Huawei, HiSilicon

Agenda item:	8.16.2.1

Document for:	approval

# Introduction

In RAN4 #106bis, WF [1] suggests to further study TxD and UL MIMO. This contribution provides further analyses on TxD, UL MIMO and CA test procedures.

# TxD

TRP captures all the power radiated from a device. Radiation from two antennas under TxD would interact destructively in certain directions while constructively in others. However, the total radiated power from a device remains the same because the total radiated power is equal to conducted power multiplied by total antenna efficiency. The total antenna efficiency, which also includes mismatch and ohmic loss, can be impacted by mutual coupling between the two antennas and the mutual coupling could vary with phase difference between the two antennas. However, those effects are there by design either intentionally or unintentionally. In other words, there is no need to account for such effect in TRP test procedures.

To illustrate this point further, Table 1 shows TRP measurement from three phones with phase differences 0°, 180°, 90° and -90°, which correspond to TPMI 2, 3, 4, and 5, respectively. The standard deviations of TRP values are between 0.2 and 0.5 dB. Variations of similar magnitude in EIRP is also observed in [3] by measuring EIRP at a few fixed grid points over 10 minutes.

The TRP differences between various TPMI values could be reduced by using finer grid because beams formed by two antennas is narrower than that from a single antenna.

Table 1 TRP measurement for TPMI 2-5 in R4-2305701

Observation 1: TRP is independent of destructive or constructive interaction between radiated energy from the two antennas in TxD.

TRP by definition is an instantaneous measurement of EIRP over a spherical surface. However, lab measurement for TRP takes several minutes because it uses a single probe and rotates device to cover such spherical surface. Phase difference between the two antennas can vary over the measurement period. This is the main source of uncertainty in TRP measurement under TxD. Obviously other variations such as transmit power also exist, but their effects on TRP are insignificant.

Observation 2: The main source of uncertainty in TRP measurement under TxD is the phase difference variation over the measurement period.

In fact, such phase drift is experienced by users under network conditions when connection durations last a few minutes.

Proposal 1: TRP measurement under TxD can be made using existing test procedure with variation due to phase drift over the measurement period accounted for in MU.

The value of such MU can be determined by measuring EIRP variation as proposed in [3] or TRP or others.

# UL MIMO

The essence of testing UL MIMO is to account for all four TPMI values. Out of the four options in RAN4 #106bis WF [1], only option 2, 3 and 4 consider TPMI 2-5. Option 3 and option 3a are essentially the same in terms of measurement time because mechanical movement consumes large part of measurement time and test equipment manufacturers confirmed in RAN4 #106bis that EIRP measurement can be taken for all four TPMI at any grid point.

Observation 3: Option 3 and option 3a are essentially the same in terms of measurement time.

Option 2 and 4 use the maximum of EIRP from TPMI 2-5 and are too optimistic from the field performance point of view. Because the four precoding vectors only provide limited angular resolution compared to UE distribution over 360 degrees. For example, network would select a precoding vector such that the main energy lobe (or beam) is directed towards the base station under line of sight or the strongest multipath clusters from the base station under non-line of sight. This is not the same as take the maximum of precoding vectors at any given point in space because it is unlikely that the network would rely on the sidelobes from a resulting precoding vector.

Observation 4: Option 2 and 4 give optimistic assessment while attempt to reflect the beam characteristics from precoding vectors.

The intention of the TRP metric is to reflect the UE ability to provide isotropic coverage, while UL MIMO with TPMI 2-5 is to provide gain in specific directions. In other words, TRP does not reflect the characteristics of UL MIMO with precoding vector TPMI 2-5.

Observation 5: TRP is not a suitable metric for UL MIMO with TPMI 2-5.

If new metric such as antenna gain or directivity is out of scope in this work item, it would save time not to discuss how to process measured EIRP.

Proposal 2: measure and list UL MIMO TRP(TPMI i=2,3,4,5) separately without further processing them.

# CA test procedure

The editor’s note in clause 6.2A.1.1 in TS 38.521-1 [4] states the following.

No test points are defined for intra-band contiguous UL CA and intra-band non-contiguous UL CA since there is no configuration satisfying MPR=0dB requirements in RAN4. Testing with lowest value of MPR will be covered in 6.2A.2.

Namely non-zero MPR or power reduction would be applied to some RBs. However, such reduction would reflect the field conditions.

Proposal 3: use CA configurations in TS 38.521-1 regardless of MPR=0 or not as non-zero MPR reflects field conditions.

CA configurations in TS 38.521-1 [4] also include dual Tx, possibly with two power amplifiers. The antennas under dual Tx in CA transmit at different frequencies, e.g. in inter-band CA, one antenna covers one band while the other antenna radiates in another band. In other words, the two antennas are independent and do not interfere with each other.

Proposal 4: include dual Tx configurations in CA tests

There are many NR CA combinations. At the initial stage, it is prudent to limit the scope of CA tests to two UL and two DL CA combinations.

Proposal 5: limit the scope of CA tests to two UL and two DL CA combinations.

Appendix in this contribution provides a draft test procedure for CA TRP and TRS in TR 38.870.

Proposal 6: Review and endorse the CA test procedure in Appendix of R4-2307245.

# Conclusions

This contribution makes the following proposals and observations.

Observation 1: TRP is independent of destructive or constructive interaction between radiated energy from the two antennas in TxD.

Observation 2: The main source of uncertainty in TRP measurement under TxD is the phase difference variation over the measurement period.

Proposal 1: TRP measurement under TxD can be made using existing test procedure with variation due to phase drift over the measurement period accounted for in MU.

Observation 3: Option 3 and option 3a are essentially the same in terms of measurement time.

Observation 4: Option 2 and 4 give optimistic assessment while attempt to reflect the beam characteristics from precoding vectors.

Observation 5: TRP is not a suitable metric for UL MIMO with TPMI 2-5.

Proposal 2: measure and list UL MIMO TRP(TPMI i=2,3,4,5) separately without further processing them.

Proposal 3: use CA configurations in TS 38.521-1 regardless of MPR=0 or not as non-zero MPR reflects field conditions.

Proposal 4: include dual Tx configurations in CA tests

Proposal 5: limit the scope of CA tests to two UL and two DL CA combinations.

Proposal 6: Review and endorse the CA test procedure in Appendix of R4-2307245.

# References

- R4-2305904 WF for Rel-18 TRP/TRS requirements

- R4-2305701 Further discussion on FR1 1 Layer UL-MIMO 2Tx SISO OTA TRP

- R4-2304028 on TRP TRS test aspects

- TS38.521-1 v17.8 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

- TS 38.508-1 v17.8 User Equipment (UE) conformance specification

Appendix: Text proposal on CA test procedure in TR 38.870

# x	CA test methodology

## x.1	General

For UE radiated conformance testing P-MPRc shall be 0 dB.

FR1 TRP and TRS radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

The NR SS should send continuous uplink power control “up” commands to the DUT to ensure the DUT’s transmitter is at maximum output power during the CA TRP and TRS test.

## x.2	Total Radiated Power (TRP)

### x.2.1	Test Conditions

The sampling step for CA TRP measurement is the same as SA mode, defined in Clause 8.2.1.

CA for both single transmit antenna and two transmit antenna devices can be tested. Because two transmit antennas would transmit at separate frequencies with no interference between them.

### x.2.2	UE configurations

For devices supporting transmit antenna switching, the guidelines specified in Clause 7.4.4 of TR 38.870 shall be used. However, devices supporting dual Tx in CA mode can be tested as normal, namely no special modification is needed.

For CA, the SS and DUT shall be configured per TS 38.521-1 [4], Section 6.2A.1 (UE Maximum Output Power for CA) using the default settings specified in TS 38.521-1 [4] and TS 38.508-1 [5] as applicable. The measurement should be carried out based on the detailed test parameters for each band, as defined in Table 6.1A-1a in TS 38.521-1 [4] for DFT-s-OFDM modulation and inner full RB allocation.

### x.2.3	Test procedure

For UE configured with CA, the measurement procedure includes the following steps:

1)  Place the DUT inside the QZ following the positioning guideline defined in Clause 6 [of TR 38.870].

2)  Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2A.1.1.4.2 of TS 38.521-1 [4] and ensure the DUT transmits with its maximum power.

3)  Measure the power at each measurement point, and calculate EIRP(θ,ϕ) by adding the composite loss of the entire transmission path.

The TRP value is calculated using the TRP integration approaches outlined in Clause 5.1.1 [of TR 38.870].

## x.3	Total Radiated Sensitivity (TRS)

### x.3.1	Test Conditions

The definition of CA TRS measurement and the sampling step is identical to that for SA mode, defined in Clause 8.3.1.

### x.3.2	UE configurations

The CA SS and DUT shall be configured per section 7.3A.1 (Reference sensitivity power level for 2DL CA without exception) of TS 38.521-1 [4], using the defaults specified in TS 38.521-1 [4] and TS 38.508-1 [5], as applicable. The measurement should be carried out based on the detailed test parameters for each band, as defined in Table 6.1A-1b in TS 38.521-1 [4] for DFT-s-OFDM modulation and outer full RB allocation.

### x.3.3	Test procedure

For UE configured with CA mode, the measurement procedure includes the following steps:

1)  Place the DUT inside the QZ following the positioning guideline defined in Clause 6 [of TR 38.870].

2)  Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 7.3A.1.4.2 of TS 38.521-1 [4] and ensure the DUT transmits with its maximum power.

3)  Follow steps 1 through 4 in section 7.3A.1.4.2 of TS 38.521-1 [4], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR sensitivity level.

The TRS value is calculated using the equation outlined in Clause 5.2.2 [of TR 38.870].