3GPP TSG-RAN WG4 Meeting #107	R4-2307248

Meeting, May 22nd – May 26th, 2023

Source:	Kumu Networks, INC.

Title:	TP to TR 38.858 section 10.2 Feasibility of FR1 Wide Area BS aspects

Agenda Item:	8.20.2.2.1

Document for: 	Approval

# 1	Introduction

During RAN4#106-bis, it agreed that each company can provide a table for Self-interference analysis, Co-channel inter-sub-band co-site inter-sector interference analysis, co-channel inter-sub-band co-site inter-sector interference analysis and company specific information on the respective feasibility studies.

It was also agreed that each company should submit text for each of the sections, and then that the editor would merge the texts.

In this TP, we provide some text for some sub-section within section 10.2 to cover feasibility of FR1 wide area BS aspects.

# 2	Text Proposal

## 10.2 Feasibility of FR1 Wide Area BS aspects

### 10.2.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

10.2.1.1          Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

| FR1 (or FR2-1) |  |  | Kumu Networks, Inc. |  |
| --- | --- | --- | --- | --- |
| BS class |  |  | Wide Area BS (Mean) | - |
| BS TX Power dBPTX = ① dBm |  |  | 53 |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dB = ② dBc | 45 |  |
|  |  | Frequency isolation techniques used | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |  |
|  | Spatial isolation | Spatial isolation capability dB = ③ dBc | 65 |  |
|  |  | Spatial isolation techniques used | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation |  |
|  | TX Beam nulling /isolation in TX sub-band dB = ④ dBc |  | 16.98 |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. (Note 1) dB |  | -57 dBm |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dB = ⑤ dBc | 27.95 |  |
|  |  | RF IC capability and other tech. in RX sub-band dB = ⑧ dBc | 30.66 |  |
|  |  | RF IC techniques and other tech. (before LNA) | Analog RF IC joint tune with beam-nulling RF SIC Complexity : 192 RF taps for 32 Tx and 32 Rx antennas |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA | 0.2 |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA (Note 1) ①-③-④-⑤ dB |  | -56.93 |  |
|  | Frequency isolation at RX | Frequency isolation capability dB= ⑥ dBc | 0 |  |
|  |  | [Frequency isolation] techniques | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |  |
|  | Rx IMD | Rx IIP3 capability (dBm) | -20 |  |
|  |  | Rx IM3 contribution (dBm) | -130.79 |  |
|  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | 5 |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized (Note 1, 2) dB |  | -130.79 |  |
|  | RX Beam nulling /isolation in RX sub-band dB = ⑨ dBc |  | 16.08 |  |
|  | RX sensitivity degradation caused by RX beam nulling |  | negligible |  |
|  | Digital IC dB = ⑦ dBc |  | 0 |  |
| Overall RSIC capability dBRSIC (Note 1) ②+③+⑧+⑨ |  |  | 156.64 |  |
| Noise floor ⑩dBm |  |  | -89 dBm/100MHz |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  | -95 |  |
| Required RSIC budget (①-⑪dBc) |  |  | 148 |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

10.2.1.2          Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

10.2.1.2.1          Kumu Networks, Inc

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.2.1.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

Simulation Parameters:

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Carrier Frequency | 4 GHz |
|  | Duplex Type | SBFD |
|  | SBFD pattern | 40:20:40 SBFD (DUD) |
|  | Channel bandwidth | 40:20:40 MHz (DUD) for SBFD |
|  | Guard Band | 0 MHz |
|  | Available resource blocks | 108:52:108 (DUD) for SBFD |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 5 (4 DL and 1 UL) |
|  | Channel model | gNB-UE: 3GPP_38.901_Uma_LOS |
| BS | Array Configuration | 32x32 |
|  | Max gNB Tx Power | 53 dBm (same antenna gain) |
|  | (dv, dh) | (0.8λ, 0.5λ) |
|  | Antenna element gain | 7 dBi |
|  | Beamforming method | Frequency domain (every 4RB) |
|  | Antenna Isolation | 65 dB |
|  | Noise figure | 5 dB |
|  | Max modulation | 256 QAM |
|  | BS height | 10 m |
|  | Panel HW assumptions | Same antenna gain, same antenna area |
|  | UE antenna | 1TX 1RX |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
| UE | SNR target | 16 dB |
|  | Noise figure | 9 dB |
|  | Max modulation | 64 QAM |
|  | UE distribution | Random cluster |
|  | Traffic split | 40:20:40 DL:UL:DL |
| RF cancellation complexity | Number of RF taps required | 192 (for 32 TX + 32 RX antennas) |

Benefit of RF analog cancellation:

RF Analog cancellation allows the use of high dynamic range passive RF circuits to enhance the isolation between the transmit and receive arrays. These circuits sample the signal after the PA and inject it before the LNA. This addresses two issues that can limit cancellation performance:

- The signal going through the RF tap captures PA non-linearities and noise. Thus, this eliminates the need for complicated non-linear modeling to use these taps for canceling the inter-subband leakage of the transmitter on the receiver sub-band.

- The cancellation signal is injected before the LNA thus avoiding LNA saturation due to the strong signal present in the transmit subband.

More details on this RF cancellation architecture can be found in [1].

Trade-offs of using RF analog cancellation:

- Insertion loss: RF analog cancellation typically requires couplers placed at the output of PAs and input of LNAs. The coupling factor and hence the insertion loss for these couplers depends on the overall loss that can be tolerated in the cancellation path while maintaining enough power to cancel the leakage from the TX to RX antenna. Typical over-the-air isolation in the FR1 antenna arrays between TX and RX is expected to be >65dB. Thus, using 20dB couplers on TX and RX leaves 25dB additional loss margin for the cancellation circuit. Insertion loss with this configuration is minimal.

Complexity analysis of RF analog cancellation:

Previous approaches to using RF analog cancellation have used 1 RF cancellation path per TX/RX pair, thus leading to an impractical n^2 complexity in the number of RF cancellation paths required. To reduce this complexity, our simulations use a combination of RF cancellation and beamnulling. By jointly optimizing beamnulling and RF cancellation, the number of RF cancellation taps needed is linearly proportional to the total number of antennas on the gNB. The RF cancellation works well for different antenna configurations and scales well to handle multipath channels. Specifically, our simulation results with multipath channels and multiple DL UEs needs 192 RF taps for 64 antennas (32 TX and 32 RX), so a complexity of 3n, where n is total number of antennas. More details on this optimization can be found in [1].

Chip Area and Power analysis for RF analog cancellation:

The area impact of RF cancellation can be analyzed based on an existing RF cancellation chip, eg. [2]. This chip integrates 4 RF taps (phase shifters and attenuators with better than 10-bit resolution each). This chip’s area is ~17.5sqmm. Adding 50% for layout + keep out, each chip requires 22sqmm space on a board. A 192-tap board (for 32 TX / 32 RX and 3x over provisioning of taps for frequency selectivity) using these chips would need 48 chips with ~1000sqmm board area (10sq cms). Since these taps are passive, the power consumption is negligible (100s of micro-Amperes with a 3.3V supply).

References

- 3GPP R1-2205815, Kumu Networks, “RF cancellation techniques for Subband non-overlapping full duplex”, RAN1 #110, Aug 2022

- https://kumunetworks.com/wp-content/uploads/2022/06/KU10408-IC-Product-Brief-v2.pdf

10.2.2        Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results.

10.2.2.1          Summary table for co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the summary table which is based on co-channel inter-sub-band co-site inter-sector interference analysis framework.

| FR1 (or FR2-1) |  |  | Kumu Networks, Inc. |  |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  | Wide Area BS (Mean) |  | - |
| BS TX Power dBPTX = ① dBm |  |  | 53 dBm |  |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dB = ② dBc | 45 dB |  |  |
|  |  | Frequency isolation techniques used | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |  |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dB = ③ dBc | 85 dB |  |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation |  |  |
|  | TX Beam nulling /isolation in TX sub-band dB = ④ dBc |  | 19.67 dB |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |
|  | Inter-sector interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. (Note 1) dB |  | -57 dBm |  |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dB = ⑤ dBc | 0 dB |  |  |
|  |  | RF IC capability and other tech. in RX sub-band dB = ⑧ dBc | 0 dB |  |  |
|  |  | RF IC techniques and other tech. (before LNA) | None used for inter-sector interference cancellation |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA | 0.2 dB (assuming 15 dB Rx coupler) |  |  |
|  | Inter-sector Interference signal in gNB TX subband, measured at the input of LNA (Note 1) ①-③-④-⑤ dB |  | <-51.65 dBm |  |  |
|  | Frequency isolation at RX | Frequency isolation capability dB= ⑥ dBc | 0 dB |  |  |
|  |  | [Frequency isolation] techniques | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |  |  |
|  | Rx IMD | Rx IIP3 capability (dBm) | -20 |  |  |
|  |  | Rx IM3 contribution (dBm) | -115.0 |  |  |
|  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | 5 dB (noise figure) |  |  |
|  | Inter-sector Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized (Note 1, 2) dB |  | -115.0 dBm |  |  |
|  | RX Beam nulling /isolation in RX sub-band dB = ⑨ dBc |  | 16.47 dB |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  | negligible |  |  |
|  | Digital IC dB = ⑦ dBc |  | 0 dB |  |  |
| Overall Inter-sector IC capability dB (Note 1) ②+③+⑧+⑨ |  |  | 146.47 dB |  |  |
| Noise floor ⑩dBm |  |  | -89 dBm/100MHz |  |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  | -95 dBm |  |  |
| Required Inter-sector IC budget (①-⑪dBc) |  |  | 148 dB |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

10.2.2.2          Feasibility study on co-channel inter-sub-band co-site inter-sector interference

Editor's note: This section captures the feasibility study on co-channel inter-sub-band co-site inter-sector interference based on individual companies’ analysis.

10.2.2.2.1          Kumu Networks, Inc.

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.2.2.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.