3GPP TSG-RAN WG4 Meeting #107 					                 R4-2307257

Incheon, KR, 22 - 26 May, 2023

Source:	China Telecom

Title:	Further discussion on RedCap PC2

Agenda Item:	11.1

Document for:	Discussion

- Introduction

In RAN #99, a new WID on NR power class 2 RedCap UE in FR1 was submitted in [1], and the following agreements were reached in RAN #99:

- RAN task RAN4 the following work in the next quarter and come back to RAN#100

  - To study how to enable HPUE including PC2 for RedCap in the band agnostic way

  - To study the applicable form factor, e.g., sensor, camera

In RAN4 #106e-bis, RAN4 discussed the issues tasked by RAN, with an WF approved in [2].

This contribution further discusses the two issues as tasked by RAN.

- Discussion

How to enable HPUE including PC2 for RedCap in the band agnostic way

In the last meeting, it was agreed that:

- PC2 RedCap HPUE requirements can be specified in the band agnostic way at least for TDD and HD-FDD.

  - The band agnostic way means the PC2 requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.

- Further discuss whether the band agnostic way can be applied for PC2 FD-FDD bands with 1Rx and 2Rx, based on more analysis on the RF requirements including A-MPR and RSD.

- Note 1: 1Tx architecture is assumed for RedCap PC2.

- Note 2: PC2 for Rel-17 Redcap is considered under this RAN task.

Firstly, we agree with the idea that only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE. With this, for TDD and HD-FDD, the RF requirements for PC2 non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way, as we agreed in the last meeting.

For FD-FDD, we need to mainly look at the A-MPR and RSD. For non-Redcap PC2 UE, the A-MPR is specified considering that non-Redcap PC2 UE can be implemented with 1 or 2 Tx chains. For RedCap, 1Tx architecture is assumed based on the UE implementation. So, RAN4 need to check whether the A-MPR for non-Redcap PC2 UE can be directly re-used for Redcap PC2 UE.

Proposal 1: For FD-FDD, considering that non-Redcap PC2 UE is implemented with 1 or 2 Tx chains and Redcap PC2 UE is implemented with 1 chain, RAN4 to check whether the A-MPR for non-Redcap PC2 UE can be directly re-used for Redcap PC2 UE on the same band.

Regarding the RSD for FD-FDD RedCap PC2, it can be discussed for 2Rx and 1Rx RedCap UE respectively. For 2Rx RedCap UE, RAN4 to check whether the RSD for non-RedCap PC2 UE can be re-used for RedCap PC2 UE. For 1Rx RedCap UE, RAN4 to discuss whether the single Rx antenna port RSD allowance for RedCap PC3 UE can be re-used for RedCap PC2 UE.

Proposal 2: For FD-FDD, RAN4 to discuss the RSD for 2Rx and 1Rx RedCap UE respectively. More specifically,

- Check whether the RSD for non-RedCap 2Rx PC2 UE can be re-used for RedCap PC2 2Rx UE.

- Discuss whether the single Rx antenna port RSD allowance for RedCap PC3 UE can be re-used for RedCap PC2 UE.

Applicable form factor, e.g., sensor, camera

In the last meeting, RAN4 has not reached consensus on whether PC2 RedCap can be applied for the wearable, and whether to restrict the applicable form factor for PC2 RedCap HPUE in the specification:

Candidate options for further discussion:

- Option 1: Not restrict the applicable form factor for PC2 RedCap HPUE in the specification

- Option 2: Clearly add the restriction on the applicable form factor for PC2 Redcap in the specification

To move forward, we suggest first confirming the feasibility of PC2 RedCap for sensor and camera.

For the support of PC2 RedCap for wearable, the main concern from the UE side is the power consumption and overheating issues [3]. Meanwhile, it was mentioned in [4] that with the PC2 support for wearable RedCap UE, the uplink coverage performance is better, which results in less number of re-transmissions and less power consumption. So, more discussion on the power consumption aspect is needed for wearable RedCap UE.

Proposal 3: RAN4 to first confirm the feasibility of PC2 RedCap for sensor and camera.

Proposal 4: Further discuss whether the UE power consumption and overheating issues would prevent wearable UE to support PC2 RedCap.

- Conclusions

In this contribution, we further discussed the two issues for RedCap PC2 as tasked by RAN, with the following proposals:

Proposal 1: For FD-FDD, considering that non-Redcap PC2 UE is implemented with 1 or 2 Tx chains and Redcap PC2 UE is implemented with 1 chain, RAN4 to check whether the A-MPR for non-Redcap PC2 UE can be directly re-used for Redcap PC2 UE on the same band.

Proposal 2: For FD-FDD, RAN4 to discuss the RSD for 2Rx and 1Rx RedCap UE respectively. More specifically,

- Check whether the RSD for non-RedCap 2Rx PC2 UE can be re-used for RedCap PC2 2Rx UE.

- Discuss whether the single Rx antenna port RSD allowance for RedCap PC3 UE can be re-used for RedCap PC2 UE.

Proposal 3: RAN4 to first confirm the feasibility of PC2 RedCap for sensor and camera.

Proposal 4: Further discuss whether the UE power consumption and overheating issues would prevent wearable UE to support PC2 RedCap.

- References

RP-230312, New WID: NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, MediaTek, RAN #99, March 2023.

R4-2306650, WF on RedCap HPUE, China Telecom, RAN4 #106-bis-e, April 2023.

R4-2305109, Discussions on RedCap HPUE, vivo, RAN4 #106-bis-e, April 2023.

R4-2305843, Redcap HPUE support in FR1, Skyworks Solutions, RAN4 #106-bis-e, April 2023.