3GPP TSG-RAN WG4 Meeting # 107	R4-2307261

# Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3139 | rev |  | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to RRM idle mode HST test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The testing principle in defining HST idle mode RRM test cases consists in a two-cell scenario in which the UE moves away from the source cell and towards the target cell (thus simulating an adequate mobility scenario). The agreed Doppler offset for the target cell is, after normalization, 2*Fd = 1944Hz for 15kHz SCS and 2*Fd = 3334Hz for 30kHz SCS. Because of this normalization, there is no offset to be applied to the source cell (as 2*offset was applied to the target cell). Therefore, the Doppler offsets for tests 6.1.2.5 and 8.2.1.2 is corrected as follows: 6.1.2.5 – The UE moves away from the NR cell and towards the LTE cell. The Doppler offsets are corrected as follows: - LTE target cell: 1944Hz - NR source cell: 0Hz 8.2.1.2 – The UE moves away from the LTE cell and towards the NR cell. The Doppler offsets are corrected as follows: - LTE target cell: 0Hz - NR source cell: 1944Hz for 15kHz SCS; 3334Hz for 30kHz SCS |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the Doppler offset and corresponding notes for the affected test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.1.2.5, A.8.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes 1>>

#### A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16

##### A.6.1.2.5.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements for UE configured with highSpeedMeasFlag-r16 specified in clause 4.2.2.5 when the E-UTRAN cell is of lower priority.

##### A.6.1.2.5.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.5.2-1, A.6.1.2.5.2-2, A.6.1.2.5.2-3 and A.6.1.2.5.2-4. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1. The E-UTRAN cell 2 is indicated by NR cell 1 as an HST cell.

Table A.6.1.2.5.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.5.2-2: General test parameters for NR to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase. |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.32 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.5.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 4 | CR.1.1 FDD |  |
|  |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 4 | CCR.1.1 FDD |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC pattern 2 |  |
|  |  | 2, 5 | SMTC pattern 1 |  |
|  |  | 3, 6 | SMTC pattern 1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 4 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -51.69 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Threshx, highP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, highP which is included in NR system information, and is a threshold for the E-UTRA target cell. Note 3: Void. |  |  |  |  |

Table A.6.1.2.5.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP (Note 2) | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP | dB | 50 |  |
| Propagation Condition |  | AWGN 1944Hz Note3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, highP which is included in E-UTRA system information, and is a threshold for the NR target cell Note 3: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. |  |  |  |

<< Start of Change 2 >>

#### A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16

##### A.8.2.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the E-UTRAN to NR inter-RAT cell reselection requirements specified in clause 4.2.2.5.6 in 36.133 [15].

The test scenario comprises of 1 E-UTRA cell and 1 NR cell as given in tables A.8.2.1.2.1-1, A.8.2.1.2.1-2, A.8.2.1.2.1-3 and A.8.2.1.2.1-4. In SIB of the E-UTRA cell, highSpeedInterRAT-NR-r16 is configured and the carrier of NR cell is configured with highSpeedCarrierNR-r16. The test consists of two time periods, with time duration of T1 and T2 respectively. Both E-UTRA cell 1 and NR cell 2 are already identified by the UE prior to the start of the test. NR cell 2 is of lower priority than E-UTRA cell 1.

Table A.8.2.1.2.1-1: Supported test configurations for UE configured with highSpeedInterRAT-NR-r16

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.2.1.2.1-2: General test parameters in E-UTRA cell re-selection FR1 NR cell test case for UE configured with highSpeedInterRAT-NR-r16

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3, 4, 5, 6 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2, 5 | 3 s | Synchronous cells |
|  |  |  | 3, 6 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.32 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.8.2.1.2.1-3: Cell specific test parameters for NR cell 2 in E-UTRA cell re-selection FR1 NR cell test case for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.1.1 FDD |  |
|  |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.1.1 FDD |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
| Pcompensation | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Qhysts | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Qoffsets, n | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4, 5, 6 | SS-RSRP |  |
|  | dB | 1, 4 | 14 | 14 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | 14 | 14 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -84 | -84 |
|  |  | 2, 5 | -84 | -84 |
|  |  | 3, 6 | -81 | -81 |
| Io | dBm/9.36 MHz | 1, 4 | -55.88 | -55.88 |
|  | dBm/9.36 MHz | 2, 5 | -55.88 | -55.88 |
|  | dBm/38.16 MHz | 3, 6 | -47.79 | -47.79 |
| Treselection | s | 1, 2, 3, 4, 5, 6 | 0 |  |
| Snonintrasearch | dB | 1, 2, 3, 4, 5, 6 | Not sent |  |
| Threshx, high | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, low | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, low | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 4, 5 | AWGN 1944Hz Note 4 |  |
|  |  | 3,6 | AWGN 3334Hz Note 5 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |

Table A.8.2.1.2.1-4: Cell specific test parameters for E-UTRA cell 1 in E-UTRA cell re-selection FR1 NR cell test case for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
| Note 2 | dBm/15 kHz | -98 |  |
| RSRP Note 3 | dBm/15 KHz | -102 | -86 |
|  | dB | -4 | 12 |
|  | dB | -4 | 12 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | 50 |  |
| Threshx, high | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void. |  |  |  |

##### A.8.2.1.2.2	Test Requirements

The cell reselection delay to a lower priority NR cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 3 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR + TSI-NR,

Where:

Tevaluate, NR	See Table 4.2.2.5.6-2 in clause 4.2.2.5.6 in [15]

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 2.24 s, allow 3 s for the cell re-selection delay to a lower priority NR cell.