3GPP TSG-RAN WG4 Meeting # 107	R4-2307263

# Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.33.2

Source:	Qualcomm Inc.

Title:	On NR SL relay

Document for:	Discussion

# Introduction

In this contribution, we present our view SL relay RRM requirements.

# Discussion

Observation: Upon checking the R17 selection/reselection requirements, there is no restrictions on the relay scenarios or number of hops.

### 12.10.1	Introduction

This section contains the requirements related to selection and reselection of relay UE.

The requirements apply for the selection and reselection of candidate relay UEs that are transmitting relay discovery signals within the resource pool as configured for the remote UE.

### 12.10.2	Selection / Reselection of relay UE

For a remote UE configured by upper layer for relay operation, the remote UE shall search for candidate relay UEs for selection and/or reselection every discovery period which is determined by resource reservation period or SPS transmission periodicity configured by network.

If the remote UE has a selected sidelink relay UE, then the remote UE shall measure the SD-RSRP or SL-RSRP of the selected relay once in every four discovery periods and evaluate if it meets the relay selection criterion as defined in TS 38.331[2] (clause 5.8.15.3).

The remote UE shall measure SD-RSRP or SL-RSRP of the candidate relay UEs every Tmeasure, SL_Relay_Intra for relay UEs that are detected and measured according to the measurement rules.

For intra-frequency relay UEs that are detected, but that has not been selected or reselected to, the remote UE shall be capable of evaluating that the intra-frequency relay UE has met selection or reselection criterion defined in TS 38.331[2] (clause 5.8.15.3) within Tevaluate, SL_Relay_Intra as specified in table 12.10.2-1.

Proposal: The legacy requirement on selection/reselection of relay UE is applicable to R18 scenarios and no specification change is needed.

# Conclusions

Observation: Upon checking the R17 selection/reselection requirements, there is no restrictions on the relay scenarios or number of hops.

Proposal: The legacy requirement on selection/reselection of relay UE is applicable to R18 scenarios and no specification change is needed.