3GPP TSG-RAN WG4 Meeting # 107		R4-2307271

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	5.2.8.4

Source: 		Qualcomm Incorporated

Title: 			Ambiguity on UL transmissions in scheduling restriction and MG

Document for:	Approval

# 1.	Introduction

We bring up an ambiguity issue on UL transmission availability in slots where scheduling restriction is applied, or measurement gap is activated.

# 2. 	Discussion

There are multiple cases where UE is not expected to conduct Rx/Tx operations except for some specific Rx/Tx operations. For instance, UE is allowed to discard Rx/Tx operations in a slot where the UE is expected to perform measurement from inter-satellite cells within a measurement gap. If the UE does not support parallelMeasurementWithoutRestriction, a scheduling restriction will be applied due to SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO. And the Rx/Tx exception rules are defined in the same manner as those in TN, like RAN4 convention. However, in contrast to TN, the UL Tx exception rules should have been more carefully defined due to a large round trip delay, otherwise UE and NW may be out of sync because the actual UL transmission timing of a particular slot may collide with a slot where the UL scheduling restriction is not applied. In order to resolve the ambiguity issue, one of ways can be to leverage UE TA-report or UE-specific Koffset. As the door to some changes to RAN2 spec (particularly a new signaling) is almost firmly closed, we would like to hear opinions from other companies on whether/how to address the UL scheduling restriction ambiguity issue in RAN4 spec.

Observation 1: In contrast to TN, in NTN, there is a high chance that UE and NW can be out of sync in terms of slot indices where UL scheduling restriction is applied because the actual UE UL transmission timing can be advanced way more than an integer multiple of slot length, which cannot be always fully known to NW.

Proposal 1: RAN4 to discuss whether/how to address the ambiguity issue on UL scheduling restriction.

# 3.	Conclusion

In this paper, we proposed the following.

Observation 1: In contrast to TN, in NTN, there is a high chance that UE and NW can be out of sync in terms of slot indices where UL scheduling restriction is applied because the actual UE UL transmission timing can be advanced way more than an integer multiple of slot length, which cannot be always fully known to NW.

Proposal 1: RAN4 to discuss whether/how to address the ambiguity issue on UL scheduling restriction.