3GPP TSG-RAN WG4 Meeting #107                               R4-2307289

Incheon, South Korea, May 22 – 26, 2023

Source: 	 SK Telecom, Murata Manufacturing Co. Ltd

Title: 		 TP for TR 37.718-11-21 to include DC_1-3-7_n40-n77

Agenda Item: 	 7.6.2

Document for:  Approval

- Introduction

A text proposal for TR 37.718-11-21 to include DC_1-3-7_n40-n77.

- Text Proposal

※ Please note that all the relevant fallback combinations are included in the following documents.

- TS 38.101-3 V18.1.0 (2023-03)

- TR 37.718-11-21 V0.5.0 (2023-04)

- R4-2306490 endorsed in the last RAN4#106bis-e meeting

--- Start of Changes ---

8.x	DC_1-3-7_n40-n77

8.x.1	Operating bands for DC

Table 8.x.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n40-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 8.x.1-2: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |

8.x.2	Channel bandwidths per operating band for DC

Table 8.x.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7A_n40A-n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-7A_n40A-n77(2A) |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.x.3	UE co-existence studies

Co-existence studies of DC_1-3-7_n40-n77 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

8.x.4	∆TIB and ∆RIB values

Referring to DC_1-3-7_n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 8.x.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-7_n40-n77 | 0.6 | 0.6 | 0.8 | 0.9 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.x.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-7_n40-n77 | - | - | 0.3 | 0.8 | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.x.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

--- End of Changes ---

- Reference

[1] 3GPP TS 38.101-1 V18.0.0 (2022-12), User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18)

[2] 3GPP TS 38.101-3 V18.0.0 (2022-12), User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 18)

[3] 3GPP TS 36.101 V18.0.0 (2022-12), User Equipment (UE) radio transmission and reception (Release 18)

[4] 3GPP TR 37.718-11-21 V0.3.0 (2022-11), Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) (Release 18)