3GPP TSG-RAN WG4 Meeting #107 	R4-2307318

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on above 10GHz NTN bands

Agenda item:	8.27.1.1

Document for:	Discussion and Approval

Introduction

The NTN WI, as presented in [1], includes the following objectives related to NTN deployment in above 10GHz bands:

4.1.2 NR-NTN deployment in above 10 GHz bands The following assumptions are taken a baseline for this work: - GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered - ESIM scenarios for NGSO in Ka band are not considered in this WI. - Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified - Regarding mobile VSAT, three types of terminal and scenario exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study. - FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2 - The ITU-R harmonized Ka band will serve as reference - Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification. The following covers the objectives for NR-NTN deployment in above 10 GHz bands. This work is expected to start after June 2022. - Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4] - Consider the satellite harmonized Ka band as a reference, according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations, define an example band suitable for development of generic 3GPP minimum performance requirements (the example RAN4 band may be a portion of or the entire harmonized Ka band). [RAN4] - Study implications of FDD operation in FR2 and derive requirements for the identified example band appropriately. Satellite bands introduced in 3GPP for NTN for FDD shall not impact the existing 3GPP TDD specifications for terrestrial bands adjacent to the NTN band (see note 3 of the approved way forward RP-211596 in RAN#92-e). [RAN4] - Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow. [RAN4] - For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands [RAN4]. - Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region; (see proposal 2 of the approved way forward RP-211596 in RAN#92-e) [RAN4] - Specify Rx/Tx requirements for satellite access node and different VSAT UE class (not only 60 cm aperture) as appropriate for the identified example band [RAN4] - Identify values for physical layer parameters chosen from the existing FR1 and FR2 sets. The following set of parameters to specify, but not necessarily limited to, are listed.as follows [RAN4]: - time relationship related enhancement (e.g., K_offset) - subcarrier spacing for different UL/DL signals/channels - PRACH configuration index for FDD above 10 GHz.

In this contribution we present further considerations for the introduction of NTN deployment in above 10 GHz bands.

Discussion

During RAN4 #106, a Way-Forward agreed for system parameters were agreed [2]. This discussion takes as starting point the listed issues:

- Band definition(s)

- Frequency range relation

- SCS

- Power classes (Deployment Scenarios)

- Protection and Additional protection requirements

- NTN UE satellite beam tracking

Band definition

At RAN4#106 it was agreed to work on the following bands:

Table 2.1-1: NTN satellite bands in FR2-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n5121 | 27500 MHz – 30000 MHz | 17300 MHz – 20200 MHz | FDD |
| n5112 | 28350 MHz – 30000 MHz | 17300 MHz – 20200 MHz | FDD |
| n5103 | 27500 MHz – 28350 MHz | 17300 MHz – 20200 MHz | FDD |
| NOTE: FR2-NTN satellite bands are numbered in descending order from n512 NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). |  |  |  |

The three bands presented in Table 2.1-1 are all dependent on regional and/or country specific regulatory requirements as provided in the corresponding notes. The bands are covering the so-called harmonized Ka-band according to the provided regulatory requirements. As a result, it can be deducted that current coverage of the bands presented in Table 2.1-1 are limited to the specified regions/countries.

All the currently agreed new NTN bands above 10 GHz correspond to regulations only defined for a specific region/country.

As discussed in previous meeting it is expected that further NTN bands above 10 GHz will be added at a later stage of the specification development. It shall be noted that for additional bands to be added a new work-item or modifications to an existing have to be agreed by RAN. The prerequisite for any new band to be treated by RAN4 is that regulations for the intended region/country of deployment are available and presented to RAN4.

The prerequisite for any new band to be treated by RAN4 is that regulations for the intended region/country of deployment are available and presented to RAN4.

It can be noted that request for new bands to be defined by RAN4 normally is operator driven. Where operator in this relation can be both of terrestrial and non-terrestrial networks.

Frequency range relation

At RAN4#106 multiple suggestions for how to capture the frequency range relation for NTN bands above 10 GHz were presented. The main issue with progress on this is that there are clear agreements as captured in the WID that:

“Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region”.

From this it is not possible to modify existing NR FR1 nor FR2 to cover the now proposed NTN bands.

As a result, an agreement was reached [3] at RAN4#106bis to capture the frequency ranges ranges in TS 38.101-5 and TS 38.108 Table 5.1-1 is show below:

Proposed Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN1 | 410 MHz – 7125 MHz |
| FR2-NTN2 | 17300 MHz – 30000 MHz |
| NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.] |  |

From the Proposed Table 5.1-1 it can be noted that the two NTN frequency ranges with notes are indicated to belong to the two existing frequency ranges from the perspective of other specifications. By this is meant that RAN1 and RAN2 normally only consider a band as belonging to one of these two groups. Therefore, as a starting point any NTN band defined in the ranges is mapped to this division.

From the proposed Table 5-1-1 it is noted that the wording of Note 1 and Note 2 may need further discussion.

Sub-Carrier Spacing (SCS)

During RAN4 #106, the Way-Forward agreed for system parameters [2] states that further progress on the SCS definition for NTN operation above 10 GHz shall wait progress on the feasibility evaluation from the RRM point of view. As a result, Nokia have provided further discussion on the SCS in [4].

At RAN4#106bis the Way-Forward agreed for system parameters [3] now includes 60kHz and 120kHz SCS, but is is also stated that lower SCSs are not precluded.

In FR2-NTN the intention of RAN4 is to support at least 60kHz and 120kHz SCS.

At RAN4#106bis the agreements captured in the Way-Forward agreed for system parameters [3] resulted in a LS to be send to RAN1 [5] to inform them about the progress. Since the content in the LS may imply work to be done by RAN1 (and potentially also RAN2) and there currently are no TU allocated for RAN1 it can’t be expected that a LS response can fully cover any needed assessment by RAN1 or RAN1 even finds time for drafting any response.

RAN4 shall consider if the agreements on system parameters for FR2-NTN have implication on the RAN1 design and TUs is needed requested at RAN to enable RAN1 asses this.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: All the currently agreed new NTN bands above 10 GHz correspond to regulations only defined for a specific region/country.

Observation 2: The prerequisite for any new band to be treated by RAN4 is that regulations for the intended region/country of deployment are available and presented to RAN4.

Observation 3: From the proposed Table 5-1-1 it is noted that the wording of Note 1 and Note 2 may need further discussion.

Observation 4: In FR2-NTN the intention of RAN4 is to support at least 60kHz and 120kHz SCS.

Proposal 1: RAN4 shall consider if the agreements on system parameters for FR2-NTN have implication on the RAN1 design and TUs is needed requested at RAN to enable RAN1 asses this.

- References

- RP-223534, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2302998, WF for system parameters of above 10 GHz NTN band, Thales (Moderator)

- R4-2305925, WF for system parameters of above 10 GHz NTN band, Thales (Moderator)

- R4-2304501, Discussion on applicability of SCS for NTN above 10 GHz, Nokia

- R4-2305926, LS on the system parameters for NTN above 10 GHz, CATT