3GPP TSG-RAN WG4 Meeting # 107		R4-2307328

# Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.35.4

Source:	Apple

Title:	On RRM requirements for NES

Document for:	Discussion

# Introduction

In last RAN4#106-bis-e meeting, the WF for NES has been agreed in [1]. There are still some opens issues for further discussion, and in this contribution, we continue the discussion on the RRM impact for NES feature.

# Discussion

In our view, RAN4 shall start work on SSB-less SCell operation first since the other enhancements still needs more conclusions from RAN2 and RAN1.

## SSB-less SCell operation for inter-band CA for FR1 and co-located cells

Issue 2-1-1: Which scenario to be considered for SSB-Less SCell operation for FR1 inter-band collocated CA

The agreement and remaining issues from last meeting are as followings:

Online session (Friday April 21, 2023) - Agreements - The following scenarios are considered for inter-band SSB-less SCell requirements definition - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell. - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

From UE perspective, inter-band CA may need two separate RF chains, even though the target SSB-less SCell can reuse the coarse timing of another inter-band active serving cell, the fine timing tracking still need to be performed based on its own TRS for PDCCH/PDSCH reception performance. Moreover, we also need to take into account the frequency domain separation between these inter-band CCs, e.g., for CCs with big separation we think fine T/F tracking cannot be directly reused between inter-band CCs. TRS transmission configuration shall be provided to UE when add such SCell via RRC and therefore such TRS transmission shall be assumed during and after the SCell activation.

Proposal 1: RAN4 to prioritize scenario 1 for SSB-less SCell operation for FR1 inter-band collocated CA; and RAN4 to consider requirements based on TRS transmission during and after SCell activation.

Issue 2-1-2: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-1?

The agreement and remaining issues from last meeting are as followings:

Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-1 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

In order to support SSB-less activation for FR1 inter-band case, we need to make sure the RTD of existing serving cell and target SSB-less SCell is within a certain threshold, i.e., ±260ns, and power difference is also within a certain threshold, i.e., 6dB. In existing SSB-less SCell activation for FR1 intra-band contiguous case, it was specified that,

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

To reuse the coarse timing from an inter-band FR1 serving cell, the RTD between the SSB-less SCell the SSB-less SCell and the FR1 inter-band active serving cell shall be at least smaller than CP length. Since the CP length in 30kHz SCS case is 2.34us, the 3us cannot be used for the condition for all SCS cases in FR1.

The TCI condition is to reflect the channel conditions through different CCs, and same condition from intra-band SSB-less SCell activation can be reused for scenario 1. Regarding the frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC, even though the enhanced scenario of large FD separation can be FFS in RF session, we think RRM work can be based on adjacent inter-bands as starting point. Otherwise, many of the RRM conditions for SSB-less Scell operation will need to wait the conclusion from RF session.

Proposal 2: to support the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, the side condition of RTD and reception power difference shall still be defined for the RAN4 requirement:

- RTD between the SSB-less SCell and the FR1 inter-band active serving cell is within X ns, X is ±260ns or CP length.

- The difference of the reception power with the FR1 inter-band active serving cell is <= Y dB, Y is 6dB.

- the TRS(s) of the target SSB-less SCell is (are) QCL-TypeC with SSB(s) of any FR1 inter-band active serving cell.

Proposal 3: RRM requirement discussion of SSB-less SCell operation for FR1 inter-band collocated CA can assume adjacent inter-band scenario as starting point.

Issue 2-3-2: Requirements for FR1 SSB-less SCell activation, if feasibility is confirmed

The agreement and remaining issues from last meeting are as followings:

- Candidate Options: - Option 1(Huawei, CTC, ZTE): For FR1 co-located SSB-less inter-band CA, when the frequency and time synchronization information of active serving cell on FR1 inter-band is able to be applied for to-be-activated SCell, and the power difference is not larger than [TBD]dB, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms). - Option 2 (Vivo): If an SSB-less SCell is configured with CSI-RS based L3 measurement, and UE has reported one MR within a predefined period, the SSB-less SCell is known - The activation delay of a known SSB-less SCell is TfirstTRS + 5ms or TfirstA-TRS + 5ms. Otherwise, the SSB-less SCell is unknown - The activation delay of an unknow SSB-less SCell is TfirstTRS + TTRS + 5ms or TFirstATRS + Tgap + TATRS + 5ms

We think it’s feasible to use A-TRS for inter-band FR1 SSB-less SCell activation scenario 1 (as in proposal 1), however this A-TRS shall use the coarse timing and coarse AGC from another inter-band FR1 active serving cell. In our view, using A-TRS for inter-band FR1 unknown SCell activation enhancement shall follow the similar requirement as for FR1 intra-band unknown SCell fast activation, i.e., TFirstATRS + Tgap + TATRS + 5ms, which means UE still needs to do fine time tracking and fine AGC settling on the unknown SCell by using A-TRS, since UE may still use different RF chains for inter-band CCs. We support option 2 in the last meeting candidates:

Proposal 4: use fast SCell Activation Delay Requirement for FR1(section 8.3.16 of TS38.133) as the baseline to specify the requirement for FR1 SSB-less SCell activation, e.g., A-TRS is used for fine timing tracking and fine AGC settling.

Issue 2-4-1: Scenario for L1 measurement operation on the SSB-less SCell apart from SCell activation procedure

The agreement and remaining issues from last meeting are as followings:

- Candidate Options: - Option 1: No SSB but with CSI-RS resource for L1 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L1 measurement on the inter-band SSB-less SCell

The L1 measurement has different kinds of types: e.g., L1-RSRP, L1-SINR/BFD-evaluation/CQI and so on. L1-RSRP is dynamic/instant measurement for beam tracking, the instantaneous channel fading coefficients is different for different CCs on inter-band, which is related with the frequency coherence distance. We don’t think the L1-RSRP measurement results from another inter-band serving cell can be directly used for beam management on target SCell (probably digital Tx beam in FR1). For L1-SINR/BFD/CQI, the interference level is varied from RB to RB, even in the same band we have whole band CQI and subband CQI, and therefore we cannot agree to reuse L1-SINR/BFD/CQI from one inter-band serving cell to the target SSB-less SCell. Thus, CSI-RS is essentially needed for such L1 measurement on the FR1 SSB-less SCell.

Proposal 5: RAN4 to assume no SSB but with CSI-RS resource for L1 measurement on the FR1 inter-band SSB-less SCell.

Issue 2-4-2: Scenario for L3 measurement operation on the SSB-less SCell apart from SCell activation procedure

The agreement and remaining issues from last meeting are as followings:

- Candidate Options: - Option 1: No SSB but with CSI-RS resource for L3 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L3 measurement on the inter-band SSB-less SCell

Regarding L3 measurement, the measurement results is based on both L3 and L1 filtering and not reflect the instantaneous channel fading coefficients, and therefore, as long as conditions in issue 2-1-2 can be met, UE can reuse L3 measurement results from one inter-band serving cell to the target SSB-less SCell. Thus, CSI-RS for L3 measurement is not that essential on the inter-band SSB-less SCell. To save the energy of network, we can compromise to option 2.

Proposal 6: RAN4 to assume either no SSB and no CSI-RS resource for L3 measurement on the inter-band SSB-less SCell.

## Others for SSB-less SCell

For the T/F information, UE can always assume co-located deployment for intra-band CA before R18, and QCLed typeC provide the delay spread information between existing serving cell and target intra-band SCell; however, this QCLed typeC cannot be used to determine the absolute RTD between existing serving cell and target inter-band SCell. The QCLed typeC definition is in TS38.214, as duplicated below,

The quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values: - 'typeA': {Doppler shift, Doppler spread, average delay, delay spread} - 'typeB': {Doppler shift, Doppler spread} - 'typeC': {Doppler shift, average delay} - 'typeD': {Spatial Rx parameter}

One approach to use QCLed typeC to support FR1 inter-band SCell activation enhancement is: expand the definition of typeC to cover the RTD as well, i.e., if TRS(s) of the SCell being activated is (are) QCL-TypeC with SSB(s) of any FR1 inter-band active serving cell, it means the RTD between the target SCell and the inter-band active serving cell is within within ±260ns. For example, the typeC can be updated as following:

- 'typeC': {Doppler shift, average delay, absolute arrival timing for R18}

However, it may cause backward compatibility issue since legacy typeC definition has been used from R15, and RAN4 needs to check with RAN1 if it’s feasible to expand the typeC definition to support this FR1 SCell activation enhancement.

Another alternative is to introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as timing source. But it will need new signaling in RAN2, and RAN4 may need to check with RAN2 on this solution.

For T/F information acquisition for FR1 SSB-less SCell, RAN4 to consider the SSB-less SCell operation for inter-band CA for FR1 and co-located cells with two possible alternatives:

- Alt 1: expand the definition of QCLed-typeC to indicate the RTD between the SCell being activated and the inter-band active serving cell is within a small range, e.g., ±260ns. RAN4 needs to check with RAN1 for this solution.

- Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as timing source for the SCell being activated. RAN4 needs to check with RAN2 for this solution.

Beside the T/F acquisition from other inter-band serving cell for target SSB-less SCell, the AGC settling is also an important aspect to consider. By using the AGC info from another inter-band serving cell measurement, there also might be two alternatives in our view:

- Alt 1: if T/F information is reused from an inter-band FR1 serving cell, the AGC info of that inter-band FR1 serving cell can also be used for the target SSB-less SCell.

- Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as AGC source for the SCell being activated.

Moreover, no matter which alternative is used to support the SSB-less SCell operation, the side condition of RTD and reception power difference shall still be defined for the RAN4 requirement.

Proposal 7: regarding T/F information acquisition for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, RAN4 to consider one of following alternatives:

- Alt 1: expand the definition of QCLed-typeC to indicate the RTD between the SSB-less SCell and the inter-band active serving cell is within a small range, e.g., ±260ns. RAN4 needs to check with RAN1 for this solution.

- Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as timing source for the SSB-less SCell. RAN4 needs to check with RAN2 for this solution.

Proposal 8: regarding AGC settling for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, RAN4 to consider one of following alternatives:

- Alt 1: if T/F information is reused from an inter-band FR1 serving cell, the AGC info of same inter-band FR1 serving cell can also be used for the target SSB-less SCell.

- Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as AGC source for the SSB-less SCell.

The SCell without SSB means the target SCell needs to get timing/frequency/AGC information from another inter-band FR1 serving cell. However, as we discussed in other L3/L1 enhancement issue in R18 eFeRRM, the inter-band serving cell and target SCell will have inter-band MRTD up to 3us (contributed by TAE) even though they are collocated, which might be still larger than CP. Moreover, we don’t think the QCL type C can directly lead UE to reuse the timing from this inter-band serving cell, if we follow the current definition of QCL type C in RAN1 spec. In current RAN2 spec TS38.306, the capability of SSB-less SCell operation for intra-band CA case was defined as following,

In order to support FR1 inter-band SSB-less SCell, a new capability like scellWithoutSSB shall be introduced to cover the R18 FR1 inter-band SSB-less SCell operation. Furthermore, the L1/L3 measurement on SSB-less SCell shall be FFS to check whether UE can use the CSI-RS for the measurement on the SSB-less SCell or other alternative.

Proposal 9: If FR1 inter-band SSB-less SCell operation is introduced in R18 NES, a new capability like existing scellWithoutSSB shall be introduced to cover the R18 FR1 inter-band CA case.

## Cell DTX/DRX

There were some RAN1 and RAN2 agreements on the cell DTX and DRX for NES WI, as duplicated below,

RAN1 R1-2304014 Discussion summary #2 of issues for enhancements on cell DTX/DRX mechanism Moderator (Intel) Agreement From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting) - FFS: - PDCCH in USS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation - PDCCH in Type-3 CSS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation - PRS - CSI-RS configured by measObjectNR (for RRM) - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD) - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking) - Periodic/Semi-persistent CSI-RS (for BM) - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: Whether the list of impacted signals/channels can be configurable - FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

In WID, it was stated that no change for SSB transmission due to cell DTX/DRX. However, it’s obvious that the transmission of P/SP CSI-RS and PRS might be impacted due to the DTX of the cell. CSI-RS can be used for L3 mobility based measurement of neighbor cell, while PRS is used for positioning measurement (e.g. RSTD) of neighbor positioning cells. The existing CSI-RS measurement for mobility was specified in RAN4 TS38.133 as followings:

Only the CSI-RS periodicity and DRX cycle(configured from serving cell) is considered in the requirement design so far, and therefore if DTX is used in the target neighbor cell, the DTX cycle shall also be considered in the CSI-RS measurement delay requirement. Regarding the UE behavior, in CSI-RS MO, there was no any information about DTX for a specific neighbor cell or specific CSI-RS resource for the measurement. In order to make UE perform the correct measurement on the target CSI-RS of neighbor cell, we think it’s necessary to provide such DTX information of neighbor cell to the UE. Or alternatively, we may not use CSI-RS based L3 measurement when the NES is used for target cell, and that means, network may configure UE to use SSB based L3 measurement when the NES is used. However, since the conclusions from RAN1/RAN2 is not sufficient yet, we think those RRM impact can be FFS.

Similarly, for PRS based positioning measurement, UE has no information of the DTX in the target positioning cell, and it may impact the PRS measurement performance if UE has no idea which PRS occasion is within DTX inactive window. Since LMF is the entity to configure positioning measurement for UE, it would be reasonable to let LMF provide such DTX information to UE via LPP when positioning assistance data is sent. Or alternatively, LMF may provide a new muting pattern for PRS after considering the DTX of positioning cells. No matter which way is adopted, the positioning measurement requirement will be revisited after considering such DTX scenarios, and it can be FFS until we have sufficient conclusions from RAN1/RAN2.

For TRS, it is required that CONNECTED UE will always receive a TRS configuration as described in 38.213, “A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.” What's more, in Rel-17 WI of UE power saving enhancement, IDLE/INACTIVE UEs can also be configured to use TRS. And the WID objective has a NOTE that impact to IDLE/INACTIVE UEs should be avoided. In RAN2 agreement, it was stated that:

Cell barring and camping for legacy and NES UEs:

- RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible

To not impact legacy UEs that do not support NES feature, it is proposed that TRS is still maintained during non-active duration of cell DTX.

Proposal 10: to indicate RAN1 that:

- Based on network configuration, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX:

  - PRS

  - CSI-RS configured by measObjectNR (for RRM)

  - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD)

  - Periodic/Semi-persistent CSI-RS (for BM)

  - FFS: PDCCH in USS to be decided by RAN1/2

  - FFS: PDCCH in Type-3 CSS to be decided by RAN1/2

- To not impact legacy UEs that do not support NES feature, TRS is still maintained during non-active duration of cell DTX.

# Conclusion

In this contribution, we discuss the RRM impact for NES feature.

Proposal 1: RAN4 to prioritize scenario 1 for SSB-less SCell operation for FR1 inter-band collocated CA; and RAN4 to consider requirements based on TRS transmission during and after SCell activation.

Proposal 2: to support the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, the side condition of RTD and reception power difference shall still be defined for the RAN4 requirement:

- RTD between the SSB-less SCell and the FR1 inter-band active serving cell is within X ns, X is ±260ns or CP length.

- The difference of the reception power with the FR1 inter-band active serving cell is <= Y dB, Y is 6dB.

- the TRS(s) of the target SSB-less SCell is (are) QCL-TypeC with SSB(s) of any FR1 inter-band active serving cell.

Proposal 3: RRM requirement discussion of SSB-less SCell operation for FR1 inter-band collocated CA can assume adjacent inter-band scenario as starting point.

Proposal 4: use fast SCell Activation Delay Requirement for FR1(section 8.3.16 of TS38.133) as the baseline to specify the requirement for FR1 SSB-less SCell activation, e.g., A-TRS is used for fine timing tracking and fine AGC settling.

Proposal 5: RAN4 to assume no SSB but with CSI-RS resource for L1 measurement on the FR1 inter-band SSB-less SCell.

Proposal 6: RAN4 to assume either no SSB and no CSI-RS resource for L3 measurement on the inter-band SSB-less SCell.

Proposal 7: regarding T/F information acquisition for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, RAN4 to consider one of following alternatives:

- Alt 1: expand the definition of QCLed-typeC to indicate the RTD between the SSB-less SCell and the inter-band active serving cell is within a small range, e.g., ±260ns. RAN4 needs to check with RAN1 for this solution.

- Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as timing source for the SSB-less SCell. RAN4 needs to check with RAN2 for this solution.

Proposal 8: regarding AGC settling for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, RAN4 to consider one of following alternatives:

- Alt 1: if T/F information is reused from an inter-band FR1 serving cell, the AGC info of same inter-band FR1 serving cell can also be used for the target SSB-less SCell.

- Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as AGC source for the SSB-less SCell.

Proposal 9: If FR1 inter-band SSB-less SCell operation is introduced in R18 NES, a new capability like existing scellWithoutSSB shall be introduced to cover the R18 FR1 inter-band CA case.

Proposal 10: to indicate RAN1 that:

- Based on network configuration, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX:

  - PRS

  - CSI-RS configured by measObjectNR (for RRM)

  - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD)

  - Periodic/Semi-persistent CSI-RS (for BM)

  - FFS: PDCCH in USS to be decided by RAN1/2

  - FFS: PDCCH in Type-3 CSS to be decided by RAN1/2

- To not impact legacy UEs that do not support NES feature, TRS is still maintained during non-active duration of cell DTX.

# References

[1] R4-2306369, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN4 #106-bis-e