3GPP TSG-RAN WG4 Meeting # 107		R4-2307329

# Incheon, Korea, May 22 – May 26, 2023

Agenda item:	5.2.8.4

Source:	Apple

Title:	On remaining issues for R17 NTN

Document for:	Discussion

# Introduction

In RAN4 #106 meeting, the WF[1] for NTN RRM was agreed, but there are still some open issues not fully addressed as followings,

## Issue 1-6: Parallel measurement capability. - FFS: - It shall be clarified that which feature between 25-2 and 25-5 defines parallel L3 measurements on serving cell and intra-frequency cells. - In case that UE doesn’t support parallel L3 measurements on serving cell and intra-frequency cells, for time-critical neighbor cell measurement requirements, we suggest to skip serving cell measurements since cell change is inevitable in very short time and the update doesn’t impact existing Ttrigger definition. ## Issue 1-7: The definition of overlapping SMTCs for cell reselection. - FFS: - RAN4 to discuss and refine the definition of overlapping SMTCs for cell reselection, considering: - NTN UEs are capable to adjust dynamically the configured offset for different satellites based on the propagation delay difference. - Two different SMTCs configuration, with same periodicity, configured for different satellites, might see some occasions that overlap while others that do not overlap.

In this contribution, we discuss the above open issues from RAN4#106 meeting.

# Discussion

Issue 1-6: Parallel measurement capability.

In the R17 feature list, the FG25-2 and FG25-5 was defined as followings:

| 25. NR_NTN_solutions | 25-2 | Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell | Support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release (Rel-17). |
| --- | --- | --- | --- |
| 25. NR_NTN_solutions | 25-5 | Parallel measurements on multiple NGSO satellites within a SMTC | Support of simultaneously measurements on target cells belonging to different NGSO satellites within a SMTC |

The FG 25-2 is the capability for L3 measurement + L1 operation or data/control transmission/reception, however, the FG25-5 is the capability for parallel L3 measurement among different target cells including serving cell and neighbor cells. Thus, FG 25-5 defines parallel L3 measurements on serving cell and intra-frequency cells.

Proposal 1: FG25-5 defines parallel L3 measurements on serving cell and intra-frequency cells.

Regarding the UE behavior when UE doesn’t support parallel L3 measurements on serving cell and intra-frequency cells, for time-critical neighbor cell measurement requirements, our understanding is as long as the Ttrigger definition is not changed/enhanced, it’s up to UE implementation to organize the measurement order of different cells, e.g., serving cell or neighbor cells. The current Ttrigger is defined as following:

If ‘t-Service’ is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area regardless of whether the distance condition based on serving cell reference location is met or the legacy Srxlev/Squal condition are met, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is acquired by the UE for the first time to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger. Ttrigger = max(Tdetect,NR_Intra, Kcarrier* Tdetect,NR_Inter), where - Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell, - Tdetect,NR_Intra refers to intra-frequency cell detection delay in IDLE/INACTIVE mode defined Table Table 4.2C.2.3-2, - Tdetect,NR_Inter refers to inter-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2C.2.4-2. The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in Table 4.2C.2.3-1 and Table 4.2C.2.3-2 is expected.

RAN4 defined in TS38.133 that when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation for service time based reselection. Thus, when UE doesn’t support parallel L3 measurements on serving cell and intra-frequency cells, for time-critical neighbor cell measurement requirements, it can be up to UE implementation on when to start the detection, measurement and evaluation on neighbour cells as long as UE can meet the reselection requirement with Ttrigger.

Proposal 2: when UE doesn’t support parallel L3 measurements on serving cell and intra-frequency cells, for time-critical neighbor cell measurement requirements, it can be up to UE implementation on when to start the detection, measurement and evaluation on neighbour cells as long as UE can meet the reselection requirement with Ttrigger.

Issue 1-7: The definition of overlapping SMTCs for cell reselection

As discussed in RAN4#106 meeting, since UE will shift the SMTC in IDLE/inactive mode according to the satellite moving to make sure the satellite can be measured in the SMTC window, and the overlapping of SMTCs may happens after such SMTC shifting, as shown below,

Figure 1. SMTC collision after time shift

#### 9.1C.9.3 Collision between multiple SMTCs on a SAN carrier Collision between occasions of multiple SMTCs may occur as specified in this clause. When UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms. The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC ocasssion.

We think the overlapped SMTC in IDLE and Inactive shall take into account the shifting due to the satellite moving. The existing SMTC collision is defined in section 9.1C.9.3 as following,

However, such SMTC occasions in the collision definition shall also consider the SMTC occasions after shifting in IDLE/Inactive mode. In cell reselection requirement, we have one clarification sentence for requirement applicability,

The requirements in this clause apply provided that SSB of neighbour cells are within the time shifted SMTC.

Since R17 core is closed, we prefer to not introduce new requirement for such special case but we can further clarify on top of existing requirement applicability. We propose to expand this applicability definition to cover the SMTC collision after shifting, i.e., The requirements in this clause apply provided that SSB of neighbour cells are within the time shifted SMTC, and this SMTC after time shifting is not collided with other SMTC occasion(s).

Proposal 3: expand existing applicability definition in IDLE/Inactive requirement to cover the SMTC collision after shifting, i.e., the requirements in IDLE/Inactive apply provided that SSB of neighbour cells are within the time shifted SMTC, and this SMTC after time shifting is not collided with other SMTC occasion(s).

In addition, we identified another issue related with such time shifted SMTC, that is, it’s unclear to decide which direction on time domain shall SMTC window shift if one SMTC window is associated with multiple satellites. As illustrated in the following figures,

Figure 2. Illustration for unclear SMTC shifting directions

In the figure 2, if two satellites are on same carrier and one single SMTC configuration is configured on that carrier, but the Sat1 and Sat2 are moving on opposite directions, it’s unclear which direction shall SMTC window shifting follows. In TS38.331, the SMTC configuration (smtc4list-r17) is independent from the cell list in the SI (e.g., SIB2, SIB3 and SIB4), then it’s ambiguous to UE which satellite shall be associated with which SMTC, and therefore, the SMTC shifting will be problematic as described in figure 2. In our view, it would be desirable if the association between satellite and SMTC configuration can be provided to UE to facilitate the dynamic SMTC shifting based on satellite moving.

Observation: it would be desirable if the association between satellites and SMTC configuration can be provided to UE to facilitate the dynamic SMTC shifting based on satellite moving.

However, we understand the core part of this WI is closed and this clarification will also need RAN2 involvement, and therefore we are open to leave it to UE implementation or send LS to RAN2 to ask for their clarification.

Proposal 4:

RAN4 to discuss the issue that: in IDLE/Inactive mode, when multiple satellites are deployed with same SMTC configuration, which satellite(s) shall UE follow to adjust the SMTC.

- Alt1: up to UE implementation and no requirement shall be defined to associate satellite with SMTC adjustment.

- Alt2: send LS to RAN2 to ask for their clarification.

# Conclusion

In this contribution, we discuss the above open issues from RAN4#106 meeting.

Proposal 1: FG25-5 defines parallel L3 measurements on serving cell and intra-frequency cells.

Proposal 2: when UE doesn’t support parallel L3 measurements on serving cell and intra-frequency cells, for time-critical neighbor cell measurement requirements, it can be up to UE implementation on when to start the detection, measurement and evaluation on neighbour cells as long as UE can meet the reselection requirement with Ttrigger.

Proposal 3: expand existing applicability definition in IDLE/Inactive requirement to cover the SMTC collision after shifting, i.e., the requirements in IDLE/Inactive apply provided that SSB of neighbour cells are within the time shifted SMTC, and this SMTC after time shifting is not collided with other SMTC occasion(s).

Observation: it would be desirable if the association between satellites and SMTC configuration can be provided to UE to facilitate the dynamic SMTC shifting based on satellite moving.

Proposal 4:

RAN4 to discuss the issue that: in IDLE/Inactive mode, when multiple satellites are deployed with same SMTC configuration, which satellite(s) shall UE follow to adjust the SMTC.

- Alt1: up to UE implementation and no requirement shall be defined to associate satellite with SMTC adjustment.

- Alt2: send LS to RAN2 to ask for their clarification.

# References

[1] R4-2303147, WF on Rel-17 NR NTN RRM Performance requirements, Xiaomi, RAN4 #106.