3GPP TSG-RAN WG4 Meeting # 107	R4-2307336

# Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.19.1.2

Source:	Apple

Title:	On test parameters and simulation results for MU-MIMO

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#106bis-e PDSCH demodulation requirements for MU-MIMO with advanced receiver were discussed and way forward [1] was agreed.  In this contribution we present our simulation results and views on test parameters for advanced receiver in MU-MIMO.

# 2. Simulation Results

In RAN4#106bis-e simulation parameters were agreed for initial simulation alignment and phase I for advanced receiver for MU-MIMO study.

The simulation assumptions for the results we present are summarized below:

- Ant config and rank for 2 paired UEs:

  - 2x2: 1+1

  - 4x4: 2+2

- DMRS ports:

  - 2x2: 1+1 – [0,1]

  - 4x4: 2+2 – [(0,1),(2,3)]

- Precoder selection for interference UE

  - Orthogonal for 4x4

  - Random and unique for 2x2

- Codebook

  - SP Type I codebook

  - Random PMI

- Channel and antenna correlation

  - Rank 1+1: ULA medium, TDLC300-100

  - Rank 2+2; ULA Low, TDLA30-10

- MCS for target UE

  - Rank 1: 16QAM (MCS13)

  - Rank 2: 16QAM (MCS13), 64QAM (MCS17)

- MCS of co-scheduled UE

  - For rank 1+1: QPSK

  - For rank 2+2: QPSK, 16QAM, 64QAM

2x2

The results below are for 2x2 antenna configuration with 1+1 layer combination:

Figure 1: Simulation results for 2x2

For 2x2 with medium correlation the gain with R-ML is high, as expected. The performance of MMSE-IRC and E-MMSE-IRC is comparable.

Observation #1: The gain with R-ML receiver is high with medium correlation with 2x2.

Observation #2: Performance with E-IRC and baseline MMSE-IRC receiver is comparable with 2x2.

4x4

For 4x4 antenna configuration we present results for 2+2 layer combination with orthogonal precoder for co-scheduled UE.

16QAM - MCS13

Figure 2: Simulation results for 4x4 with TDLA channel

With 2+2 TDL-A channel, low correlation, the gain with R-ML receiver is seen with co-scheduled UE using QPSK. With low correlation, the gain from R-ML reduces when modulation order of co-scheduled UE is equal or larger than target UE. With higher modulation of co-scheduled UE compared to target UE, the performance of R-ML is comparable to E-IRC.

Observation #4: With orthogonal precoders, with TDL-A channel and 2+2 larger gain with R-ML receiver is observed when co-scheduled UE is using QPSK.

Observation #5: When target and co-scheduled UE use modulation order 16QAM and 64QAM respectively, the performance of R-ML is comparable to E-IRC with orthogonal precoders and low correlation.

64QAM - MCS17

Figure 3:  Simulation results for 4x4 with TDLC channel

With higher MCS (64QAM) for target UE, the performance gain with E-IRC is comparable to 16QAM with TDLA channel, low correlation. Larger gain with R-ML is observed when modulation order of co-UE is much smaller than target UE.

Observation #6: With orthogonal precoders, TDL-A channel, low correlation gain of E-IRC over baseline MMSE-IRC is the same with 16QAM and 64QAM on target UE.

Observation #7: Larger gains are observed with R-ML when modulation order of co-UE is much smaller than target UE.

We summarize the simulation results in the table below:

Table 1: Summary of simulation results

| Rank Combo | Modulation of co-sch UE | MIMO | Channel Model | MCS | Precoder of co-sch UE | SNR @ 70% Max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC | E-IRC | R-ML |
| 1+1 | QPSK | 2x2 ULA Med | TDLC300-100 | 13 | Random | 24.3 | 24.2 | 16.4 |
|  | QPSK | 4x4 ULA Low | TDLA30-10 | 13 | Orthogonal | 14.4 | 14.0 | 11.8 |
|  | 16QAM |  |  | 13 | Orthogonal | 14.4 | 14.0 | 13.7 |
| 2+2 | 64QAM |  |  | 13 | Orthogonal | 14.4 | 14.0 | 14.0 |
|  | QPSK |  |  | 17 | Orthogonal | 19.1 | 18.5 | 15.1 |
|  | 16QAM |  |  | 17 | Orthogonal | 19.1 | 18.5 | 17.6 |
|  | 64QAM |  |  | 17 | Orthogonal | 19.1 | 18.5 |  |

# 3. Conclusion

In this paper, we provide present our simulation results and views on test parameters for advanced receiver in MU-MIMO. Our observations and proposals are captured below:

Simulation Results

2x2:

Observation #1: The gain with R-ML receiver is high with medium correlation with 2x2.

Observation #2: Performance with E-IRC and baseline MMSE-IRC receiver is comparable with 2x2.

4x4:

Observation #4: With orthogonal precoders, with TDL-A channel and 2+2 larger gain with R-ML receiver is observed when co-scheduled UE is using QPSK.

Observation #5: When target and co-scheduled UE use modulation order 16QAM and 64QAM respectively, the performance of R-ML is comparable to E-IRC with orthogonal precoders and low correlation.

Observation #6: With orthogonal precoders, TDL-A channel, low correlation gain of E-IRC over baseline MMSE-IRC is the same with 16QAM and 64QAM on target UE.

Observation #7: Larger gains are observed with R-ML when modulation order of co-UE is much smaller than target UE.

# Reference

- R4-2305914, “WF on advanced receiver for MU-MIMO scenario”, China Telecom.