3GPP TSG-RAN WG4 Meeting #107		R4-2307345

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.8.2.2

Source:	Apple

Title:   	RF requirement for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

At RAN4#106bis, a WF on FR2 UE RF requirements for 2AoA DL RX [1] was agreed. In the WF, the requirement concept was agreed. In addition, further agreements on beam/antenna module selection, requirement metric, weighting function for spherical coverage, and further alignment of simulation procedure were made.

In this contribution, we share updated simulation results based on which proposals and observations are provided.

# 2	Discussion

## 2.1 Simulation

2.1.1 Simulation assumptions and UE implementations

Key simulation assumptions are given below.

- Two UE implementations are simulated, as shown in Fig. 1. Implementation 1 has back-to-back panels pointing to opposite directions, Implementation 3 has two panels with one at the top and the other on the side. Each panel consists of 4 dual-polarized antenna elements.

- The UE selects the beam for each AoA based on the criterion of Option 1 in the WF (Option 1: UE assigns ‘first’ module to track TRP that yields highest RSRP among all combinations of modules and TRPs. The best of the other modules is assigned to track the other TRP), respectively. As discussed at the last meeting, when calculating the SINR for one AoA, the signal from the other AoA is treated as interference.

- An AoA pair is considered a qualified one (or Pass) when min(SINR_AoA1, SINR_AoA2) >= -1dB.

- We consider a channel bandwidth of 100MHz in band n257. The corresponding 50%-ile EIS value is -74.4dBm. Unless otherwise indicated, the DL power level is set as -74.4dBm.

- In the simulation, the setting is set according to the WF as shown below:

“In the coordination system of z-axis pointing to AoA1 (P0), the two AoAs (probes) shall be located in xz plane.”

- For each implementation, three UE orientations are simulated, i.e., UE front facing positive X, Y, Z axis.

- As agreed, we take Option 1 as the baseline for calibration, i.e., adjust the beam shape or scale the antenna gain to make UE align with both peak EIS and spherical coverage.

Implementation 1					 Implementation 3

Figure 1. Two UE implementations considered in the simulations.

2.1.2 Simulation results

We provide the results in the following tables.

Table 1. Implementation 1, coverage probability based on OR combining

| AoA offset (degrees) | 30 | 60 | 90 | 120 | 150 | 180 |
| --- | --- | --- | --- | --- | --- | --- |
| Orientation X | 0.0% | 0.0% | 1.0% | 2.6% | 5.7% | 2.9% |
| Orientation Y | 0.0% | 0.0% | 1.0% | 4.6% | 24.1% | 31.8% |
| Orientation Z | 0.0% | 0.0% | 0.1% | 1.9% | 5.5% | 8.0% |

Table 2. Implementation 1, coverage probability based on arithmetic mean combining

| AoA offset (degrees) | 30 | 60 | 90 | 120 | 150 | 180 |
| --- | --- | --- | --- | --- | --- | --- |
| Orientation X | 0.0% | 0.0% | 0.0% | 1.3% | 3.3% | 2.9% |
| Orientation Y | 0.0% | 0.0% | 0.5% | 2.6% | 13.6% | 31.8% |
| Orientation Z | 0.0% | 0.0% | 0.1% | 0.9% | 2.8% | 8.0% |

Table 3. Implementation 3, coverage probability based on OR combining

| AoA offset (degrees) | 30 | 60 | 90 | 120 | 150 | 180 |
| --- | --- | --- | --- | --- | --- | --- |
| Orientation X | 3.0% | 15.0% | 26.4% | 26.5% | 19.8% | 5.9% |
| Orientation Y | 0.0% | 1.6% | 5.2% | 14.7% | 15.5% | 5.5% |
| Orientation Z | 1.0% | 7.0% | 15.0% | 18.2% | 13.2% | 6.1% |

Table 4. Implementation 3, coverage probability based on arithmetic mean combining

| AoA offset (degrees) | 30 | 60 | 90 | 120 | 150 | 180 |
| --- | --- | --- | --- | --- | --- | --- |
| Orientation X | 1.5% | 7.8% | 13.2% | 13.3% | 10.7% | 5.9% |
| Orientation Y | 0.0% | 0.8% | 2.7% | 7.4% | 7.7% | 5.5% |
| Orientation Z | 0.5% | 3.8% | 7.7% | 9.1% | 7.3% | 6.1% |

From Tables 1-4, the following observations can be made:

- UE orientation greatly impacts the coverage probability. This is especially pronounced in Implementation 1.

- For Implementation 1, the coverage probability increases with the AoA offset. For Implementation 3, the coverage probability is the greatest when the AoA offset is 120 degrees. This is in line with the expectation, due to their different panel placement in the UE.

- For both Implementations 1 and 3, it is observed that except for AoA offset of 180 degrees, using arithmetic mean would lead to roughly half of the coverage probability compared to OR combining. For AoA offset of 180 degrees, there is no difference between the two methods since in this case, +AoA offset and -AoA offset would have exactly the same SINR values for each AoA pair (+180 degrees offset is equivalent to -180 degree offset).

- With the DL power level set to -74.4dBm, the two AoA coverage probability is much smaller than 50% in most combinations of AoA offset and UE orientation.

Observation 1: 	As AoA offset increases, not all implementations perform better.

Observation 2: 	Either Implementation 1 or Implementation 3 may perform better depending on different AoA offset.

Observation 3: 	For both Implementations 1 and 3, except for AoA offset of 180 degrees, using arithmetic mean would lead to roughly half of the coverage probability compared to OR combining.

Observation 4: 	With the DL power level set to -74.4dBm, the two AoA coverage probability is much smaller than 50% in most combinations of AoA offset and UE orientation.

Based on the observations, we propose

Proposal 1: 	It is up to UE to declare its orientation and AoA offset in the test system.

Proposal 2: 	It is preferred to use the OR combining in deriving the coverage probability.

## 2.2 Fixed AoA offset

Further agreement on AoA offset was made in the WF [1]:

### AoA separation(s) for specifying the requirement

Agreement:

Down-select to following options:

Option 1: UE meets requirement at one AoA offset. FFS if AoA offset is by specification or by UE declaration

Option 2: UE meets the requirement at two AoA offsets. One is selected from {30⁰, 60⁰, 90⁰} and another is selected from {120⁰, 150⁰, 180⁰}. FFS if AoA offsets are by UE declaration or by specification.

As the results in Tables 1-4 show, different UE implementations perform differently with different AoA offset. To allow UE implementation flexibility, it is necessary to allow UE to declare which fixed AoA offset it supports in meeting the core requirement. We further believe given the various performances with different AoA offset, it is reasonable to require the UE to meet the requirement for one AoA offset only.

Proposal 3: 	It is up to UE to declare one fixed AoA offset it supports in meeting the core requirement.

## 2.3 How to specify the requirement

2.3.1 Options on how to specify the requirement

As proposed, to accommodate different UE implementations, which may require/declare different AoA offsets to meet the requirement, there are three options of specifying the requirement.

Option1: Define separate requirements for different pre-defined AoA offsets. The tested AoA offset is based on UE declaration. The UE is required to pass the requirement for at least one AoA offset. The requirement would look like:

- Requirement 1: ratio of x% for AoA offset of 45 degrees.

- Requirement 2: ratio of y% for AoA offset of 90 degrees.

Option2: Define a single requirement for different pre-defined AoA offsets. The tested AoA offset is based on UE declaration. The UE is required to pass the requirement for at least one AoA offset. The requirement would look like:

- Single requirement: ratio of z% for AoA offset of 45 degrees and 90 degrees.

Option 3: Define a single requirement averaged across multiple pre-defined AoA offsets. No UE declaration is needed. UE is required to be tested for all pre-defined offsets. The requirement would look like:

- Single requirement: ratio of q% averaged between AoA offsets of 45 degrees and 90 degrees.

Given the dependence of two AoA performance on the UE implementation and the AoA offset values, and the performance difference varies significantly between different implementations, we believe Option 1 is the best choice.

Proposal 4: 	RAN4 selects Option 1 to define requirement.

2.3.2 Need for additional margins for requirement

So far in the simulation assumptions, calibration is done to adjust the beam shape or scale the antenna gain to make UE align with both peak EIS and spherical coverage requirement in single AoA case. While this is a good approach, the following factors may require consideration.

- With at least two panels required to support two AoA reception, UE implementation impairments should be re-discussed. They may include physical limitations and constraints, such as thermal noise effects, routing losses, and panel interaction (as both are active at the same time), etc.

- As discussed before, besides the AoA mutual interference, if there is power imbalance between AoA1 and AoA2, its impact on AGC performance of each Rx chain needs to be considered.

- The antenna performance difference between UE’s V/H element need to be considered in requirement design, as captured in the WF [1].

Proposal 5: 	RAN4 further discusses what additional RF impairments/implementation constraints are to be considered in defining the final RF requirement.

# 3	Conclusions

In this contribution, we make the following observations and proposals.

Observation 1: 	As AoA offset increases, not all implementations perform better.

Observation 2: 	Either Implementation 1 or Implementation 3 may perform better depending on different AoA offset.

Observation 3: 	For both Implementations 1 and 3, except for AoA offset of 180 degrees, using arithmetic mean would lead to roughly half of the coverage probability compared to OR combining.

Observation 4: 	With the DL power level set to -74.4dBm, the two AoA coverage probability is much smaller than 50% in most combinations of AoA offset and UE orientation.

Proposal 1: 	It is up to UE to declare its orientation and AoA offset in the test system.

Proposal 2: 	It is preferred to use the OR combining in deriving the coverage probability.

Proposal 3: 	It is up to UE to declare one fixed AoA offset it supports in meeting the core requirement.

Proposal 4: 	RAN4 selects Option 1 to define requirement.

Proposal 5: 	RAN4 further discusses what additional RF impairments/implementation constraints are to be considered in defining the final RF requirement.

# 4	References

R4-2306604, "WF on FR2-1 UERF requirements for FR2 multi-Rx," vivo, Apple