3GPP TSG-RAN WG4 Meeting #107	R4-2307362

Incheon, South Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	SSB-less operation for FR1 inter-band co-located CA

Agenda item:	8.35.4

Document for:	Discussion

Introduction

At RAN4#106bis-e meeting, the work item Network energy savings was firstly discussed and the WF is captured in [1]. In this paper, we will continue the discussion on the SSB-less operation for FR1 inter-band co-located CA.

Scenarios with SSB-less operation

In last meeting, the following scenarios were agreed to be considered for inter-band SSB-less operation, but there were different understandings on the implications in terms of the impact on RRM requirements. We will elaborate our views on these scenarios.

Issue 2-1-1: Which scenario to be considered for SSB-Less SCell operation for FR1 inter-band collocated CA

Online session (Friday April 21, 2023)

- Agreements

  - The following scenarios are considered for inter-band SSB-less SCell requirements definition

    - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell

      - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation

    - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell

    - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell.

    - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

Scenario 1 is originated from the intra-band SSB-less scenario hence is well received during the discussion. In particular, the TRS is assumed being transmitted in the SCell as there is one side condition requiring the QCL relation with the RS of the SCell and the SSB of an active serving cell.

TS 38.133 clause 8.3.2:

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided

-	The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

-	The difference of the reception power with the contiguous active serving cell is <= 6dB, and

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

However, the activation time for intra-band SSB-less operation is defined as 3ms which means the TRS monitoring or measurement is not considered when activating the SCell. UE just applies the PCell information to SCell and performs channel measurement to report a valid CSI. Taking this as baseline, we should study if it is feasible to achieve 3ms activation delay for the inter-band SSB-less collocated scenario. From UE behaviour point of view, it means the TRS monitoring is not needed when activating the SSB-less SCell.

Observation #1: For intra-band SSB-less operation, TRS monitoring is not needed when activating the SSB-less SCell.

Proposal 1: RAN4 to study if it is feasible to achieve 3ms SCell activation delay for inter-band SSB-less operation.

Another interpretation of Scenario 1 was to use TRS to assist the SSB-less SCell activation. It was argued that the UE operating inter-band CA is receiving different carriers via separate Rx chains, which is different from the UE architecture assumed for intra-band operation. In this case, the UE cannot fully rely on PCell information, instead it needs to monitor the reference signals e.g. TRS on the SSB-less SCell at least for fine time tracking. Such TRS monitoring will increase the SCell activation delay so that the activation delay longer than 3ms is expected.

Observation #2: If TRS monitoring is needed for activating the inter-band SSB-less SCell, the SCell activation delay longer than 3ms is expected.

Proposal 2: RAN4 to study the SSB-less SCell operation for scenario 1 considering the following cases:

- Case 1: TRS monitoring is not needed for SSB-less SCell activation (i.e. 3ms SCell activation delay is achieved).

- Case 2: TRS monitoring is needed for SSB-less SCell activation (wherein >3ms SCell activation delay is expected).

Besides, when Scenario 1 is described as “no SSB but with TRS transmission”, it is unclear what type of TRS is referred to. In the existing intra-band SSB-less SCell activation, the TRS used for QCL relation is understood as the periodic TRS. But if TRS is used to assist SCell activation in Case 2 above, it could also be A-TRS as specified for fast SCell activation. RAN4 should clarify what type of TRS is intended for SSB-less SCell operation in above cases.

Observation #3: In Scenario 1, TRS may refer to the periodic TRS used for QCL relation or the aperiodic TRS used for fast SCell activation.

Proposal 3: RAN4 to clarify what type of TRS is intended for SSB-less SCell operation in Scenario 1.

For Scenario 2 “no SSB and no TRS transmission”, neither SSB nor TRS is transmitted on the SSB-less SCell. In our understanding, TRS is transmitted based on network configuration of CSI-RS resources. But RAN4 has not discussed how to activate the SCell without TRS (including periodic and aperiodic TRS) and SSB. The feasibility of this scenario may also need to be confirmed by RAN1.

Observation #4: RAN4 has not discussed how to activate the SCell without TRS transmission (including periodic and aperiodic TRS) and SSB in Scenario 2.

As one special case of Scenario 2, Scenario 2a aims at UL-only operation on the SSB-less SCell. Based on the explanation from proponent company, we would see it quite similar as a supplementary UL (SUL) operation. As SUL has been well defined including the RRM requirements in legacy releases, it needs to be clarified as to what from RAN4 needs to be studied for UL-Only SCell. Besides, the current scope of this activity is to extend the SSB-less operation from intra-band CA to the inter-band. It is not clear what additional requirements need to be studied in scenario 2a and why to consider this for inter-band CA.

Observation #5: The UL-only operation on the SSB-less SCell in Scenario 2a is quite similar as SUL, which has been well defined including the RRM requirements in legacy releases.

In any case, as Scenario 1 is originated from intra-band SSB-less SCell operation, we suggest RAN4 prioritize the study of FR1 inter-band SSB-less operation in this scenario.

Proposal 4: RAN4 to prioritize the SSB-less SCell operation for FR1 inter-band collocated CA in Scenario 1.

Side conditions

In last meeting, some conditions were collected and listed below to enable the SSB-less SCell operation for inter-band CA. While the feasibility of these candidate values is up to simulation, we can discuss the starting point of the values based on the inter-band collocated scenario.

Issue 2-1-2: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-1?

Online session (Monday April 24, 2023)

Agreements

  - Conditions for SSB-less inter-band CA SCell operation for scenario-1

    - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell

      - Option 1-1: 260ns

      - Option 1-2: 3us

      - Option 1-3: smaller than 3us

      - Other values are not precluded

    - The difference of the reception power with the FR1 inter-band active serving cell

      - Option 2-1: 6dB

      - Option 2-2: 6dB+offset

      - Other values are not precluded

    - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell)

    - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations.

    - FFS if requirements will be defined subject to feasibility analysis

    - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

As illustrated in Fig.1, the RTD at the UE side is derived from TAE between the PCell and SCell transmission timing and the difference on the propagation delay (PL).

Fig.1 Inter-band collocated CA

For FR1 inter-band CA operation, the TAE requirement is specified as 3us. In last meeting, RF agreed there is no impact to legacy requirements of inter-band CA TAE defined in TS38.104 [2]. There were arguments that TAE in deployment may be smaller in collocated scenario. However, the TAE value in use is based on network deployment; this is unknown to the UE and network cannot ensure a value smaller than 3us based on existing deployment. Therefore, 3us TAE shall be assumed when deriving RTD for FR1 inter-band collocated CA.

Observation #6: For inter-band collocated CA, the TAE requirement is specified as 3us.

Proposal 5: 3us TAE shall be assumed when deriving RTD for FR1 inter-band collocated CA.

Besides RTD, the propagation delay difference between the PCell and SCell needs to be considered. With collocated deployment, the propagation delay can be close because of the similar distance and transmission paths. But there could still be variations due to different channel characteristics over the two bands. This will add additional value X to the RTD received at the UE. Hence the RTD is expected to be 3+X us for FR1 inter-band collocated CA.

Observation #7:  There may be propagation delay difference due to different channel characteristics over the two bands.

Proposal 6: RTD is expected to be 3+X us for FR1 inter-band collocated CA.

Regarding to the difference of reception power, 6dB has been assumed for intra-band CA and 25dB for inter-band case. While the collocation deployment enables a smaller value of power difference, the transmission over inter-bands may experience different pathloss hence bring additional power difference at the UE. We would foresee the receive power difference may vary between 6dB and 25dB.

Observation #8: The transmission over inter-bands may experience different pathloss even for collocated scenario.

Proposal 7: The difference of reception power is expected to vary between 6dB and 25dB.

Another condition which may impact the feasibility is the frequency separation between inter-band carriers. As discussed above, if the two bands are close enough, the difference on the propagation delay as well as the receive power could be quite small. Otherwise, the impact on RTD and receive power may not be ignored. In any case, RAN4 should study the feasible range of frequency separation to enable the SSB-less SCell operation.

Proposal 8: RAN4 should study the feasible range of frequency separation between inter-band carriers to enable the SSB-less SCell operation.

About TCI state listed in the WF, we wonder if this indicates the QCL relation between the RSs from inter-band carriers. Although the QCL is up to network configuration, this should not be assumed always indicated considering different network deployment. The feasibility needs to be discussed considering both with and without QCL relation between the RSs from inter-band carriers.

Proposal 9:  For FR1 inter-band SSB-less operation, the feasibility needs to be discussed considering both with and without QCL configuration between the RSs from inter-band carriers.

Feasibility study

RAN4 should prioritize the feasibility study of Scenario 1, Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell. FFS whether to consider requirements based on TRS transmission during and/or after SCell activation ), for the case in which SCell’s TRS are not used for SCell activation. Under such case, the UE acquires time and frequency synchronization with the SCell only based on the PCell's SSB (i.e. with no assistance from SCell's TRS).

Based on the previous section, the feasibility study should consider the following aspects:

- The TAE between PCell and SCell

- The frequency difference between PCell and SCell

- The difference in received power between PCell and SCell

- The channel model characterizing the PCell and SCell’s operating frequency

RAN4 should identify the key performance metrics to be used for evaluating the feasibility or lack of it. It seems reasonable to evaluate the impact of the frequency separation between the PCell and SSB-less SCell on the key performance metrics of the physical downlink shared channel (and/or physical downlink control channel). Then the outcome of the study could be to identify the frequency separation between the PCell and SCell and conditions under which the SCell may achieve tolerable performance degradation.

Observation #9: The outcome of the feasibility study should be to identify the frequency separation between the PCell and SSB-less SCell (i.e., feasible band combinations) and any other conditions such that the demodulation performances (such as BLER) of the physical downlink data channel (PDSCH) transmitted by the SSB-less SCell are within a tolerable performance degradation.

Proposal 10: RAN4 to discuss and agree that the performance evaluation metrics to establish the feasibility of SSB-less SCell operation include at least metrics related to the demodulation performances of the PDSCH transmitted by the SSB-less SCell (e.g. BLER).

RRM impact due to SSB-less operation

It was agreed in RAN4#106bis-e to define the SSB-less based SCell activation delay, but this shall be based on the feasibility study. RAN4 can discuss the SCell activation delay requirements after feasibility study is concluded.

Issue 2-3-1: Requirements for SSB-less SCell activation

SSB-less based SCell activation delay is to be specified for inter-band CA for FR1 and co-located cells based on feasibility study.

Proposal 11: RAN4 to discuss the SCell activation delay for activating an inter-band co-located SSB-less SCell after the feasibility study is concluded.

Another aspect is the L1/L3 measurement on SSB-less SCells. In legacy specification, the measurement requirements have been defined for both SSB-based and CSI-RS based measurement. However, when SSB-less SCell is considered as suggested in Proposal 1, there is no SSB transmission hence not available for L1/L3 measurement as indicated in Fig.1. In any case, the UE is not required to perform SSB-based L1/L3 measurements on the SSB-less SCells.

Issue 2-4-1: Scenario for L1 measurement operation on the SSB-less SCell apart from SCell activation procedure

- Candidate Options:

  - Option 1: No SSB but with CSI-RS resource for L1 measurement on the inter-band SSB-less SCell

  - Option 2: No SSB and No CSI-RS resource for L1 measurement on the inter-band SSB-less SCell

Issue 2-4-2: Scenario for L3 measurement operation on the SSB-less SCell apart from SCell activation procedure

- Candidate Options:

  - Option 1: No SSB but with CSI-RS resource for L3 measurement on the inter-band SSB-less SCell

  - Option 2: No SSB and No CSI-RS resource for L3 measurement on the inter-band SSB-less SCell

Issue 2-4-3: L1/L3 measurement operation on the SSB-less SCell

- Candidate Options:

  - Option 1: given that the SSB-less SCell can fully leverage the information from an already activated serving cell, the corresponding L1/L3 measurements can be skipped.

  - Option 2: For UE deriving SSB-less SCell L1/L3 measurements results by performing L1/L3 measurements on PCell or another SCell, whether and how to assist UE to determine the coverage of SSB-less SCell should be further investigated.

  - Option 3: The UE is not required to perform SSB-based L1/L3 measurements on the SSB-less SCells. The CSI-RS based L1/L3 measurement requirements need to be specified for SSB-less SCell operation.

Observation #10: In SSB-less SCell operation, there is no SSB transmission hence not available for L1/L3 measurement.

Proposal 12: The UE is not required to perform SSB-based L1/L3 measurements on the SSB-less SCells.

In existing spec, CSI-RS resources can be used for L1-RSRP measurement and/or L3 measurements. This is up to network configuration. If CSI-RS based L1-RSRP measurement and/or L3 measurement is configured, we need identify the impact on the RRM requirements due to SSB-less SCell operation.,

Observation #11: CSI-RS based L1/L3 measurements are up to network configuration.

Proposal 13: RAN4 needs to discuss the impact on the CSI-RS based L1/L3 measurement requirements due to SSB-less SCell operation.

Conclusion

In this paper we have made the following proposals and observations related to the SSB-less SCell operation for inter-band CA for FR1 and co-located cells:

Observation #1: For intra-band SSB-less operation, TRS monitoring is not needed when activating the SSB-less SCell.

Proposal 1: RAN4 to study if it is feasible to achieve 3ms SCell activation delay for inter-band SSB-less operation.

Observation #2: If TRS monitoring is needed for activating the inter-band SSB-less SCell, the SCell activation delay longer than 3ms is expected.

Proposal 2: RAN4 to study the SSB-less SCell operation for scenario 1 considering the following cases:

- Case 1: TRS monitoring is not needed for SSB-less SCell activation (i.e. 3ms SCell activation delay is achieved).

- Case 2: TRS monitoring is needed for SSB-less SCell activation (wherein >3ms SCell activation delay is expected).

Observation #3: In Scenario 1, TRS may refer to the periodic TRS used for QCL relation or the aperiodic TRS used for fast SCell activation.

Proposal 3: RAN4 to clarify what type of TRS is intended for SSB-less SCell operation in Scenario 1.

Observation #4: RAN4 has not discussed how to activate the SCell without TRS transmission (including periodic and aperiodic TRS) and SSB in Scenario 2.

Observation #5: The UL-only operation on the SSB-less SCell in Scenario 2a is quite similar as SUL, which has been well defined including the RRM requirements in legacy releases.

Proposal 4: RAN4 to prioritize the SSB-less SCell operation for FR1 inter-band collocated CA in Scenario 1.

Observation #6: For inter-band collocated CA, the TAE requirement is specified as 3us.

Proposal 5: 3us TAE shall be assumed when deriving RTD for FR1 inter-band collocated CA.

Observation #7:  There may be propagation delay difference due to different channel characteristics over the two bands.

Proposal 6: RTD is expected to be 3+X us for FR1 inter-band collocated CA.

Observation #8: The transmission over inter-bands may experience different pathloss even for collocated scenario.

Proposal 7: The difference of reception power is expected to vary between 6dB and 25dB.

Proposal 8: RAN4 should study the feasible range of frequency separation between inter-band carriers to enable the SSB-less SCell operation.

Proposal 9:  For FR1 inter-band SSB-less operation, the feasibility needs to be discussed considering both with and without QCL configuration between the RSs from inter-band carriers.

Observation #9: The outcome of the feasibility study should be to identify the frequency separation between the PCell and SSB-less SCell (i.e., feasible band combinations) and any other conditions such that the demodulation performances (such as BLER) of the physical downlink data channel (PDSCH) transmitted by the SSB-less SCell are within a tolerable performance degradation.

Proposal 10: RAN4 to discuss and agree that the performance evaluation metrics to establish the feasibility of SSB-less SCell operation include at least metrics related to the demodulation performances of the PDSCH transmitted by the SSB-less SCell (e.g. BLER).

Proposal 11: RAN4 to discuss the SCell activation delay for activating an inter-band co-located SSB-less SCell after the feasibility study is concluded.

Observation #10: In SSB-less SCell operation, there is no SSB transmission hence not available for L1/L3 measurement.

Proposal 12: The UE is not required to perform SSB-based L1/L3 measurements on the SSB-less SCells.

Proposal 13: RAN4 needs to discuss the impact on the CSI-RS based L1/L3 measurement requirements due to SSB-less SCell operation.

References

[1] R4-2306369, WF on RRM requirements for NR network energy saving, Huawei

[2] R4-2306639, WF on RF feasibility study of NR Network Energy Saving, Huawei