3GPP TSG-RAN WG4 Meeting #107                                                                                    R4-2307383

Incheon, KR, May 22 – May 26, 2023

Source: 	CATT

Title: 	Further discussion on RF requirements for NCR-Fwd

Agenda item:	8.29.2.1

Document for:	Approval

- Introduction

In RAN4 #106-bis-e meeting, A WF [1] on NCR system parameters and RF requirements was agreed. Some remaining issues concerning NCR-Fwd in WF need further discussion in next meeting.

In this contribution, we provide our views on remaining issues for RF requirements for NCR-Fwd.

- Discussion

2.1 Repeater classes and its declaration for NCR

As per the WF [1], the remaining issues concerning repeater class and its declaration are shown as below:

- Agreement: - For NCR-fwd, following NCR classes introduced - DL/UE side: wide-area, medium range and local area - UL/BS side: Wide-area and local area - Above NCR classes applicable for all NCR types - NCR classes for NCR-MT - [Wide-area and local area] - FFS whether NCR class can be declared separately for NCR-fwd, and NCR-MT

Referring to TS 38.115-1 and TS 38.115-2, for RF repeater, the repeater need to be declared as three classes： Wide Area, Medium Range,  Local Area, and can be declared  separately for UL and DL. Therefore，for NCR, the same approaching could be considered. The NCR class could be declared as Wide Area NCR, Medium Range NCR, or Local Area NCR, and declarations could be made separately for Fwd UL, Fwd DL and MT. In addition，whether NCR class for Fwd UL and NCR class for MT is same class can be left to declaration.

Proposal 1: The NCR class can be declared as Wide Area NCR, Medium Range NCR, or Local Area NCR, and declarations can be made separately for Fwd UL, Fwd DL and MT. Whether NCR class for Fwd UL and NCR class for MT is same class can be left to declaration.

2.2 RF requirements for NCR type 1-H Fwd

Our analysis and views for RF requirements for NCR type 1-H Fwd are provided in Table 2-1.

Table 2-1 Summary of proposals for RF requirements for NCR type 1-H Fwd

| Requirement | Proposals |
| --- | --- |
| Repeater output power(for DL transmission, for UL transmission) | The scaling factor NTXU,counted = min(NTXU,active , 8×Ncells) can be used for repeater output power for UL transmission to limit sum of Prated,p,TABC for all TAB connectors for a passband. 1) For DL transmission Reuse BS type 1-H rated output power limits for BS classes in Table 6.2.1-2 of sub-clause 6.2.1 in TS 38.104 + X, where X = 10*log (ceil (passband bandwidth/20MHz)). Reuse the ±2.0 dB and ±2.5 dB accuracy from the BS type 1-H in sub-clause 6.2.3 of TS 38.104 for normal test conditions and extreme test conditions, respectively. This requirement for NCR type 1-H is as below: Repeater type 1-H DL transmission classes rated output power limits for repeater classes Repeater class; Prated,p,sys; Prated,p,TABC. Wide Area repeater; Note 1; Note 1. Medium Range repeater; ≤ 38 dBm +10log(NTXU,counted) + X, Note 2; ≤ 38 dBm+ X, Note 2. Local Area repeater; ≤ 24 dBm +10log(NTXU,counted) + X, Note 2; ≤ 24 dBm+ X, Note 2. NOTE1: There is no upper limit for the Prated,p,sys or Prated,p,TABC of the Wide Area repeater. NOTE2: X = 10*log (ceil (passband bandwidth/20MHz)). 2) For UL transmission Reuse minimum requirement for repeater type 1-C specified in sub-clause 6.2.2 of TS 38.106 for each TAB connector, and add Prated,p,sys with considering 10log(NTXU,counted) scaling. Reuse the ±2.0 dB and ±2.5 dB accuracy from the BS type 1-H in sub-clause 6.2.3 of TS 38.104 for normal test conditions and extreme test conditions, respectively. This requirement for NCR type 1-H is as below: Repeater type 1-H UL transmission classes rated output power limits for repeater classes Repeater class; Prated,p,sys; Prated,p,TABC. Wide Area repeater; Note 1; Note 1. Local Area repeater; ≤ 24 dBm + 10log(NTXU,counted) + X, Note 2; ≤ 24 dBm+ X, Note 2. NOTE 1: There is no upper limit for the Prated,p,TABC rated passband output power of the Wide Area repeater. NOTE 2: X = 10*log (ceil (passband bandwidth/20MHz)). |
| Adjacent Channel Leakage Power Ratio | The scaling factor X = 10log10(NTXU,countedpercell) can be used for ACLR(CACLR) absolute basic limits for UL, where NTXU,countedpercell = NTXU,counted / Ncells, and NTXU,counted = min(NTXU,active , 8×Ncells). Reuse the approach from BS type 1-H in sub-clause 6.6.3.4 of TS 38.104. 1) Use ACLR(CACLR) limits for repeater type 1-C specified in sub-clauses 6.5.2.2 of TS 38.106 as ACLR(CACLR) basic limits. This ACLR (CACLR) basic limits, whichever is less stringent, shall apply for each TAB connector TX min cell group for NCR 1-H. 2) Use ACLR(CACLR) absolute basic limits for repeater type 1-C specified in sub-clauses 6.5.2.2 of TS 38.106 as ACLR(CACLR) absolute basic limits. This ACLR (CACLR) absolute basic limits + X (where X = 10log10(NTXU,countedpercell)), whichever is less stringent, shall apply for each TAB connector TX min cell group for NCR 1-H. |
| Operating band unwanted emissions (Minimum requirements Category A, Minimum requirements Category B, Minimum requirements for additional requirements, Minimum requirements inside passband with no UL input signal) | The scaling factor X = 10log10(NTXU,countedpercell) can be used for operating band unwanted emissions for UL, where NTXU,countedpercell = NTXU,counted / Ncells, and NTXU,counted = min(NTXU,active , 8×Ncells). Reuse the approach from BS type 1-H in sub-clause 6.6.4.4 of TS 38.104. Use operating band unwanted emissions for repeater type 1-C specified in sub-clauses 6.5.3.2.1 - 6.5.3.2.6 of TS 38.106 as basic limit. The power summation emissions at the TAB connectors of the TAB connectors of each TAB connector TX min cell group shall not exceed a limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation. |
| Transmitter spurious emissions (General transmitter spurious emissions requirements, Additional spurious emissions requirements, Co-location with base stations and repeater type 1-C Nodes) | The scaling factor X = 10log10(NTXU,countedpercell) can be used for transmitter spurious emssion for DL and UL, where NTXU,countedpercell = NTXU,counted / Ncells, and NTXU,counted = min(NTXU,active , 8×Ncells). Reuse the approach from BS type 1-H in sub-clause 6.6.5.4 of TS 38.104. Use transmitter spurious emissions for repeater type 1-C specified in sub-clauses 6.5.4.2.1, 6.5.4.2.2 and 6.5.4.2.3 of TS 38.106 as basic limit. The power summation emissions at the TAB connectors of the TAB connectors of each TAB connector TX min cell group shall not exceed a limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation. |
| Receiver spurious emissions | The scaling factor X = 10log10(NRXU,countedpercell) can be used for receiver spurious emissions for DL and UL, where NRXU,countedpercell = NRXU,counted / Ncells, and NRXU,counted = min(NRXU,active , 8 × Ncells). Reuse the approach from BS type 1-H in sub-clause 7.6.4 of TS 38.104. Use Receiver spurious emissions for repeater type 1-C specified in sub-clause 6.5.5.2 of TS 38.106 as basic limit. The power sum of emissions at respective TAB connectors for each TAB connector RX min cell group shall not exceed the BS limits specified as the basic limits + X, where X = 10log10(NRXU,countedpercell), unless stated differently in regional regulation. |
| Output intermodulation(Minimum requirements(Co-location),Intra-system minimum requirements, Additional requirements) | 2) Intra-system minimum requirements Reuse intra-system minimum requirement for BS type 1-H specified in sub-clause 6.7.3.2 of TS 38.104. |

Proposal 2: For some RF requirements for NCR type 1-H Fwd, further adopt Table 2-1 as starting point.

2.3 RF requirements for NCR type 1-O Fwd

Our analysis and views for RF requirements for NCR type 1-O Fwd are provided in Table 2-2.

Table 2-2 Summary of proposals for RF requirements for NCR type 1-O Fwd

| Requirement | Proposals |
| --- | --- |
| OTA output power(EIRP for DL, TRP for DL, EIRP for UL, TRP for UL) | The scaling factor 9 dB can be used for TRP for UL. 1) EIRP for DL Define the requirement based on EIRP manufacturer declaration, and reuse the ±2.2 dB and ±2.7 dB EIRP accuracy from the BS type 1-O in sub-clause 9.2.2 of TS 38.104 for normal test conditions and extreme test conditions, respectively. 2) TRP for DL Use Prated,p,AC for repeater 1-C DL in Table 6.2.1-1 of sub-clause 6.2.1 of TS 38.106 + 9 dB scaling factor as Prated,p,TRP for NCR type 1-O Fwd DL. Reuse the ±2.0dB TRP accuracy from the BS type 1-O in sub-clause 9.3.2 of TS 38.104 for normal test conditions. This requirement for NCR type 1-O is as below: Repeater rated carrier TRP output power limits for NCR type 1-O Fwd DL transmission Repeater class; Prated,p,TRP. Wide Area; (note 1). Medium Range; ≤ + 47 dBm +X (note 2). Local Area; ≤ + 33 dBm +X (note 2). NOTE1: There is no upper limit for the Prated,p,TRP of the Wide Area Repeater. NOTE 2: X = 10*log (ceil (passband bandwidth/20MHz)). 3) EIRP for UL Define the requirement based on EIRP manufacturer declaration, with the accuracy of the declared output power value(Prated,p,EIRP) reusing the ±2.2 dB and ±2.7 dB from the BS type 1-O in sub-clause 9.2.2 of TS 38.104 for normal test conditions and extreme test conditions, respectively. 4) TRP for UL Use Prated,p,AC for repeater 1-C UL in Table 6.2.1-2 of sub-clause 6.2.1 of TS 38.106 + 9 dB scaling factor as Prated,p,TRP for NCR type 1-O Fwd UL. Reuse the ±2.0 dB TRP accuracy from the BS type 1-O in sub-clause 9.3.2 of TS 38.104 for normal test conditions. This requirement for NCR type 1-O is as below: Repeater rated carrier TRP output power limits for NCR type 1-O Fwd UL transmission Repeater class; Prated,p,TRP. Wide Area; (note 1). Local Area; ≤ + 33 dBm +X (note 2). NOTE1: There is no upper limit for the Prated,p,TRP of the Wide Area Repeater. NOTE 2: X = 10*log (ceil (passband bandwidth/20MHz)). |
| OTA Adjacent Channel Leakage Power Ratio | The scaling factor 9 dB can be used for ACLR(CACLR) absolute basic limits for UL. Reuse the approach from BS type 1-O in sub-clause 9.7.3.2 of TS 38.104. 1) Use ACLR(CACLR) limits for repeater type 1-C specified in sub-clauses 6.5.2.2 of TS 38.106 as ACLR(CACLR) basic limits. This ACLR (CACLR) basic limits, whichever is less stringent, shall apply NCR type 1-O. 2) Use ACLR(CACLR) absolute basic limits for repeater type 1-C specified in sub-clauses 6.5.2.2 of TS 38.106 as ACLR(CACLR) absolute basic limits. This ACLR (CACLR) absolute basic limits + X (where X = 9), whichever is less stringent, shall apply for NCR type 1-O. |
| OTA Operating band unwanted emissions (OTA OBUE Category A, OTA OBUE Category B, Additional OTA OBUE requirements) | The scaling factor 9 dB can be used for operating band unwanted emissions for UL. Reuse the approach from BS type 1-O in sub-clause 9.7.4.2 of TS 38.104. Use Operating band unwanted emissions for repeater type 1-C specified in sub-clauses 6.5.3.2.1 - 6.5.3.2.5 of TS 38.106 as basic limit. The TRP power of any unwanted emission shall not exceed a limit specified as the basic limit + X, where X = 9, unless stated differently in regional regulation. |
| OTA Transmitter spurious emissions (General OTA transmitter spurious emissions requirements, Additional OTA spurious emissions requirements) | The scaling factor 9 dB can be used for transmitter spurious emissions for UL. Reuse the approach from BS type 1-O in sub-clause 9.7.5.2 of TS 38.104. Use Transmitter spurious emissions for repeater type 1-C specified in sub-clauses 6.5.4.2.1, 6.5.4.2.2 and 6.5.4.2.3 of TS 38.106 as basic limit. The TRP power of any spurious emission shall not exceed a limit specified as the basic limit + X, where X = 9, unless stated differently in regional regulation. |
| OTA Receiver spurious emissions | N/A |
| OTA Error Vector Magnitude(Downlink repeater error vector magnitude, Uplink repeater error vector magnitude) | 1) Downlink repeater error vector magnitude Use minimum input power for repeater type 1-C specified in Table 6.6.1.1-1 in sub-clause 6.6.1.1 of TS 38.106 - GRX_ANT for NCR type 1-O Fwd DL. Where GRX_ANT is the gain of the receive side antennas and is based on EIRP and TRP declaration. This requirement for NCR type 1-O is as below: Minimum input power for repeater EVM for Fwd DL Repeater DL class; Minimum input power spectral density (dBm/MHz). QPSK, 16 QAM, 64QAM; 256QAM1. WA; -82- GRX_ANT; -75- GRX_ANT. MR; -77- GRX_ANT; -70- GRX_ANT. LA; -74- GRX_ANT; -67- GRX_ANT. Note 1: support of 256QAM is based on the declaration. Reuse EVM for repeater type 1-C specified in sub-clause 6.6.1.2 of TS 38.106 NCR type 1-O Fwd DL. 2) Uplink repeater error vector magnitude Use minimum input power for repeater type 1-C specified in Table 6.6.2.1-1 in sub-clause 6.6.2.1 of TS 38.106 - GRX_ANT for NCR type 1-O Fwd UL. Where GRX_ANT is the gain of the receive side antennas and is based on EIRP and TRP declaration. This requirement for NCR type 1-O is as below: Minimum input power for repeater EVM for Fwd UL Repeater UL class; Minimum input power spectral density (dBm/MHz). QPSK, 16 QAM, 64QAM; 256QAM1. WA; -82- GRX_ANT; -75- GRX_ANT. LA; -74- GRX_ANT; -67- GRX_ANT. Note 1: support of 256QAM is based on the declaration. Reuse EVM for repeater type 1-C specified in sub-clause 6.6.2.2 of TS 38.106 for NCR type 1-O Fwd UL. |
| OTA input intermodulation | 1) General requirement Reuse general requirement for repeater type 2-O approach for NCR type 1-O. Use Interfering Signal Levels for repeater type 1-C specified in Table 6.7.1.2-1 in sub-clause 6.7.1.2 of TS 38.106 - GRX_ANT for NCR type 1-O Fwd DL. Where GRX_ANT is the gain of the receive side antennas and is based on EIRP and TRP declaration. This requirement for NCR type 1-O is as below: Input intermodulation requirement f1 offset; Interfering Signal Levels; Type of signals; Measurement bandwidth. 1 MHz; -40 dBm - GRX_ANT; 2 CW carriers; 1 MHz. 2) Co-location with BS/repeater in other systems This requirement is co-location requirement, which is from co-location minimum requirement of out-of-band blocking of BS in TS 38.104. Reuse co-location minimum requirement of out-of-band blocking for BS type 1-O for NCR type 1-O. Use interfering signal mean power for repeater type 1-C specified in Table 6.7.2.2-1 in sub-clause 6.7.2.2 of TS 38.106 + 30 dB as interfering signal mean power for NCR type 1-O Fwd DL. This interferer power levels are specified at the co-location reference antenna conducted input. This requirement for NCR type 1-O is as below: Input intermodulation requirement for NR repeater DL when co-located with BS/repeater in other frequency bands. Frequency range of interfering signal; Interfering signal mean power for repeater with WA UE side (dBm); Interfering signal mean power for repeater with MR UE side(dBm); Interfering signal mean power for repeater with LA UE side(dBm); Type of interfering signals. Frequency range of co-located BS’s downlink operating band or located repeater’s passband; + 46; + 38; + 24; 2 CW carriers. NOTE 1: The requirement does not apply when the interfering signal falls within the passband. NOTE 2: For unsynchronized base stations (except in band n46, n96, and n102) or repeaters, special co-location requirements may apply that are not covered by the 3GPP specifications.. Use interfering signal mean power for repeater type 1-C specified in Table 6.7.2.2-2 in sub-clause 6.7.2.2 of TS 38.106 +30 dB as interfering signal mean power for NCR type 1-O Fwd UL. This interferer power levels are specified at the co-location reference antenna conducted input. This requirement for NCR type 1-O is as below: Input intermodulation requirement for NR repeater UL when co-located with BS/repeater in other frequency bands. Frequency range of interfering signal; Interfering signal mean power for repeater with WA BS side(dBm); Interfering signal mean power for repeater with LA BS side(dBm); Type of interfering signals. Frequency range of co-located BS’s downlink operating band or located repeater’s passband; + 46; Prated,p,TRP; 2 CW carriers. NOTE 1: The requirement does not apply when the interfering signal falls within the passband. NOTE 2: For unsynchronized base stations (except in band n46, n96, and n102) or repeaters, special co-location requirements may apply that are not covered by the 3GPP specifications.. 3) Co-existence with other systems Use interfering signal mean power for repeater type 1-C specified in Table 6.7.3.2-1 in sub-clause 6.7.3.2 of TS 38.106 +30 dB as interfering signal mean power for NCR type 1-O Fwd. This interferer power levels are specified at the co-location reference antenna conducted input. This requirement for NCR type 1-O is as below: Input intermodulation requirement for NR repeater when co-exist with BS/repeater in other non-overlapping frequency bands Frequency range of interfering signal; Interfering signal mean power (dBm); Type of interfering signals; Measurement bandwidth. Frequency range of co-existence system operating band; + 15; 2 CW carriers; 1MHz. NOTE 1: All the interfering signals should be limited into the frequency ranges that are either X MHz higher than FUL,high or X MHz lower than FUL,low, where X equals to 20MHz when FUL,high - FUL,low is not larger than 200MHz, otherwise X equals to 60MHz. |
| Output intermodulation(Minimum requirements(Co-location)) | 1) Minimum requirements(Co-location) Reuse OTA transmitter intermodulation for BS type 1-O specified in sub-clause 9.8.2 of TS 38.104. And an interfering signal is injected into the co-location reference antenna. |
| OTA Adjacent Channel Rejection Ratio (ACRR) | Reuse ACRR for repeater type 1-C specified in sub-clause 6.9.2 of TS 38.106. |
| OTA Transmit ON/OFF power(OTA Transmitter OFF power, OTA Transmitter transient period) | 1) OTA Transmitter OFF power: Reuse OTA transmitter OFF power for BS type 1-O specified in sub-clause 9.5.2.2 of TS 38.104. The total power from all co-location reference antenna conducted output(s) shall be less than -106 dBm/MHz. 2) OTA Transmitter transient period Reuse OTA transmitter transient period for BS type 1-O specified in sub-clause 9.5.3.2 of TS 38.104. Transient period length (µs) is 10us. |

Proposal 3: For some RF requirements for NCR type 1-O Fwd, further adopt Table 2-2 as starting point.

- Conclusion

This contribution provides our consideration on RF requirements for NCR-Fwd. The following proposals are concluded as follows:

Proposal 1: The NCR class can be declared as Wide Area NCR, Medium Range NCR, or Local Area NCR, and declarations can be made separately for Fwd UL, Fwd DL and MT. Whether NCR class for Fwd UL and NCR class for MT is same class can be left to declaration.

Proposal 2: For some RF requirements for NCR type 1-H Fwd, further adopt Table 2-1 as starting point.

Proposal 3: For some RF requirements for NCR type 1-O Fwd, further adopt Table 2-2 as starting point.

- References

- R4-2305931, WF on system parameters and RF requirements, ZTE, RAN4#106-bis-e

- R4-2302903, WF for NCR system parameters and RF requirements, ZTE, RAN4#106

- R4-2220255, WF for NCR system parameters and RF requirements, ZTE, CMCC, RAN4#105

- R4-2217471, WF on NCR RF requirements, ZTE, RAN4#104bis-e