3GPP TSG-RAN WG4 Meeting  # 107	R4-2307396

# Incheon, KR, May 22th – May 26th, 2023

Title: 	Discussion on measurement requirements for Rel-18 ATG

Source: 	CATT

Agenda item:	8.14.4.5

Document for:	Discussion

- Introduction

In RAN4#106 meeting, a WF on NR ATG RRM core requirements was approved and many issues were discussed and the current states have been summarized in it [1].

This document will further discuss these issues for measurement requirements for ATG RRM requirements and present our views.

- Discussion

The conclusions on measurement requirements in the last meeting are extracted as following [1], and we further provide our discussion.

Issue 5-1: Measurement mechanism

- Option 1:

  - If ATG UE altitude higher than certain threshold, ATG UE perform measurement and measurement report

  - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement and/or does not perform measurement report

- Option 2: Wait for the progress in RAN2 for the height-dependent configuration for measurements in connected mode for reusing the framework in ATG.

- Option 3: Do not introduce height-dependent requirements for measurement procedures for ATG UES

  - Option 4-1: All requirements are applicable for the ATG CPE which is not lower than 3km

- Option 4: De-prioritize new measurement mechanism

We believe that ATG UE does not need to measure ATG cells before take-off and after landing, there is no need to perform measurement and measurement reports when ATG UE is on the ground. Actually, CPE will be a controlled device, it could be powered off when the height is lower than a certain value, and there is no need to specify the enhanced measurement requirements.

Proposal 1: Prefer not to introduce height-dependent requirements for measurement procedures for ATG UEs

Issue 5-3: Scheduling restriction

- Option 1:

  - The legacy scheduling restriction requirement can be reused for ATG.

    - When it is enabled, the restriction is on SSB symbols and 1 data symbol before and after.

    - When it is disabled, the restriction is on all symbols in SMTC.

- Option 2:

  - When NW enables the deriveSSB-IndexFromCell, the UE is allowed to not transmit or receive data more than 1 symbols before and after SSB symbols to be measured. The dedicated symbol value can be indicated by NW.

  - When NW disables the deriveSSB-IndexFromCell, the UE is allowed to not transmit or receive data more than 1 symbols before and after SMTC to be measured. The dedicated symbol value can be indicated by NW.

- Option 3:

  - Legacy scheduling restriction to measure neighbor cell is not suitable, so RAN4 needs to decide how to define scheduling restriction considering

    - Alt 1: define scheduling restriction based on maximum ISD in ATG

    - Alt 2: define scheduling restriction based on the difference of propagation delay from serving cell and neighbor cells

We prefer option 1 to reuse the legacy scheduling restriction requirement. In our view, when to enable or disable deriveSSB-IndexFromCell is an NW implementation issue. When NW enables the deriveSSB-IndexFromCell, that is, NW ensures the frame of boundary alignment between serving cell and target cell could meet the requirement of min(2 SSB symbols, 1 PDSCH symbol) [2], so 1 data symbol is enough. When NW disables the deriveSSB-IndexFromCell, it is hard to define new scheduling restriction based on ISD or the difference of propagation delay compared with performance loss led by the restriction on all symbols in SMTC.

Proposal 2: The legacy scheduling restriction requirement can be reused for ATG.

- When it is enabled, the restriction is on SSB symbols and 1 data symbol before and after.

- When it is disabled, the restriction is on all symbols in SMTC.

- Conclusion

In this paper, we provide our views on measurement requirements. From this discussion we have derived the following proposals:

Proposal 1: Prefer not to introduce height-dependent requirements for measurement procedures for ATG UEs

Proposal 2: The legacy scheduling restriction requirement can be reused for ATG.

- When it is enabled, the restriction is on SSB symbols and 1 data symbol before and after.

- When it is disabled, the restriction is on all symbols in SMTC.

- Reference

[1]     R4-2306344, WF on NR ATG RRM requirements,	CMCC

[2]     R4-2306243, Topic summary for [106-bis-e][212] NR_ATG, CMCC