3GPP TSG-RAN WG4 Meeting # 107	            R4-2307399

# Incheon, KR, May 22th – May 26th, 2023

Agenda Item:	8.25.2.2

Source: 	CATT

Title: 	Discussion on L1-RSRP measurement requirements for L1/L2 based inter-cell mobility

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, WF on L1/L2 inter-cell mobility was approved. In this contribution [1], some open issues on L1-RSRP measurement requirements for L1/L2 based inter-cell mobility are further discussed.

2. Discussion

Applicability rule for L1-RSRP measurement

Issue 2-2-3: Whether L1 measurement configured after receiving L3 measurement report on that cell

< Way forward >: FFS the following options

  - Option 1 (MTK, Xiaomi, Huawei, CTC, Apple, CATT): UE performs L1 measurement on a neighbour cell after UE has performed L3 measurement on that cell in the last [5] seconds.

  - Option 2 (MTK, Ericsson): When NW configures a cell for L1 measurement, and if the UE did not measure that cell in last [5] seconds, UE performs L1 measurement on that cell after performing L3 measurement.

  - Option 3 (CMCC, MTK, Nokia): L3 measurement report is not a prerequisite for LTM L1-measurement configuration.

This issue should be clarified from two perspective, and the above option may not conflict with each other.

For the behavior of the NW, we think that its configuration shouldn’t be restricted. Therefore, we agree that L3 measurement report is not a prerequisite for LTM L1 measurement configuration, that is, the NW does not rely on L3 measurement report to configure L1 measurement, the cells configured by the NW can be known or unknown .

For the behavior of the UE, it is more reasonable and convenient to performs L1 measurement on the known cell, that is, UE performs L1 measurement on a neighbour cell after UE has performed L3 measurement on that cell in the last [5] seconds.

Proposal 1: For the behavior of the NW, its configuration shouldn’t be restricted, the NW does not rely on L3 measurement report to configure L1 measurement, the cells configured by the NW can be known or unknown.

Proposal 2: For the behavior of the UE, it is more reasonable to perform L1 measurement on the known cell.

Issue 2-2-4: known cell condition for L1-RSRP measurement

< Way forward >: FFS the following options

  - Option 1(MTK, Xiaomi, Huawei, CTC, Apple, ZTE, OPPO, Ericsson, QC):

    - Target cell is considered as known if the following conditions are met in this requirement:

      - UE has performed L3 measurement on the target cell during the last [5] seconds, and

      - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

    - Otherwise, it is unknown

  - Option 2 (Nokia):

    - In L1-RSRP measurement for neighbour cell, target cell is considered as known if the following conditions are met in this requirement:

      - The UE has sent a valid L3 measurement report during the last [5] seconds, and

      - The SSB from the target cell remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

    - Otherwise, it is unknown

  - Option 3 (CMCC):

    - In L1-RSRP measurement for neighbour cell, target cell is considered as known if the following conditions are met in this requirement:

      - The UE has sent a valid L3 measurement report during the last [5] seconds, and

      - The SSB from the target cell remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

    - Otherwise, it is unknown

In our understanding, the known cell condition discussed in this issue is used to make sure the cell to perform L1-RSRP measurement is known to UE [2]. Given that we support the NW not relying on L3 measurement report to configure L1 measurement in the issue 2-2-3, so we support the wording in option 1 for the known cell condition.

Proposal 3: Target cell is considered as known if the following conditions are met in this requirement:

- UE has performed L3 measurement on the target cell during the last [5] seconds, and

- The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

Otherwise, it is unknown.

Measurement capability

Issue 2-4-1: The number of cells/resources supported per frequency layer for L1 measurement

< GTW Agreement>:

  - The supported maximum number of cells/SSBs configured for L1 measurement [on neighbour cell] is up to UE capability

    - Details of capability signalling are FFS including at least:

      - Whether to differentiate intra and inter-frequency measurements

      - Constraints on the lowest value of the capability

      - Signalling granularity

RAN1 discussed the related issues in the last meeting, and agreement for the beam selection for SSB based L1-RSRP measurement report is as follows [3]:

Agreement For the beam selection for SSB based L1-RSRP measurement report, - Beam selection is performed across the L cells from configured (or activated, if introduced) cells, i.e. M beams for each of the L cells - FFS: How to select the L cells and M beams per cells is up to UE - M x L beams are reported in a single report instance - Max values of M and L are based on UE capability, and at least M x L=4 is supported as a UE capability, other UE capabilities are FFS - FFS if UE is allowed to report less than M x L beams - The values of M and L are configured to the UE in the reporting configuration - FFS: The following configurability is introduced in the report configuration - 1) Whether serving cell is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 - 2) at least one of the inter-frequency cells is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 and at least one cell in inter-frequency

According to the above agreement, M is the number of beams and L is the number of cells supported for SSB based L1-RSRP measurement report. In our understanding, M and L is discussed from the aspect of the reporting configuration in RAN1. From RAN4’s perspective, the supported maximum number of cells configured for L1 measurement [on neighbour cell] is up to UE capability, the lowest value of the capability should not smaller than L.

Proposal 4: The supported maximum number of cells configured for L1 measurement [on neighbour cell] is up to UE capability

- The lowest value of the capability should not smaller than L in RAN1.

Issue 2-4-3: How to handle the number of cells NW configured to measure exceeds the configuration to exceed UE capability?

< Way forward >: FFS the following options

  - Option 1: The configured SSB for L1 measurement on neighbor cell shall not exceed UE capability.

  - Option 2: UE autonomous down selection of cells and/or SSBs for L1 measurements and/or L1 measurement and report based on the event conditions NW configured.

  - Option 3: Down selection can be defined based on NW indication.

Option 1 is not conflict with option 2 and option 3. From NW configuration point of view, though the configuration of NW should not be restricted, we consider that it makes no sense to configure anything beyond UE capability. Therefore, it is suggested to focus on the case where the configured SSB for L1 measurement on neighbor cell does not exceed UE capability. However, it is also possible for UE to autonomously down select cells and/or SSBs for L1 measurements. We believe that down selection criteria can be defined based on NW indication or event configuration, and it would be better to discuss the event conditions that trigger the downward selection of cells and/or SSBs in RAN2 [2].

Observation 1: From NW configuration point of view, the configuration of NW should not be restricted, but it makes no sense to configure anything beyond UE capability.

Proposal 5: It is possible for UE to autonomously down select cells and/or SSBs for L1 measurements.

- The down selection criteria can be defined based on NW indication or event configuration.

- It would be better to discuss the event conditions that trigger the downward selection of cells and/or SSBs in RAN2.

Inter-frequency L1-RSRP Measurement delay

2.6.1 Scenario

Issue 2-6-1-1: Type of inter-frequency L1-RSRP measurement to support

RAN4 have agreed to introduce inter-frequency L1-RSRP measurement in the previous meeting. However, no conclusion has been reached on which type of inter-frequency L1-RSRP measurement requirements in Rel-18 LTM are introduce, the options for way forward are as follows [1]:

< Way forward >: FFS the following options

  - Option 1: As baseline, define the requirement for inter-frequency L1-RSRP measurement with type 1 MG.

  - Option 2: Both inter-frequency L1-RSRP measurement with gap and without gap should be considered

  - Option 3:

    - Consider both inter-frequency with Type 1 MG and inter-frequency without gap (the target cell’s SSB is completely contained in the DL active BWP)

    - Introduce a UE capability for inter-frequency without gap (the target cell’s SSB is completely contained in the DL active BWP).

      - FFS the details

  - Option 4: For R17 L1 measurement, NOT to define inter-frequency L1 measurement requirements. For new Type of L1 measurement if agreed in other WGs, inter-frequency requirements are specified for both the within gap case and outside gap case, and the legacy L3 measurement behaviour is re-used.

In our view, considering the integrity of the defined scenario, at least both inter-frequency L1-RSRP measurement with gap and without gap should be considered.

Generally, inter-frequency L1-RSRP measurement without gap can be introduced if UE capability allow like Rel-16. If considering the workload of the WI and the inter-frequency L1-RSRP measurement with gap is a more typical scenario, at least we can compromise to define the requirement for inter-frequency L1-RSRP measurement with type 1 MG as a baseline.

Proposal 6: At least define the requirement for inter-frequency L1-RSRP measurement with type 1 MG as a baseline.

Proposal 7: Consider both inter-frequency with Type 1 MG and inter-frequency without gap (the target cell’s SSB is completely contained in the DL active BWP)

Proposal 8: Introduce a UE capability for inter-frequency L1-RSRP measurement without gap (the target cell’s SSB is completely contained in the DL active BWP) like Rel-16.

2.6.2 L1 inter-frequency with Type 1 MG

Issue 2-6-2-1: The principles in defining inter-frequency L1-RSRP measurement period with MG in FR1

< Way forward >: FFS the following options

  - Proposal 1: In FR1, UE carries out L1-RSRP inter-frequency SSB measurements within L3 measurement gaps.

  - Proposal 2: For FR1 L1-RSRP measurement on candidate cells with gap in LTM, UE can perform L3-RSRP measurement and L1-RSRP measurement simultaneously during measurement gap.

For principles in defining inter-frequency L1-RSRP measurement period with MG in FR1, if UE carries out L1-RSRP inter-frequency SSB measurements within L3 measurement gaps in FR1, we need to further discuss the priority of using MG for L1 measurement and L3 measurement to ensure that the measurement delay will not be too long compared with intra-frequency L1-RSRP measurement. And the impact on legacy L3 measurement should be discussed.

Proposal 9: In FR1, UE carries out L1-RSRP inter-frequency SSB measurements within L3 measurement gaps.

- The priority of using MG for L1 measurement and L3 measurement to ensure that the measurement delay will not be too long compared with intra-frequency L1-RSRP measurement should be further discussed.

- FFS: Other impact on legacy L3 measurement.

2.6.4 L1 inter-frequency with NCSG

Issue 2-6-4-1: The principles in defining inter-frequency L1-RSRP measurement period with NCSG

< Way forward >: FFS the following options

  - Option 1 (Huawei): For SSB based L1-RSRP inter-frequency measurement with NCSG:

    - In FR1, SSB based L1-RSRP measurement can be performed simultaneously with L3-RSRP measurement;

    - In FR2, SSB based L1-RSRP measurement is to be shared with L3 measurement with NCSG. The measurement delay with NCSG is the same as that with shared legacy gap

  - Option 2: FFS

Considering the workload, we think that L1 inter-frequency with NCSG can be postponed for further study in the future.

Proposal 10: L1 inter-frequency with NCSG can be postponed for further study in the future.

L1-RSRP measurement accuracy

Issue 2-7-1: side condition of intra-frequency L1-RSRP measurement accuracy requirements

< Way forward >: FFS the following options

  - Option 1: Reuse legacy value SNR= -3dB

  - Option 2: SNR = -6dB (same as L3 measurement)

  - Option 3: For R17 L1 measurements, the side condition for measurement accuracy requirements is -3dB. For new type of L1 measurements if agreed by other WGs, the side condition for measurement accuracy requirements is -6dB.

In Rel-15, because the number of samples for L1-RSRP measurement is smaller than L3 measurement, a higher SNR value is set for L1-RSRP measurement to ensure measurement accuracy. For R17 ICBM, the SNR side condition of intra frequency L1-RSRP measurement L1-RSRP is also set to - 3dB, and LTM is expected to achieve performance similar to that of ICBM in beam management. Therefore, we believe that the same requirements should be followed for LTM, and it is more reasonable to set the SNR side condition of intra frequency L1-RSRP measurement to - 3dB.

Proposal 11: It is suggested to set the SNR side condition of intra frequency L1-RSRP measurement to - 3dB.

3. Conclusion

In this paper, we provide our views on L1-RSRP measurement requirements for L1/L2 based inter-cell mobility. From this discussion we have derived the following observation and proposals:

Proposal 1: For the behavior of the NW, its configuration shouldn’t be restricted, the NW does not rely on L3 measurement report to configure L1 measurement, the cells configured by the NW can be known or unknown.

Proposal 2: For the behavior of the UE, it is more reasonable to perform L1 measurement on the known cell.

Proposal 3: Target cell is considered as known if the following conditions are met in this requirement:

- UE has performed L3 measurement on the target cell during the last [5] seconds, and

- The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

Otherwise, it is unknown.

Proposal 4: The supported maximum number of cells configured for L1 measurement [on neighbour cell] is up to UE capability

- The lowest value of the capability should not smaller than L in RAN1.

Observation 1: From NW configuration point of view, the configuration of NW should not be restricted, but it makes no sense to configure anything beyond UE capability.

Proposal 5: It is possible for UE to autonomously down select cells and/or SSBs for L1 measurements.

- The down selection criteria can be defined based on NW indication or event configuration.

- It would be better to discuss the event conditions that trigger the downward selection of cells and/or SSBs in RAN2.

Proposal 6: At least define the requirement for inter-frequency L1-RSRP measurement with type 1 MG as a baseline.

Proposal 7: Consider both inter-frequency with Type 1 MG and inter-frequency without gap (the target cell’s SSB is completely contained in the DL active BWP)

Proposal 8: Introduce a UE capability for inter-frequency L1-RSRP measurement without gap (the target cell’s SSB is completely contained in the DL active BWP) like Rel-16.

Proposal 9: In FR1, UE carries out L1-RSRP inter-frequency SSB measurements within L3 measurement gaps.

- The priority of using MG for L1 measurement and L3 measurement to ensure that the measurement delay will not be too long compared with intra-frequency L1-RSRP measurement should be further discussed.

- FFS: Other impact on legacy L3 measurement.

Proposal 10: L1 inter-frequency with NCSG can be postponed for further study in the future.

Proposal 11: It is suggested to set the SNR side condition of intra frequency L1-RSRP measurement to - 3dB.

4. Reference

- R4-2306395 WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.

- R4-2306249 Topic summary for [106-bis-e][218] NR_Mob_enh2_part1, MediaTek Inc.

- R1-2304276 LS on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM, RAN1