3GPP TSG-RAN WG4 Meeting # 107	            R4-2307403

# Incheon, KR, May 22th – May 26th, 2023

Agenda Item:	8.25.5

Source: 	CATT

Title: 	Discussion on enhanced CHO configurations

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, WF on NR Mobility Enhancements RRM requirements (part 2) was approved [1]. In this contribution, some open issues on enhanced CHO configurations are further discussed.

2. Discussion

CHO including target MCG and target SCG in NR-DC (obj. 3)

Issue 3-2-1: PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3)

- Agreements:

  - PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as:

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution.

  - Definition of each component, except Tprocessing, is same as that defined in CHO requirement in TS38.133 clause 6.4.1.2.

  - Definition of Tprocessing:

    - Option 1: same as that defined in requirements of handover with PSCell.

    - Option 2: same as that defined in requirements of CHO.

In the last meeting, there was no consensus on the definition of Tprocessing. The scenario is CHO with PSCell, the main controversial point is that Tprocessing is same as that defined in requirements of handover with PSCell or that defined in requirements of CHO [2].

Considering that after the conditions in CHO is met, UE behavior is similar as HO with PSCell, we can support it is same as that defined in requirements of HO with PSCell, so Tprocessing in requirements of handover with PSCell can be directly reused.

Proposal 1: Tprocessing for CHO with PSCell is same as that defined in requirements of HO with PSCell.

Issue 3-2-2: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3)

- Candidate solutions:

  - Companies are encouraged to further discuss the following issues:

    - When does UE execute PSCell addition/change

      - Option 1: after PCell CHO is executed but before it is completed.

      - Option 2: after PCell CHO is completed.

    - Whether Tsearch_PCell_Conditional is neeed in DCHOwithPSCell_PSCell?

For when does UE execute PSCell addition/change, we prefer option 1. UE has no motivation to have additional time to execute PSCell addition/change because UE has been continuing measurement the target Pcell [2].

For whether Tsearch_PCell_Conditional, we prefer to keep it from DCHOwithPSCell_PSCell, and we need to clarify the case of target known PCell and target unknown PSCell.

Proposal 2: UE execute PSCell addition/change after PCell CHO is executed but before it is completed.

Proposal 3: Tsearch_PCell_Conditional is neeed in DCHOwithPSCell_PSCell.

CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

Issue 3-3-1: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4)

- Candidate solutions:

  - Option 1: same as that in obj. 3.

  - Option 2: DCHOwithCPAC = Max [DCHO,  DCPAC]

    - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

    - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition

    - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition

  - Option 3: same as CHO

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

    - Tprocessing = 25 ms

    - TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

Issue 3-3-2: PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4)

- Candidate solutions:

  - Option 1:

    - if conditions for CPC are met before conditions for CHO are met, follow objective 3 requirements.

    - If conditions for CPC are met after CHO is complete, reuse existing CPC requirements.

  - Option 2: DCHOwithCPAC = Max [DCHO,  DCPAC]

    - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

    - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition

    - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition

  - Option 3:

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

    - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell

    - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

It was agreed that UE needs to wait to execute conditional PCell/PScell handover until both CHO and CPC conditions are met, so DCHOwithCPAC will depend on the larger handover delay of the CHO and CPC . So Option 2 can be a baseline. In addition, since UE needs to wait until conditions for two events are met, the case will become more complex, so TEvent_DU may need to be updated [2].

Proposal 4: DCHOwithCPAC for PCell /PScell in Option 2 can be a baseline.

- TEvent_DU may need to be updated.

3. Conclusion

In this paper, we provide our views on enhanced CHO configurations. From this discussion we have derived the following proposals:

Proposal 1: Tprocessing for CHO with PSCell is same as that defined in requirements of HO with PSCell.

Proposal 2: UE execute PSCell addition/change after PCell CHO is executed but before it is completed.

Proposal 3: Tsearch_PCell_Conditional is neeed in DCHOwithPSCell_PSCell.

Proposal 4: DCHOwithCPAC for PCell /PScell in Option 2 can be a baseline.

- TEvent_DU may need to be updated.

4. Reference

- R4-2306396 WF on NR Mobility Enhancements RRM requirements (part 2), Apple

- R4-2306250 Topic summary for [106bis-e][219] NR_Mob_enh2_part2, Apple