3GPP TSG-RAN4 Meeting #107	                                                            R4-2307406

Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on case 1 requirements for combination of pre-MG and concurrent MGs

Source:	CATT

Agenda Item:	8.10.2.2

Document for:	Discussion

1 Introduction

In RAN4#106bis-e meeting, RRM requirements for the combination of Pre-MG, concurrent MGs and NCSG were further discussed and the conclusions for case 1 (combination of Pre-MG and concurrent MGs) were captured in the approved WF [1]. In this paper, we will further discuss this part and present our views.

2 Discussion

Scope:

The scope of the WI (i.e. whether to consider Type-1 MG) has been discussed for many times and needs to be concluded. Since there are no priority and association with frequency layers configured for Type-1 MG, the signaling needs to be updated for Type-1 MG if we are going to include it together with Pre-MG/NCSG and define the requirements. Based on the discussions in the meeting, we understand the essence of this issue is still whether type-1 MG can be configured simultaneously with concurrent MG which has been discussed in Rel-17. In Rel-17, the simultaneous configuration is allowed but there are no requirements. To move forward, we can compromise to option 2a which is same as Rel-17.

- Issue 2-1-1: Which Type of MG is considered together with Pre-MG/NCSG in the WI? < Way forward >: - Option 1: Type-1 MG is not considered with Pre-MG/NCSG in the WI. - Option 2a: Type-1 MG can be considered with Pre-MG/NCSG in the WI, but there is no requirement when type 1 MG is colliding with pre-MG or NCSG in the same FR.

Proposal 1: Prefer option 1 not to consider Type-1 MG with Pre-MG/NCSG in the WI, but can compromise to option 2a to move forward.

Activation/deactivation delay:

In last meeting, RAN4 further discussed the delay requirements for simultaneous Pre-MG activation/deactivation and the scenarios are differentiated into two scenarios as summarized below:

- Fully overlap:

  - Both Pre-MGs are triggered by the same event, as shown below:

  - Two Pre-MGs are triggered by 2 events of the same type at the same time, as shown below:

- Partially overlap:

  - Two Pre-MGs are triggered by 2 events of the same type at different time, as shown below:

  - Two Pre-MGs are triggered by different type of events at the same time, as shown below:

  - Two Pre-MGs are triggered by different type of events at different time, as shown below:

  - Two Pre-MGs are triggered by different type of events at different time but the delay finish at the same time, as shown below:

And the requirements for both scenarios were discussed and agreed as below:

- Issue 3-2-2: [Case 1] Whether to extend the delay for fully overlapped simultaneous activation/deactivation for Pre-MG+Pre-MG < Agreement >: - The fully overlapped simultaneous multiple Pre-MGs activation/deactivation delay equals the BWPs/SCells/RRC reconfiguration delay plus existing processing time (5ms) plus the additional post-processing time T1, where T1 value is FFS. - An illustration example is captured below: < Way forward >: - FFS the value of T1 with the following options: - Option 1: 0 ms - Option 2: < = 5 ms - Other values are not precluded - Issue 3-2-3: [Case 1] Whether to extend the delay for partially overlapped simultaneous activation/deactivation for Pre-MG+Pre-MG < Way forward >: FFS the options: - Option 1: - RAN4 shall extend the delay for partially overlapped simultaneous Pre-MGs activation/deactivation, when multiple Pre-MGs activation/deactivation processes are overlapped in time, until the end of the last Pre-MG activation/deactivation duration + T2. Where the value of T2 is FFS. - Option 3: - For partially overlapping simultaneous activation/deactivation of two Pre-MGs due to different events, RAN4 shall discuss necessity of defining requirement. The following two options can be considered: - only clarify in spec that extra delay can be expected in high-level. - the completion of activation/deactivation of the first Pre-MG is extended to the end of completion of activation/deactivation of the second Pre-MG. - Option 4: - Since the UE capability of supporting simultaneous pre-MG multiple activation/deactivation procedure has been identified, for the UE capable of such capability, the UE is capable to perform multiple activation/deactivation procedure in parallel, so we prefer to reuse the R17 activation/deactivation switching delay.

For scenario #1 i.e. fully overlap, we understand both cases can be considered and the typical event is BWP switching. To define the requirements for this scenario, the Rel-17 requirements can be the baseline. In R17, the (de)activation delay is generally defined as the sum of the time for trigger event and 5ms which is used for UE preparation and implementation margin. For case a, the status changes of both Pre-MGs are triggered by the same event. When defining the delay requirements, the time for trigger event is based on the BWP switching on single CC as defined in clause 8.6.2 and 8.6.3, and no additional preparation time is needed, i.e. Rel-17 (de)activation delay requirements can be reused. For case b, the typical case is DCI, timer or RRC based active BWP switching on multiple CCs, in this case, the time for trigger event should be based on the requirements in section 8.6.2A, 8.6.2B and 8.6.3A which means the trigger event can be regarded as one and is equivalent to case a. So the requirements for case a can be reused.

For scenario #2 i.e. partially overlap, we think all the cases can be deprioritized since they are not typical cases. For example case a, c and d, the first trigger would cause interruption on activated serving cell and it is not typical to configure another trigger event during that time. For case b, theoretically, it may be possible to configure different trigger events at the same time, but we are not clear about the use cases. So we prefer to deprioritize the scenario #2. And even if scenario #2 is considered, the same principle for defining the requirements should be used for all the cases since they are same in essence. And in our understanding, since the support of simultaneous pre-MG (de)activation is up to UE capability, for the UE capable of such capability, the UE is capable to perform multiple activation/deactivation procedure in parallel, so we prefer to reuse the R17 activation/deactivation switching delay if scenario #2 is considered.

Proposal 2: For fully overlapped simultaneous Pre-MGs (de)activation, the existing Rel-17 Pre-MG (de)activation delay requirements (time of trigger event plus 5ms) can be reused for each Pre-MG, i.e.T1 = 0.

Proposal 3: For partially overlapped simultaneous Pre-MGs (de)activation, prefer to deprioritize all the cases. And even considered, the existing Rel-17 Pre-MG (de)activation delay requirements (time of trigger event plus 5ms) can be reused for each Pre-MG.

Dynamic collision:

- Issue 3-3-1: [Case 1] Required changes for Pre-MG on collision < Way forward >: - Option 1: RAN4 shall stick to agreed baseline that collision and priority rule on Pre-MG are considered only when Pre-MG is activated (deactivated Pre-MG is not considered in collisions). - Option 2: De-activated pre-MG is considered in collisions handling.

It has been agreed in previous meeting that only activated Pre-MG is considered when defining collision between components of concurrent gap. But companies point out that this will cause dynamic collisions. In our understanding, this should be OK since UE behavior is clear when there is collision and we understand the Pre-MG (de)activation is not a very frequent procedure. If the deactivated Pre-MG with higher priority is also considered in the collision, then the other MG with lower priority will never be used which results in that the associated measurement cannot be performed. On the other hand, when the Pre-MG is deactivated, UE behavior is totally same as that no gap, it is unreasonable to drop the nearby MG. So we think the previous agreement should be stuck to. And with clear UE behavior defined as the following issues, we think no need to define the additional UE capability for dynamic collisions.

Proposal 4: Stick to the previous agreement that only activated Pre-MG is considered when defining gap collision.

Proposal 5: No need to define additional UE capability for dynamic collisions.

Overlapping between activation procedure and activated MG:

In last meeting, the issue on UE behavior when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion was further discussed and still has no consensus.

- Issue 3-3-2: [Case 1] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG) < Way forward >: - FFS the following options: - Option 1: - A collision between a change in the status of a pre-configured MG and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG. (UE shall extend the activation procedure) - Option 1a: - If a change in the status of a pre-configured MG collides with a gap instance, the change in status is delayed by (MGL of the gap instance plus 5 ms) to avoid the collision. - Option 2: - For some gap patter configuration, for example when the current gap pattern is gap pattern 4 (MGL = 6ms and MGRP = 20ms), after the activation of Pre-MG, that Pre-MG occasion will collide with the next gap occasion of the concurrent gap and gaps will be dropped based on priority rule. Clarifications may need since for a particular window W there is an extra collision due to the extension of the activation procedure and the collision may not happen again for other time window W. - Option 3: - Alternatively, when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion, the Pre-MG status shall not be changed immediately and its status shall be changed prior to the next gap occasion. - Option 4: - When Pre-MG activation/deactivation period collides Type-2 gap, - during Pre-MG activation/deactivation period, - the UE is not required to receive or transmit in the corresponding NR serving cells in the Pre-MG occasions. - gap dropping rule won’t be applied. - how to use such gap occasion is up to UE implementation. - the gap dropping rule will be re-applied after the 1st effective MG occasion after Pre-MG activation. - Option 5: - If the priority of pre-MG is higher than the other MG, applying priority rule, i.e. pre-MG activation/deactivation procedure prioritizes the other MG occasion, so the other MG occasion is dropped; - then the MO(s) associated with this MG accordingly be canceled at this MG occasion. - Option 6: - If two MG occasions collide, one of the two MGs is pre-MG and the pre-MG has higher priority, then UE is allowed to keep or drop any of the two occasions if - the (de)activation procedure of pre-MG ends earlier than the start of pre-MG occasion, and - the (de)activation procedure of pre-MG overlaps with time period T, where T starts from 4ms before the other MG occasion and ends at 4ms after the other MG occasion - Option 7: - Agree on option 2 for issue 3-3-1 and close this issue.

Firstly, we need to have a common understanding on the overlapping definition. In R17, the two gaps are considered as overlapping when the distance between the end of the first gap and the start of the second gap is smaller than 4ms. Similarly, the overlapping between activation procedure and activated gap can use the same condition, i.e. the (de)activation procedure and activated gap instance can be considered as overlapping when the activation occurs < 4 ms before the start or < 4 ms after the end of a gap instance of an activated concurrent MG.

Then for the UE behavior, we think we have two logics to follow. The first approach is to define the UE behavior based on gap priority, i.e. to drop the gap occasions with lower priority. If the activating Pre-MG has higher priority, UE drop the other gap occasion and complete the Pre-MG activation. Otherwise, UE drop the activation procedure and continue measurement based on the other gap occasion. The second approach is to guarantee the ongoing measurement since the Pre-MG doesn’t take effect. In this case, UE will drop or extend the activation procedure and continue the ongoing measurement based on the other gap occasion.

Between the above two approaches, we think the second one is more reasonable. On the other hand, in last meeting, it was suggested that the gap collision dropping rule can be applied directly when the MG has higher priority than the Pre-MG. But after further thinking, we understand that the activation delay is overlapped with activated concurrent MG doesn’t mean the two gaps are overlapped, and it is possible that the two gaps are not overlapped. In that case, the Pre-MG should be activated and no dropping rules would apply. So we think no matter the Pre-MG has higher or lower priority, the activation delay should be extended and the priority rules apply after the Pre-MG is activated if overlapping. So we suggest to define the UE behavior based on second approach, i.e. UE drop or extend the activation procedure and continue the measurement based on the other gap occasion.

Proposal 6: The activation procedure and activated gap instance can be considered as overlapping when the activation occurs < 4 ms before the start or < 4 ms after the end of a gap instance of an activated concurrent MG.

Proposal 7: UE continue the measurement based on the activated gap instance and extend the activation procedure when overlapping no matter the Pre-MG has higher or lower priority.

Measurement delay requirements:

- Issue 3-5-1: [Case 1] Measurement delay requirements < Wayforward >: - FFS the options: - Option 1: - When gap combinations including pre-configured MGs (Case 1) are provided to the UE, measurement requirements do not apply if the following parameters change during the measurement period due to changes in the status of any pre-configured MGs: - Kp for intra-frequency and inter-frequency measurements without gaps - Kgap for intra-frequency and inter-frequency measurements with gaps - Kgap_EUTRA for inter-RAT measurements - Kp_CSI-RS for CSI-RS L3 measurements - Kp,PRS,i for NR positioning measurements - CSSFintra for intra-frequency measurements - CSSFinter for intra-frequency measurements - CSSFinterRAT for intra-RAT measurements - P scaling factor for L1-RSRP and L1-SINR measurements - Option 2: - R17 measurement delay requirements for concurrent MG can be reused for case 1 without any status change. - Option 3: - RAN4 to investigate whether measurement requirements in case of changes of Pre-MG status are based on more relaxed requirements between both Pre-MG status.

For the measurement requirements, we need to consider two aspects, one is the requirements when Pre-MG is activated and the second is the requirements applicability when status of Pre-MG is changed during the measurement period. In Rel-17, the measurement requirements for concurrent gaps and for Pre-MG are defined individually, and the measurement requirements for Pre-MG are defined when there is no status change. We think the same principle can also be used for Rel-18, i.e. the requirements are defined when Pre-MGs are configured as the components of concurrent and are activated. Since we don’t consider status change for Pre-MG, there is no need to consider delayed Pre-MG activation in the measurement delay and the delay requirements don’t apply when there is status change during the measurement period.

Proposal 8: Rel-17 measurement delay requirements for concurrent MG can be reused for case 1 without any status change of Pre-MG.

Proposal 9: The measurement delay requirements don’t apply when there is status change of Pre-MG during the measurement period.

Proposal 10: No need to consider the delayed Pre-MG activation in measurement requirements.

3 Summary

In this paper, we have some further discussions on the case 1  requirements for the combination of Pre-MG and concurrent MG, and the following proposals are given：

Proposal 1: Prefer option 1 not to consider Type-1 MG with Pre-MG/NCSG in the WI, but can compromise to option 2a to move forward.

Proposal 2: For fully overlapped simultaneous Pre-MGs (de)activation, the existing Rel-17 Pre-MG (de)activation delay requirements (time of trigger event plus 5ms) can be reused for each Pre-MG, i.e.T1 = 0.

Proposal 3: For partially overlapped simultaneous Pre-MGs (de)activation, prefer to deprioritize all the cases. And even considered, the existing Rel-17 Pre-MG (de)activation delay requirements (time of trigger event plus 5ms) can be reused for each Pre-MG.

Proposal 4: Stick to the previous agreement that only activated Pre-MG is considered when defining gap collision.

Proposal 5: No need to define additional UE capability for dynamic collisions.

Proposal 6: The activation procedure and activated gap instance can be considered as overlapping when the activation occurs < 4 ms before the start or < 4 ms after the end of a gap instance of an activated concurrent MG.

Proposal 7: UE continue the measurement based on the activated gap instance and extend the activation procedure when overlapping no matter the Pre-MG has higher or lower priority.

Proposal 8: Rel-17 measurement delay requirements for concurrent MG can be reused for case 1 without any status change of Pre-MG.

Proposal 9: The measurement delay requirements don’t apply when there is status change of Pre-MG during the measurement period.

Proposal 10: No need to consider the delayed Pre-MG activation in measurement requirements.

4 References

- R4-2306330, WF on Rel-18 NR MG enhancements (part 1), MTK, RAN4#106bis-e

- R4-2306237, Topic summary for [106-bis-e][206] NR_MG_enh2_part1, MTK, RAN4#106bis-e