3GPP TSG-RAN4 Meeting #107	                                                            R4-2307417

Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on RRM requirements of Carrier Phase Positioning

Source:	CATT

Agenda Item:	8.23.3.6

Document for:	Discussion

1 Introduction

In last meeting, RAN4 has discussed general RRM impact of carrier phase positioning with conclusions captured in [1]. RAN1#112bis-e meeting has also discussed the work item and the agreements were captured in [2]. In this paper, we further discuss the potential RRM requirements for carrier phase positioning based on RAN1 progress.

2 Discussion

In RAN1#112bis-e meeting, RAN1 has agreed to introduce DL RSCP and DL RSCPD as DL carrier phase measurements, so RAN4 should define measurement requirements for these two measurement types.

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.

Proposal 1: RAN4 to define measurement period requirements, accuracy requirements and report mapping for DL RSCP and DL RSCPD measurements.

So far, RAN1 only agreed the cases when DL RSCP can be reported together with UE Rx – Tx time difference measurement and DL RSCPD can be reported together with RSTD measurement.

For DL RSCP measurement, when it is reported together with UE Rx-Tx time difference measurement performed on single carrier, the measurement requirements for DL RSCP should be the same as that for UE Rx-Tx time difference, i.e. the existing measurement requirements for UE Rx-Tx time difference can be reused.

When DL RSCP is reported together with UE Rx-Tx time difference measurement performed on multiple carriers, i.e. DL RSCP is reported on one of the carriers, the measurement requirements for DL RSCP should be same as the measurement delay for UE Rx-Tx time difference measurement on carrier i.

In summary, for both cases, the measurement requirements for DL RSCP are the same as the existing UE Rx-Tx time difference measurement requirements for single carrier.

Similarly, for DL RSCPD measurement, when it is reported together with RSTD measurement, the measurement requirements for RSTD can be reused. And the mobility impact in RRC_CONNECTED and the collision in RRC_INACTIVE state can also be reused

Based on RAN1 agreement, UE measurement without measurement in RRC_CONNECTED state is still FFS and the indicated time window for DL measurements is also FFS, so the measurement requirements for measurement without gap in RRC_CONNECTED should be postponed and wait for RAN1 progress. And the existing measurement requirements with gap in RRC_CONNECTED state and the measurement in RRC_INACTIVE state can be reused.

Proposal 2: When DL RSCP is reported together with UE Rx–Tx time difference measurement, the existing Rel-17 UE Rx-Tx time difference measurement requirements for single carrier can be reused including the requirements for measurement with gap in RRC_CONNECTED and the measurement in RRC_INACTIVE.

Proposal 3: When DL RSCPD is reported together with RSTD measurement, the existing Rel-17 RSTD measurement requirements for single carrier can be reused including the requirements for measurement with gap in RRC_CONNECTED and the measurement in RRC_INACTIVE.

Proposal 4: The mobility impact in RRC_CONNECTED and the collision handling in RRC_INACTIVE state in Rel-17 can also be reused.

Agreement To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_INACTIVE state

Based on RAN1 agreement, both UE and PRU can be requested to perform the measurements on indicated DL PRS resources within dedicated time window. So we understand the requirements defined in RAN4 should apply to both UE and PRU(s). But considering the better implementation of PRU, it can be further studied in performance part whether to define better accuracy for PRU.

Proposal 5: The measurement period requirements defined in RAN4 apply to both UE and PRU(s). Further discuss in performance part whether to define higher accuracy requirements for PRU.

For the side condition of CPP measurement, the Rel-17 condition for AWGN can be reused. For fading channel, the side condition can be further studied after the channel models are decided. If using the same channel model as Rel-17, the side condition can also be reused. Also since the same reference signals are used for CPP measurement and the measurements are reported together with legacy measurements, we understand the Rel-17 side conditions and PRS configurations can be reused.

Proposal 6: The Rel-17 side condition and PRS configurations for UE Rx-Tx time difference can be reused for DL RSCP measurement.

Proposal 7: The Rel-17 side condition and PRS configurations for RSTD can be reused for DL RSCPD measurement.

For the report mapping, mainly three aspects need to be considered, the reporting range, unit and resolution. For carrier phase measurement, the reporting range is clear, [0, 2*pi). And the unit can be degree or radian. Since the two units are corresponding to each other, we understand it should be OK using either of them. To align the unit for angle with existing requirements e.g. AOA/ZOA, we use degree as the unit of carrier phase. For the resolution, it can be further decided after performance evaluation.

Proposal 8: The reporting range for DL RSCP and RSCPD is [0, 360) degree. And the resolution can be further decided after performance evaluation.

3 Summary

In this paper, we discuss the RRM aspects on carrier phase positioning and the following proposals are given:

Proposal 1: RAN4 to define measurement period requirements, accuracy requirements and report mapping for DL RSCP and DL RSCPD measurements.

Proposal 2: When DL RSCP is reported together with UE Rx–Tx time difference measurement, the existing Rel-17 UE Rx-Tx time difference measurement requirements for single carrier can be reused including the requirements for measurement with gap in RRC_CONNECTED and the measurement in RRC_INACTIVE.

Proposal 3: When DL RSCPD is reported together with RSTD measurement, the existing Rel-17 RSTD measurement requirements for single carrier can be reused including the requirements for measurement with gap in RRC_CONNECTED and the measurement in RRC_INACTIVE.

Proposal 4: The mobility impact in RRC_CONNECTED and the collision handling in RRC_INACTIVE state in Rel-17 can also be reused.

Proposal 5: The measurement period requirements defined in RAN4 apply to both UE and PRU(s). Further discuss in performance part whether to define higher accuracy requirements for PRU.

Proposal 6: The Rel-17 side condition and PRS configurations for UE Rx-Tx time difference can be reused for DL RSCP measurement.

Proposal 7: The Rel-17 side condition and PRS configurations for RSTD can be reused for DL RSCPD measurement.

Proposal 8: The reporting range for DL RSCP and RSCPD is [0, 360) degree. And the resolution can be further decided after performance evaluation.

4 References

- R4-2306351 WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT, RAN4#106bis-e

- Draft Report of 3GPP TSG RAN WG1 #112bis-e v0.1.0

Annex RAN1 agreements on CPP

RAN1#112bis-e

Agreement

Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements.

- Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note.

- DL RSCP can be reported together with UE Rx – Tx time difference measurement

- DL RSCPD can be reported together with RSTD measurement

- FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed

- Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.

Agreement

Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113).

- Option 1:

  - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for frequency range 1 and frequency range 2.

  - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for frequency range 1 and frequency range 2.

  - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting.

- Option 2:

  - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2.

  - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF

  - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame

Agreement

To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements:

- Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s).

  - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time

- Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s).

  - Note: this may be a different indicated time window

Agreement

To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements:

- Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s).

- FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time.

Agreement

Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including

- UE in RRC_CONNECTED state with measurement gap.

- FFS: UE in RRC_CONNECTED state without measurement gap

- UE in RRC_INACTIVE state

Agreement

The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default.

- Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them.

Agreement

The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose.

- Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them.

Agreement

- Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF

- Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference.

- Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting.

Agreement

Further study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements.

Agreement

For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP).

- FFS: the details of the enhancements.

Agreement

Adopt one of the following options for a timestamp associated with a reported RSCP/RSCPD measurement (make the decision in RAN1#113):

- Option 1:

  - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp with the granularity of a slot.

  - FFS: Whether to clarify in the specification the reported RSCP/RSCPD value presents the RSCP/RSCPD of a specific OFDM symbol within the slot identified by the NR-TimeStamp.

- Option 2:

  - NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements.

Agreement

To address the impact of the phase delays on Tx/Rx RF chains, support one or more of the following options (down-selection in RAN1#113):

- Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals

  - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs.

  - FFS: the details of \the UE/TRP Tx/Rx PEGs

- Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals.

  - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs

  - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID.

- Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals.

  - The transmission/reception associated with the same ARP ID is assumed from the same ARP.

  - FFS: the maximum number of ARP IDs.

- Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value.

- Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.