3GPP TSG-RAN4 Meeting #107	R4-2307421

# Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.171 | CR | 0029 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS 36.171 requirements for relative signal power levels of BDS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CAICT, CENC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Based on the BDS ICD reference, the relative signal power level for B1C signal in TS 36.171 is incorrect. - The B1C signal power levels for MEO satellite and IGSO satellite are different. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the relative signal power levels for B1C signal in TS 36.171. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The signal power levels for BDS signal in TS 36.171 is not aligned with BDS ICD reference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 4.8	UEs supporting multiple signals

For UEs supporting multiple signals, different minimum performance requirements may be associated with different signals. The satellite simulator shall generate all signals supported by the UE. Signals not supported by the UE do not need to be simulated. The relative power levels of each signal type for each GNSS are defined in Table 4.1. The individual test scenarios in clause 6 define the reference signal power level for each satellite. The power level of each simulated satellite signal type shall be set to the reference signal power level defined in each test scenario in clause 6 plus the relative power level defined in Table 4.1.

Table 4.1: Relative signal power levels for each signal type for each GNSS

|  | Galileo |  |  | GPS/Modernized GPS |  |  | GLONASS |  |  |  | QZSS |  |  |  | SBAS |  |  |  | BDS |  |  |  | NavIC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal power levels relative to reference power levels | E1 |  | 0 dB | L1 C/A | 0 dB |  | G1 |  | 0 dB |  | L1 C/A |  | 0 dB |  | L1 |  | 0 dB |  | B1I |  | D1 | 0 dB | L5 |  | 0 dB |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | D2 | +5 dB |  |  |  |
|  | E6 | +2 dB | L1C | +1.5 dB | G2 | -6 dB | L1C | +1.5 dB |  |  | B1C | DMEO | +4 dB |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | DIGSO | +2dB |  |  |
|  | E5 |  | +2 dB | L2C | -1.5 dB |  |  |  |  |  | L2C |  | -1.5 dB |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | L5 | +3.6 dB |  |  |  |  |  | L5 |  | +3.6 dB |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE 1:	For test cases which involve “Modernized GPS”, the satellite simulator shall also generate the GPS L1 C/A signal if the UE supports “GPS” in addition to “Modernized GPS”.

NOTE 2:	The signal power levels in the Test Parameter Tables represent the total signal power of the satellite per channel not e.g. pilot and data channels separately.

NOTE 3:  For test cases which involve "BDS", D1 represents MEO/IGSO satellites for B1I signal types and D2 represents GEO satellites for B1I signal type.

NOTE 4:	For test cases involving NavIC all satellites are either GEO or IGSO as per ICD [20]

NOTE 5: For test cases which involve "BDS", DMEO represents MEO satellites for B1C signal types and DIGSO represents IGSO satellites for B1C signal type.

<End of Change 1>