3GPP TSG-RAN WG4 Meeting#107						R4-2307436

22nd May, 2023 – 26th May, 2023

Source: 	Qualcomm Incorporated

Title: 	NTN NB-IoT/eMTC demodulation performance requirements

Agenda item:	 9.7.7.1

Document for:	Approval

- Introduction

In this paper, simulation results of agreed test cases in [1] of both NBIoT and CATM1 are shared for alignment purpose.

- NBIoT NTN Scenarios

Scenarios in the following table were considered in [1]. All numbers provided here are with 0.1 ppm maximum residual frequency error.

Table 2.1

| Test number | CBW | Carrier type | MCS | NPDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 200kHz | Anchor | QPSK 1/2 (Reuse R.NB6.FDD) | 32 | NTN-TDLC5-200 | 1x1 | 8ms | -8.7 |
| 2 | FDD 200kHz | Non-anchor | QPSK 1/3 (Reuse R.NB6-1.FDD) | 128 | NTN-TDLA100-10 | 1x1 | 8ms | -13.1 |

Proposal 1: NBIoT: Propose SNR numbers in the Table 2.1 for alignment purpose

- eMTC Scenarios

Proposal 3: eMTC: Propose SNR numbers in the Table 3.1 for alignment purpose

The shared numbers are from simulation results with 0.1 ppm maximum residual frequency error.

Table 3.1

| Test number | CBW | MCS | Transmission mode | PDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 1.4MHz | 16QAM 1/2 | TM1 | 1 | NTN-TDLC5- 30 | 1x1 | 8ms | 7.1 |
| 2 | FDD 1.4MHz | QPSK 1/3 | TM1 | 8 | NTN-TDLA100-200 | 1x1 | 8ms | -6.3 |
| 3 | FDD 1.4MHz | QPSK 1/10 | TM1 | 64 | NTN-TDLA100-10 | 1x1 | 8ms | -13.5 |

- Conclusions

Following is the summary of the proposals.

Proposal 1: NBIoT: Proposed SNR numbers in Table 2.1 for alignment purpose

Proposal 2: eMTC: Proposed SNR numbers in Table 3.1 for alignment purpose

# References

[1] R4-2302932, “WF for IoT over NTN UE demodulation requirements”, #106, Feb 2023.

[2] R4-2305960, “WF on IoT-NTN UE demodulation requirements”, #106_bis_e, April 2023.