3GPP TSG-RAN WG4 Meeting#	107                                            R4-2307462

Incheon, Korea, May 22 – May 26,2023

Agenda item:		8.29.5

Source:	Dell Technologies

Title:	Discussion on NCR RRM requirements

Document for:	Discussion

# Introduction

Way Forward/Agreements:

Issue 4-1: adaptive beamforming for NCR-MT

- Agreements

    - Do not define TCI switching and uplink spatial relationship requirements for LA NCR-MT

Issue 4-2: adaptive beamforming for NCR access link

- Agreements

    - FFS whether to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

Issue 5-1:  BFD/BFR/RLM for NCR-MT

- Agreements

    - For LA NCR-MT

      - Define RLM/BFD/BFR requirements

    - For WA NCR-MT

    - Do not define BFD/BFR requirements

    - FFS for RLM requirements.

In RAN4-#106 bis-e meeting, the WFs for NCR RRM requirements, R4-230630 [1], is approved. In this contribution, the adaptive beamforming for NCR access link and BFD/BFR/RLM for NCR-MT are discussed with our views based on the WF.

- Discussions

  - Issue 4-2 Adaptive beamforming for NCR access link

For adaptive beamforming on NCR-Fwd access link, we consider that:

- Since the NCR-Fwd is functionalized only to perform the amplify and forwarding of UL/DL signals between gNB and UE with backhaul/access links and controlled by NCR-MT with received control information [2].

- For NCR access link, the beamforming function is configurated by gNB and the service beams are usually switched where the caused limited beam configuration/switching delay can be ignored.

Proposal 1: The RRM requirements for adaptive beamforming on NCR-Fwd access link shall not be defined.

  - Issue 5-1 BFD/BFR/RLM for WA NCR-MT

In last meeting, it has been agreed that define BFD/BFR/RLM requirements for LA NCR-MT and not define BFD/BFR requirements for WA NCR-MT. For WA NCR-MT RLM requirements, our view is based on:

Observation 1: Since NCR is fixed or nomadic deployed, the radio links for NCR-MT C-link and NCR -Fwd backhaul link will be remained comparatively stable for longer periods of time, particular in WA NCR.

Observation 2: Beam and radio link failures are not typical happened in stable links for WA NCR. It is no need for NCR-MT to continuously monitor link quality based on SSB or CSI-RS measurements.

Proposal 2: It is not necessary to define BFD/BFR/RLM requirements for WA NCR-MT.

- Conclusions

In this contribution, we provide our views on NCR RRM requirements for adaptive beamforming on NCR-Fwd access link and for BFD/BFR/RLM in WA NCR-MT. From the discussion, we have presented our observations and proposals as followings:

Proposal 1: The RRM requirements for adaptive beamforming on NCR-Fwd access link shall not be defined.

Observation 1: Since NCR is fixed or nomadic deployed, the radio links for NCR-MT C-link and NCR -Fwd backhaul link will be remained comparatively stable for longer periods of time, particular in WA NCR.

Observation 2: Beam and radio link failures are not typical happened in stable links for WA NCR. It is no need for NCR-MT to continuously monitor link quality based on SSB or CSI-RS measurements.

Proposal 2: It is not necessary to define BFD/BFR/RLM requirement in WA NCR-MT.

- References

[1] R4-230630 WF on NCR-MT RRM requirements.

[2] 3GPP TR38.867 Study on NR network-controlled repeater.