3GPP TSG-RAN WG4 Meeting# 107	                                            R4-2307463

Incheon, Korea, May 22 – May 26,2023

Agenda item:		8.29.1.2

Source:	Dell Technologies

Title:	Discussion on NCR feature list and class declaration

Document for:	Discussion

# Introduction

Way Forward/Agreements

Issue 1-2: NCR-MT feature list

- Agreement:

  - Further discuss NCR-MT feature list, FFS whether feature list required or not for NCR-fwd from RAN4 perspective

  - Proposal 1: to focus on the mandatory UE capability firstly and to check whether this is still applicable for NCR-MT and identify some features which are not applicable for NCR-MT; [for the rest of UE features, it should be optional for NCR-MT].

Issue 1-3-1: NCR Class

- Agreement:

  - For NCR-fwd, following NCR classes introduced

    - DL/UE side: wide-area, medium range and local area

    - UL/BS side: Wide-area and local area

    - Above NCR classes applicable for all NCR types

  - NCR classes for NCR-MT

    - [Wide-area and local area]

  - FFS whether NCR class can be declared separately for NCR-fwd, and NCR-MT

In RAN4-#106bis-e meeting, the WF for NCR RF general requirements, R4-2305931 [1], is approved. In this contribution, the NCR-MT feature list and NCR class are discussed with our views based on the WF.

- Discussions

  - Issue 1-2: NCR-MT feature list

Since NCR-MT is functionalized similar as IAB-MT and works similar as UE, the feature list and requirements for IAB-MT may be reused for NCR-MT. It may take Rel-15 UE mandatory feature list [2] as starting point as discussed in the meeting of R4-106bis-e and study optional features based on Rel-16/Rel-17.

For the feature list of NCR- Fwd, it may not need to be considered in RAN4 since NCR-Fwd link transmission is controlled by NCR-MT.

Proposal 1: It is suggested that there is no need to require NCR-Fwd feature list in RAN 4.

  - Issue 1-3-1 NCR Class

As described in 3GPP TR38.867, the NCR-MT is defined as a function entity to communicate with a gNB via a Control link (C-link) to enable exchange of control information (e.g. side control information at least for the control of NCR-Fwd). The C-link is based on NR Uu interface. The NCR-Fwd is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB. Additionally, at least one of the NCR-MT's carrier(s) should operate in the frequency band forwarded by the NCR-Fwd and the NCR-MT and NCR-Fwd operating in the same frequency band is prioritized for the study [3].

For NCR class, it has been agreed in meeting of RAN4-106bis-e as followings:

- For NCR-Fwd, there are wide-area, medium range, and local area in DL/UE side and there are wide-area, and local area in UL/BS side.

- For NCR-MT, there are wide-area and local area.

Observation 1: Based on TR38.867, it is suggested NCR-MT and NCR-Fwd operating at the same frequency band (FR1 or FR2) as the priority. In FR1, it can be wide area /medium range/local area and in FR2, it is usually local area.

Proposal 2: It is suggested that, in FR1, NCR-MT, NCR-Fwd BS side and NCR-Fwd UE side class shall be declared independently whereas in FR2, the class for NCR-MT, NCR-Fwd BS side and NCR-Fwd UE side can be the same declaration.

- Conclusions

In this contribution, we provide our views on NCR feature list and NCR class declaration and present our observation and proposals as followings:

Proposal 1: It is suggested that there is no need to require NCR-Fwd feature list in RAN 4.

Observation 1: Based on TR38.867, it is suggested NCR-MT and NCR-Fwd operating at the same frequency band (FR1orFR2) as the priority. In FR1, it can be wide/medium/local area and in FR2, it is usually local area.

Proposal 2:  It is suggested that, in FR1, NCR-MT, NCR-Fwd BS side and NCR-Fwd UE side class shall be declared independently whereas in FR2, the class for NCR-MT, NCR-Fwd BS side and NCR-Fwd UE side can be the same declaration.

- References

[1] R4-2305931 WF on system parameters and RF requirements

[2] 3GPP TR38.822 User Equipment (UE) feature list

[3] 3GPP TR38.867 Study on NR network-controlled repeater