3GPP TSG-RAN WG4 Meeting #107 	R4-2307467

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Advanced Receivers Blind Detection - Simulation results

Agenda item:	9.19.1.1

Document for:	Discussion

Introduction

This document details Nokia’s current simulation results for alignment on the WID for advanced receivers for MU-MIMO scenarios with blind detection of interference parameters. The following main agreements were established in RAN4#106-bis [1] for facilitating companies to provide simulation results for Phase I.

Simulation assumptions for Phase I

Table 1 provides an overview of the already agreed simulation assumptions for Phase I taken from the WF [1]:

Table 1: Agreed simulation assumptions for Phase I

| Reference receiver for BD simulation | R-ML |
| --- | --- |
| Rank allocation for the target and co-scheduled UEs, with 1 co-scheduled UE | 2Rx UE: 1+1 4Rx UE: 2+2 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - Use the same CDM group for rank 1+1 |
| Antenna configuration | For rank 1+1: cover 2T2R For rank 2+2: 4T4R |
| Channel model | Use TDLC300-100 when the rank of the target UE is 1 Use TDLA30-10 when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium Rank 2+2: ULA Low |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 Random PMI selection for the target UE Cover both orthogonal and random PMI selection (same as Rel-17 approach) for the co-scheduled UE in phase I |
| Transmission PDSCH power ratio of co-scheduled UEs | 0 dB : Precoder scaling factor sqrt(1/2) 3 dB: Precoder scaling factor sqrt(2/3), sqrt(1/3) |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | 1 co-scheduled UE |
| DMRS sequence for the co-scheduled UE | For BD simulations, assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s) |
| MCS for the target UE | MCS 13 for rank 1 and rank 2 |
| Modulation order for the co-scheduled UE | For rank 1+1: QPSK (high priority for the next meeting) For rank 2+2: 64QAM (high priority for the next meeting) For rank 2+2: QPSK (high priority for the next meeting) |
| PDSCH resource allocation for the target and co-scheduled UE | Maximum transmission bandwidth for target UEs. Co-scheduled UE bandwidth as per test. Full OFDM symbol allocation |
| Assumptions on the required information | Assume the needed parameters of the co-scheduled UE are all known to UE (upper bound for the potential performance gain). |
| MCS | Table 1 |
| ZP-CSI-RS configuration | Single port, density 0.5, l_0=3, k_0=0, full CHBW, periodicity every MU PDSCH slot |

Based on the information in Table 1, a blind detection study of interference is pursued with 1 co-scheduled UE for the following parameters:

- Presence of co-scheduled UE (limited to 1 interference UE).

- DMRS port and sequence detection (DMRS det): DMRS ports 1000 to 1003 and DMRS 0 or 1.

- Modulation order detection (MO det).

- FDRA detection (FDRA det): full and partial CHBW.

We provide the simulation results for several relevant cases, summarized in Table 2. These cases vary, for the agreed Rank 1+1 and Rank 2+2 scenarios, the power ratio between the target UE and co-scheduled UE, as well as their modulation order and number of blind detected parameters.

Table 2 - Simulation Overview

| Case | # Co-scheduled UE | Target to interference UE, PDSCH ratio (dB) | Rank target UE | Rank Co-scheduled UE | Modulation order target UE | Modulation order co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE | BD parameters |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | DMRS & MO det | FDRA & DMRS & MO det |
| 1 | 1 | 0 | 1 | 1 | MCS13 | MCS4 | 2Tx 2Rx ULA medium | TDLC300-100 | random | X | X |
| 2 | 1 | 0 | 1 | 1 | MCS13 | MCS13 | 2Tx 2Rx ULA medium | TDLC300-100 | random | X | X |
| 3 | 1 | 3 | 1 | 1 | MCS13 | MCS4 | 2Tx 2Rx ULA medium | TDLC300-100 | random | X | X |
| 4 | 1 | 3 | 1 | 1 | MCS13 | MCS13 | 2Tx 2Rx ULA medium | TDLC300-100 | random | X | X |
| 5 | 1 | 0 | 2 | 2 | MCS17 | MCS13 | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal | X |  |
| 6 | 1 | 0 | 2 | 2 | MCS13 | MCS19 | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal |  | X |
| 7 | 1 | 3 | 2 | 2 | MCS13 | MCS19 | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal |  | X |

Blind Detection Simulation Results

This section portrays the impact of aided interference parameter detection on the throughput as compared to a genie R-ML receiver (Genie), with full knowledge of interference parameters, captured at 70% maximum throughput point. These are organized into two sets of results: the results for interference presence, DMRS ports and sequence and MO blind detection, and the the results for interference presence, FDRA, DMRS ports and sequence and MO blind detection.

Interference presence, DMRS ports, sequence, MO Detection

Table 3 shows the results of the RAN4#106bis agreed test cases assuming interference presence, DMRS ports and sequence and MO blind detection Figure 1 to Figure 6 display the throughput curves for all covered scenarios.

Table 3 ZP-CSI-RS aided interference parameter detection, without FDRA detection

| Case | # Co-scheduled UE | Target to interference UE, PDSCH ratio (dB) | Rank target UE | Rank Co-scheduled UE | Modulation order target UE | Modulation order co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE | SNR@70% (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Genie | Max MO 64 QAM | Max MO 256 QAM |
|  |  |  |  |  |  |  |  |  |  |  | Aided detection | Aided detection |
| 1 | 1 | 0 | 1 | 1 | MCS13 | MCS4 | 2Tx 2Rx ULA medium | TDLC300-100 | random | 14.23 | 14.61 | 14.84 |
| 2 | 1 | 0 | 1 | 1 | MCS13 | MCS13 | 2Tx 2Rx ULA medium | TDLC300-100 | random | 18.19 | 18.28 | 18.53 |
| 3 | 1 | 3 | 1 | 1 | MCS13 | MCS4 | 2Tx 2Rx ULA medium | TDLC300-100 | random | 13.71 | 14.37 | 13.98 |
| 4 | 1 | 3 | 1 | 1 | MCS13 | MCS13 | 2Tx 2Rx ULA medium | TDLC300-100 | random | 16.86 | 17.20 | 17.11 |
| 5 | 1 | 0 | 2 | 2 | MCS17 | MCS13 | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal | 16.24 | 16.47 | 16.64 |

| Figure 1 – Case 1 vs. Case 3 (Max MO 64 QAM) | Figure 2 – Case 1 vs. Case 3 (Max MO 256 QAM) |
| --- | --- |
| Figure 3 – Case 2 vs. Case 4 (Max MO 64 QAM) | Figure 4 – Case 2 vs. Case 4 (Max MO 256 QAM) |
| Figure 5 – Case 5 (Max MO 64 QAM) | Figure 6 – Case 5 (Max MO 256 QAM) |

Interference presence, FDRA, DMRS ports, sequence, MO Detection

Table 4 shows the results of the RAN4#106bis agreed test cases assuming interference presence, FDRA, DMRS ports and sequence and MO blind detection. Figure 7 to Figure 11 display the throughput curves for all covered scenarios.

While our ZP-CSI-RS aided interference parameter detection is based on 3 possible interference scenarios (no interference, full CHBW interference and partial CHBW interference),Table 5 portrays only the last two scenarios.

Table 5 ZP-CSI-RS aided interference parameter detection, with FDRA detection

| Case | # Co-scheduled UE | Target to interference UE, PDSCH ratio (dB) | Rank target UE | Rank Co-scheduled UE | Modulation order target UE | Modulation order co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE | SNR@70% (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Full CHBW |  | Partial CHBW |  |
|  |  |  |  |  |  |  |  |  |  | Genie | Aided detection | Genie | Aided detection |
| 1 | 1 | 0 | 1 | 1 | MCS13 | MCS4 | 2Tx 2Rx ULA medium | TDLC300-100 | random | 14.22 | 14.64 | 12.26 | 12.97 |
| 2 | 1 | 0 | 1 | 1 | MCS13 | MCS13 | 2Tx 2Rx ULA medium | TDLC300-100 | random | 18.19 | 18.59 | 14.09 | 14.65 |
| 3 | 1 | 3 | 1 | 1 | MCS13 | MCS4 | 2Tx 2Rx ULA medium | TDLC300-100 | random | 13.71 | 14.46 | 11.33 | 12.20 |
| 4 | 1 | 3 | 1 | 1 | MCS13 | MCS13 | 2Tx 2Rx ULA medium | TDLC300-100 | random | 16.86 | 17.72 | 12.42 | 12.97 |
| 6 | 1 | 0 | 2 | 2 | MCS13 | MCS19 | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal | 12.99 | 13.87 | TBA | TBA |
| 7 | 1 | 3 | 2 | 2 | MCS13 | MCS19 | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal | 11.73 | 11.52 | TBA | TBA |

| Figure 7 – Case 1 vs. Case 3 (Full CHBW) | Figure 8 – Case 1 vs. Case 3 (Partial CHBW) |
| --- | --- |
| Figure 9 – Case 2 vs. Case 4 (Full CHBW) | Figure 10 – Case 2 vs. Case 4 (Partial CHBW) |
| Figure 11 – Case 6 vs. Case 7 (Full CHBW) |  |

Conclusion

This paper presents Nokia's simulation results for the study on blind detection for detecting interference parameters.

- References

- R4-2305914 WF on advanced receiver for MU-MIMO scenario