3GPP TSG-RAN WG4 Meeting #107 	R4-2307473

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements - PDSCH

Agenda item:	8.8.4.2

Document for:	Discussion

Introduction

In RAN4#106-bis-e good progress was made on the topic of requirements for PDSCH (see [1]).

In the following we will provide our view on the remaining open issues and make new observations and proposals where needed.

Discussion

Simulation assumptions for PDSCH performance evaluation

Channel model

In RAN4#106bis-e the channel model for requirement definition was discussed. The following options were captured in the WF [1]:

Issue 2-2-1: Channel model <Way forward > - Proposals - Option 1: Consider including TDLA30-300 for 120kHz/200MHz requirements, if introduced - Option 2: Consider TDLA30-300 for 100 MHz/120 kHz. - Option 3: Consider TDLA30-75 for 100MHz/120KHz

In addition, we would highlight that in RAN4#106 it was agreed to use TDLA30-75 (from RAN4#106 WF [3]):

Issue 2-2-1: Channel model - Tentative agreements: - Include TDLA30-75; Other channel models are not precluded;

Based on the WF from RAN4#106 we see it already agreed to include TDLA30-75 in the requirement definition. The pending question is if other doppler settings should also be introduced. To reduce the initial simulation workload, we are fine with keeping to the RAN4#106 agreement to use TDLA30-75 for the initial alignment. If other companies provide simulation results with higher doppler settings, we are open to discuss introducing requirements with doppler settings additional to 75Hz.

It was agreed in RAN4#106 to include TDLA30-75 channel model.

Introduction of additional doppler settings can be further discussed based on simulation results.

Use TDLA30-75 channel model as baseline for 120kHz/100MHz requirements.

PTRS Port

In RAN4#106bis-e the PTRS port allocation for requirement definition was discussed. The following options were captured in the WF [1]:

Issue 2-1-5: PTRS Port <Way forward > - Proposals - Option 1: One PTRS port per TRP for both sDCI and mDCI schemes - Option 2: One port PTRS for s-DCI SDM scheme

Enabling PT-RS is needed for FR2-1 and already agreed in RAN4#106bis. The remaining question is if PT-RS is required to be send on each TRP of if it is enough for sDCI to only receive PT-RS on one TRP. We see it critical especially for scenarios with joint demodulation (i.e. sDCI) to transmit PT-RS on each TRP as it cannot be assumed that each TRP will be received with the same phase difference.

It cannot be assumed that each TRP will be received with the same phase difference, hence there is a need for transmitting PT-RS on each TRP.

Use one PTRS port per TRP for both sDCI and mDCI schemes (Option 1)

Time offset between TRPs

In RAN4#106bis-e the time offset between TRPs was discussed. The following options were captured in the WF [1]:

Issue 2-2-7: Time offset between TRPs <Way forward > - Proposals - Option 1: Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}. - Option 2: Use time offset of TRP1 with respect to TRP2 as {0.2us, -0.05us} for different test cases. - Option 3: For NR FR2-1 multi-Rx chain DL reception, consider sDCI and mDCI based SDM scheme with timing offset between TRxPs as follow - Timing offset of the second TRxP from the first TRxP [us] as -0.0625 or 0.25. - Option 4: Select following value for timing offset of the second TRxP from the first TRxP for demodulation cases. Timing offset[us]. Multi-DCI based multi-TRP; non-overlapping; -0.0625. full-overlapping; -0.0625. Single-DCI based multi-TRP; SDM; 0.25.

In the current requirement definitions for mTRP in [2] defining the time offset of the second TRP from the first TRP where 15kHz SCS used the values of {-0.5,2} and for other SCS settings the formula

∆t=2-μ∆t1 , ∆t1=2us, -0.5us

was used to calculate the time offset.

Based on this, we calculate time offset between the two TRPs transmission points as {0.25us, -0.0625us}.

We do not see the need to define all requirements with both time offset values ({0.25us, -0.0625us}), however would like companies views into how a selection of time offset value can best be matched to transmission scenarios.

Based on earlier agreements in RAN4 on calculation of time offset, we see using {0.25us, -0.0625us} as acceptable.

Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us} and further discuss which time offset use with which configuration.

Frequency offset between TRPs

In RAN4#106bis-e the frequency offset between TRPs was discussed. The following options were captured in the WF [1]:

Issue 2-2-8: Frequency offset between TRPs <Way forward > - Proposals - Option 1: Select 3000Hz frequency offset for all demodulation cases as starting point. - Option 2: 600Hz - Option 3: 400Hz

The frequency offset between the two TRPs should be defined based on the center frequency being agnostic for the PDSCH tests. Assuming the most global band used for deployed networks is n257 and with a reference precision of the gNB frequency of maximum 0.1ppm, we can set the frequency offset between the two TRPs to 3000Hz.

We see the most globally used band for FR2-1 in deployment to be n257. With max 0.1ppm deviation on the gNB, we can set the frequency offset between the two TRPs to 3000Hz as a starting point.

Select 3000Hz frequency offset for all demodulation cases as starting point (option 1). Final frequency offset can be selected pending test feasibility.

Conclusion

This paper presents Nokia's view on the open issues with relation definition of PDSCH requirements for MultiRx Demodulation performance.

In the paper, the following Observations and Proposals were made:

Channel model

- It was agreed in RAN4#106 to include TDLA30-75 channel model.

Introduction of additional doppler settings can be further discussed based on simulation results.

- Use TDLA30-75 channel model as baseline for 120kHz/100MHz requirements.

PTRS Port

It cannot be assumed that each TRP will be received with the same phase difference, hence there is a need for transmitting PT-RS on each TRP.

Use one PTRS port per TRP for both sDCI and mDCI schemes (Option 1)

Time offset between TRPs

Based on earlier agreements in RAN4 on calculation of time offset, we see using {0.25us, -0.0625us} as acceptable.

Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us} and further discuss which time offset use with which configuration.

Frequency offset between TRPs

We see the most globally used band for FR2-1 in deployment to be n257. With max 0.1ppm deviation on the gNB, we can set the frequency offset between the two TRPs to 3000Hz as a starting point.

Select 3000Hz frequency offset for all demodulation cases as starting point (option 1). Final frequency offset can be selected pending test feasibility.

- References

- R4-2306000 - WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception

- 38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements v17.7.0

- R4-2302941 - WF for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception